MINISTRY OF EDUCATION AND SCIENCE OF UKRAINE

NATIONAL TECHNICAL UNIVERSITY OF UKRAINE

«IGOR SIKORSKY KYIV POLYTECHNICAL INSTITUTE»

- DIGITAL CIRCUITS

Lecture synopsis for studying the credit module of

«Digital circuits»

discipline

Recommended by the Methodical council of Igor Sikorsky KPI as a tutorial for students, who study in specialty 163 - Biomedical Engineering, specialization "Clinical Engineering"

Kyiv

Igor Sikorsky KPI

2020

DIGITAL CIRCUIT TECHNIQUE: lecture synopsis for studying the credit module of «Digital circuits» discipline [Electronic resource]: tutorial for students, who study in specialty 163 - Biomedical Engineering, specialization "Clinical Engineering" / Igor Sikorsky KPI; Compilers: Zubchuk: V.I., Delavar-Kasmai M. - Electronic text data (1 file  2.11 MB). - Kyiv: Igor Sikorsky KPI, 2020. - 188 p.

The vulture was provided by the Igor Sikorsky KPI Methodical Council (Protocol No. 8 dated 09. 04.2020) on the submission of the Academic Council of the Faculty of Biomedical Engineering (Protocol No. 7 of 24.02.2020)

Electronic online educational publication

DIGITAL CIRCUITS

Lecture synopsis for studying the credit module of «Digital circuits» discipline

Compilers: 	Zubchuk: Viktor Ivanovich, Ph.D., associate professor

Delavar-Kasmai Mohammad, Ph.D., senior lecturer

In the series of lectures the theoretical bases, principles of functioning and calculations of the main functional nodes of digital circuits technique methods are considered by sections: electronic switches on diodes and transistors, logical elements on the principles, DTL-, TTL-, ECL-, MOS- and І2L- technology, code converters, including encoders, decoders, multiplexers, demultiplexers, combination adders, digital comparators, major trigger types (synchronous and asynchronous RS-, D-, JK- triggers), registers, asynchronous and synchronous counters, pulse devices for the detection, generation and generation of pulses, digital-to-analog and analog-to-digital converters.

© Igor Sikorsky KPI

- Contents

1. Logic switches	7

1.1. Diode as a switch	7

1.2. Bipolar transistor as a switch	11

1.3. Transition processes	14

1.4. The switch based on a bipolar transistor with a nonlinear feedback	19

1.5. Electronic switches based on field effect transistors (FET)	22

2. Diode-transistor logic elements (DTL)	29

2.1. The basic element of NAND (Sheffer stroke)	30

2.2. Logic Element AND-OR-INVERT, NOR	33

3. Transistor - transistor logic (TTL)	35

3.1. The basic element of NAND (Sheffer stroke)	35

3.2. Modifications of  LE NAND	38

3.2.1. LE with free collector	38

3.3. LE with block	40

4. Emitter-coupled logic (ECL)	42

4.1. Basic logic element OR / NOR	43

4.2. Basic logic element E2CL	46

5. Logic elements on MOS transistors	47

5.1. LE NAND type	47

5.1.1. n-MOS-technology	48

5.1.2. CMOS-technology	49

5.2. LE NOR type	51

5.2.1. n-MOS technology	51

5.2.2. CMOS-technology	52

5.3. Elements of two-level logic	53

5.3.1. n-MOS-technology	54

5.3.2. CMOS-technology	55

5.4. Buffer Amplifiers	56

6. Integral injection logic Elements (І2L)	58

6.1. Base element	58

6.1.1. Input characteristic (dependence I1=f1(U1))	59

6.1.2. Output characteristic	60

6.2. Realization of logical functions OR, NOR, AND, NAND	61

7. Code Converters (CC)	65

7.1. Synthesis converter codes	65

7.2. Encoder	68

7.3. Decoder	72

7.3.1. Linear decoder	73

7.3.2. Pyramidal decoder	75

7.4. Multiplexers	76

7.5. Demultiplexer	79

7.6. Realization of logic functions multiplexers	80

7.7. Combination devices shift	83

8. Combinational adders (CA)	85

8.1. Half adders	85

8.2. Full adders	87

8.3. Subtractor	89

8.4. An algebraic adder	92

8.5. Multi-digit combiners	93

8.6. Binary-coded decimal adder	95

8.7. Combinational Multiplier	98

9. Digital comparators	99

9.1 Single-bit digital comparator	100

9.2. Multi-bit digital comparator	100

9.3. Digital comparator based on adder	103

9.4. Sectioned digital comparator	104

10. Flip-flops (Triggers)	106

10.1. Asynchronous RS-triggers	106

10.1.1. RS-trigger logic elements NOR 	106

10.1.2. RS-trigger logic elements NAND	108

10.2. Synchronous RS-triggers	109

10.2.1. RS-trigger logic elements NAND	109

10.2.2. RS-trigger logic elements NOR 	110

10.3. Varieties RS- trigger	111

10.3.1. R-trigger	111

10.3.2. S-Trigger	112

10.3.3. E-Trigger	113

10.4. RS-trigger “Latch” type	114

10.5. RS-trigger MS (Master-Slave) type	115

10.6. D-triggers	116

10.7. D-trigger “Latch” type	118

10.8. D-trigger MS (Master-Slave) type	119

10.9. Universal JK-trigger MS type	120

10.10. JK-trigger "Latch" type	122

11. Registers	124

11.1. Parallel registers	125

11.2. Shift register	128

12. Counters (CT)	132

12.1. Asynchronous binary counters	133

12.2. Reversible binary counters	135

12.3. Counters with arbitrary coefficient of account	136

12.4. Binary counter with controlled ratio Kсч	138

12.5. Synchronous binary CT	139

12.6. Reversible synchronous counters	141

12.7. Synthesis of synchronous CTs with an arbitrary conversion table	142

12.8. Synchronous counter with multiplex control	148

13. Impulse devices	149

13.1. Impulse front detectors	149

13.1.1. Detector of positive front (DPF)	150

13.1.2 Detector of negative front (DNF)	150

13.1.3. Front detector (DF)	150

13.2. Pulse Expander	153

13.3. Single vibrators	154

13.3.1. SV on logical elements NAND	152

13.3.2. SV on logical elements NOR	153

13.3.3. SV based on RS-trigger	154

13.3.4. SV on operational amplifier	155

13.4. Multivibrators (MV)	157

13.4.1. MV on inverters (NOT)	157

13.4.2. MV on NAND with auto start	160

13.4.3. MV on the Schmitt trigger (TS)	163

13.4.4 MV based on an operational amplifier	165

13.4.5. MV on RS – trigger	165

13.4.6. MV on two operational amplifiers	168

13.5. Generators of linear output voltage	170

13.5.1. LOV generator on the dinistor (Shockley diode)	172

13.5.2. Shaper of LOV on an operational amplifier	173

14. Digital to analog and analog to digital converters	175

14.1. DAC based on analog adder	176

14.2. DAC based on R - 2R series of resistors	174

14.3. DAC on current switches	176

14.4. ADC of deployment compensation	178

14.5. ADC following compensation	179

14.6. Successive approximation ADC	181

14.7. ADC of double integration	182

14.8. АDC of parallel transformation	186

14.9. Conveyer АDC	187

- 1. Logic switches

Technical implementation of digital circuits, in which the signals are presented as discrete quantum levels of voltage (current), is based on the use of electronic voltage (current) commutators, are called logic switches.

As nonlinear devices with controlled resistance in logic switches, semiconductor diodes, transistors, phototransistors, thyristors, optrons, and electronic lamps are used.

- 1.1. Diode as a switch

Fig. 1.1

In diode switches we use, dependence of diode resistance to magnitude and sign of the applied voltage. In fig. 1.1. (a) is shown typical static current–voltage characteristics (I–V curve) of germanium (Ge) and silicon (Si) diodes, and in Fig. 1.1. (б) and (в) are equivalent circuits for replacing a diode with a direct current for linearized (allocated by hatching) areas, respectively, 1 and 2. The current of the p-n-junction and the diode and the voltage Uд on it are related by the ratio:

ід = І0 {exp [Uд / (m φT)] - 1} = І0 {exp [Uд q /(m k Т)] - 1},	(1.1)

where І0 - thermal current saturation;

φT - Temperature potential (at normal temperature T = 298 ° K, φT= 26 mV);

k - Boltzmann's constant;

T - Absolute temperature;

q - Charge of an electron.

m - is the coefficient taking into account the influence of surface currents of leakage of germanium diodes and generation and recombination currents in the p-n transition of silicon diodes (for germanium diodes mGe ≈ 1.2 ... 1.5, for silicon - mSi ≈ 1.2 ...  2). The thermal current saturation I0 practically does not depend on the voltage applied to the diode and is determined by the electrophysical properties of the semiconductor material and its heating temperature:

I0 = I00 exp [- Uk / φT)],

where I00 is the constant due to the material of the semiconductor and the concentrations of impurities;

Uk - contact potential difference (Uk (Ge) = 0.3 ... 0.4 V, Uk (Si) = 0.6 ... 0.7 V).

The thermal current saturation I0 exponentially increases with increasing temperature. In approximate calculations it is assumed that the current I0 in the case of a germanium diode doubles with an increase in temperature for every 10 ° С, and in the case of silicon - every 7 ° С.

Taking into account the active resistance of the p- and n-regions, the contact resistances of the outputs and the ratio (1.1) for the real diode:

Uд = U0 + ід r = m φT ln(ід / І0 +1) + ід r,			(1.2)

where r is the total active resistance of the p – n- regions and contacts of the diode.

The differential active resistance of the diode will be obtained from the expression (1.2):

rд = dUд / diд = m φT / (I0 + ід) + r,

The resistance rд is nonlinear and depends on the current of the diode.  But at sufficiently large direct-bias voltages (Uд>>Uк), the resistance rд ≈ r, i.e. the resistance of the equivalent diode replacement circuit (Fig. 1.1. (б)) rпр ≈ r and has the order of units and tens of ohms.  At the inverse displacement of the diode (Uд<0) ід ≈ 0, rд = rобр = mφT / І0 + r ≈ m φT)/ І0  and is in the order of tens and hundreds of kΩ.

The voltage U0 of the equivalent voltage source in the substitution scheme (Fig. 1.1-б) is obtained by extrapolation of the quasilinear regions of the region 1 I–V curve to the intersection with the abscises axe, and it has the order of the contact potential difference: U0 ~ Uk.

The scheme of the simplest diode switch, controlled by the source of the signal U1 and loaded on the resistance Rн, is shown in Fig. 1.2-a, and in fig. 1.2-б, and в-equivalent circuit of the substitution respectively for the logic signal "1" and "0" at the input.

Fig. 1.2

Determine the value of the output voltages in each of the states of the switch, using the principle of superposition.  At low input voltage, level (U01<< 0) the diode VD is closed, the switch is open (Fig 1.2-в) and at its output set the low level:

U02 = U01[(R||Rн) / (Rr+ rобр + R||Rн)] - I0[rобр||(R||Rн + Rг)],		(1.3)

Since usually rобр>> R, Rн can be approximated to be U20 ≈ -I0 (R || Rн).

At high level of input signal U11 the diode VD is open, the switch is closed (Fig 1.2.-б) and the switch output has a high level:

U12 = [(U11 – U0)(R||Rн)] / [Rг + rпр + R||Rн], 		(1.4)

Since usually rпр<< R, Rн can be considered approximately U21≈U11-U0.

From the relations (1.3) and (1.4) it is seen that the voltage drop across the output: ΔU2 = U12 - U02< ΔU1 = U11 - U01 (less than U0). The attenuation of the output signal is greater, the greater the resistance of the signal source Rг.  The disadvantage of the diode switch in Fig. 1.2 is the direct dependence of the level U12 on the input signal U11.

The second variant of the diode switch is shown in Fig. 1.3.  At the low level of the input signal, the diode VD is open and the current of the power source Uип flows through it, which is limited to the resistance R. If R >> Rг, then virtually the entire supply voltage Uип falls on the support R, so the low potential U02 (Fig. 1.3-б) is set on the load support of the Rн switch:

U02 = Uип.[(Rн||(Rг + rпр)) / (R + Rн||(Rг + rпр))] + (U01 + U0)[(R||Rн) / (Rг +rпр + R||Rн)].

If Rг + rпр<< R||Rн, then U02 ~ U01 + U0.

Fig. 1.3

At a high level of the input signal, if the condition U11>Uип Rн / (R + Rн), the VD diode is closed, the input circuit is disconnected from the load (Fig. 1.3-в) and the output voltage is set to high:

U12 = Uип [(Rн||(rобр + Rг)) / (R + Rн||(rобр + Rг))] + U11[R||Rн / (Rг + rпр + R||Rв)] +I0(R|| Rн ||(Rг + rобр)),

Usually rобр>>Rг, Rн, so U12 ≈ (Uип + RI0) Rн / (R + Rн) and it does not depend on the input signal. Note that if in the circuit diagram of Fig. 1.2 the low output signal is U02 ≈ 0, and the high depends on the input signal U11, then in the circuit diagram of Fig. 1.3, on the contrary, the low level U02 ≈ U01 + U0 is proportional to the input signal, and the high U12 does not depend on it.

Considered diode switches can be used to construct the simplest combinational logic circuits. In Fig. 1.4-a shows the scheme, and in Fig. 1.4-б the truth table and conditional graphic designation of a two-input logic element OR on diode switches.  Only in the case when on both inputs have low level U011 and U012, diodes VD1, VD2 are closed, the output is disconnected from the inputs, and is set to low.  In any other combination, the output connects to the input to which the highest level is given, i.e. U12 ≈ max {U11, U12} - U0. This expression is an analogue expression of disjunction.

Scheme on Fig 1.4-в - variant of the switch on Fig. 1.3 for two inputs.  If at least one of the VD1 diodes, VD2 is open by a low input signal, the circuit output is connected to the lowest input. Only when all the diodes are locked up by high levels at the inputs, the output circuit is disconnected from the inputs and the load is set to high potential. As can be seen from the truth table on Fig. 1.4-г, such a scheme implements a logical operation of conjunction.  Its analog equivalent is determined by the expression U02 ≈ min {U11, U12} + U0

Fig. 1.4

Based on diode switches, two-step combinational logic elements are constructed. Listed in Fig. 1.4-д scheme implements the logical function у = х1х2 + х3х4 or for electric signals U2 ≈ max {min [U11, U12], min [U13, U14]}. Since the circuit consists of passive components, the input signal is weakened by passing each logical degree. In order for the amplitude of the output signal U2 to be maximal, it is necessary to fulfill the conditions R1<< R2<<Rн, which inevitably leads to an increase in the power consumption of the circuit.

- 1.2. Bipolar transistor as a switch

The use of bipolar transistors in electronic switches is based on the properties of transistors to change the resistance of a very large (hundreds of kΩ) in a Cut-off mode to significantly less, in an active mode (kΩ units) and very small in saturation mode (units of Ω) under the action of the control signal.

In Fig. 1.5-а shows the static characteristics of the bipolar transistor when included in the scheme with a common emitter (CE); on Fig 1.5-б - input characteristics ІБ = f1 (Uбе) while Uке = const; in Fig 1.5-в - output characteristics ік = f2 (Uке) while Іб = const. The transistor in switch devices operates in a high signal mode and its properties can be described by a nonlinear Ebers-Molla injection model of ideal diodes and controlled current sources (Fig. 1.6).

Fig. 1.5

Fig.1.6

The current of electrodes of the transistor, depending on the voltage on them, are determined by the relations:

іc = [(Ik0αI) / (α (1 - ααI))] [(exp(λUbe) - 1) – α (exp(λUcb) - 1)];
іэ = [Ik0 / (1 - ααI)] [(exp(λUbe) - 1) α1 – exp(λUcb) + 1)];
іb = іe – іc,

where: λ = 1 / (mφt);

Iк0 - collector reverse current;

α and αI - emitter current transfer coefficients in the collector circuit, respectively, in normal and inverted active modes.

Such model is used mainly in machine design methods, and in approach calculations, piecewise linearly approximated models are used.

Let's consider the scheme of the simplest switch on the bipolar transistor of npn-type (Fig. 1.7-а).  The load on the transistor VT is a resistor connected between the output and the common bus (R''н), or between the output and the power supply (R'н), or the load can be divided (R'н, R''н).  The mode of operation of the transistor is determined by the sources of the input signal Uг and the input circuit of the resistors R1, R2 and the source of the closing offset - Uип2.  The R1 and R2 resistors must be selected so that, at low input levels, the U01 transistor VT is reliably locked throughout the operating range of ambient temperatures, and at high U11 level, the transistor must be saturated at the input.

Fig.1.7

The bias source –Uип2 is not required if the transistor cutoff mode is provided by the low level of the input signal U01 and the condition of closure is not disturbed at maximum ambient temperature.

To simplify the analysis of the statics and dynamics of such a switch, we transform the scheme using the equivalent generator theorem.  The components of the transformed scheme (fig. 1.7-b) are determined from the relations:

Uи1 = [Uип.Rн+UнR’к] / [Rн+R'к],	Rк = R'к||Rн,	U1 = (UгRн + Uип.2R1) / (Rн + R1).

To provide the transistor cutoff mode, it is necessary that, at a low level U01 of the input signal, the emitter junction of the transistor was locked. As can be seen from Fig. 1.5-б, locking can be considered low potential U01 at the input, which provides on the emitter junction voltage Uбе ≤ U0. If this condition is fulfilled, then both transistor junctions are locked and the transistor in the circuit of Fig. 1.7-б the first approximation can be replaced with the source of the reverse current of the collector junction Iк0 (Fig. 1.8-а).

Fig. 1.8

The thermal current Iк0 flows through the resistor Rб and increases the potential of the base.  The higher the collector junction temperature, the greater the current IK0 and the voltage Uбе. The condition for locking the transistor must be performed in the worst case, that is, at the maximum temperature of the collector junction and the corresponding current Iк0max through it:

Uбе = U 01 + Iк0maxRб ≤ U0,		(1.5)

If the condition (1.5) is satisfied, the transistor VT is locked; on its collector, which is the output of the circuit, a high level is set:

U12 = Uип - Iк0Rк.				(1.6)

To unlock the transistor, you must submit a high level U11 > U0, on input of the switch. In this case, the transistor can be in active mode or in saturation mode. The saturation mode occurs if the base current Іб of the transistor, reaches or exceeds the Ібн value, which corresponds to the position of the transistor operating point at the interface between the active mode and the saturation mode: Iб ≥ Іб.н.

The input circuit of the saturated transistor in the linear approach version can be present in the same way as a diode by a series-connected volume impedance of the base rб and a voltage source U0.  The resistance between the collector and the emitter of the saturated transistor is determined by the slope of the saturation line (Fig. 1.5-в): rкн = ΔUке / ΔIк when Іб ≥ Iбн.  An equivalent circuit of a transistor switch in saturation mode is shown in Fig. 1.8-б. For this scheme, the saturation condition has the form:

Іб = (U11 - U0) / (rб + Rб) ≥ Iбн = Iкн / βmin = Uип / (βmin(Rк + rкн)),	(1.7)

Since the gain of the current transistor β has a technological range, inequality (1.7) must be performed in the worst conditions, that is, with the smallest allowable value β = βmin. If the condition (1.7) is satisfied, the transistor VT is saturated and the low-level transistor is set at the output of the open switch:

U20 = rкнIкн= rкнUип  /(Rк+rкн).

- 1.3. Transition processes

The inertia of the switch based on the bipolar transistor is characterized by the duration of the switching cycle, which includes: tзт.вкл.- delay of switching on the transistor when input of the switch by a high level signal U11, which satisfies the condition (1.7);  tвкл - the duration of the transistor's inclusion, that is, the time of the increase of current through the transistor from the thermal Iк0 to the saturation current Iк.н;  tр- the duration of the shutdown delay due to the recombination of charge in the base when the transistor passes from saturation to the active mode;  tвикл- the time of the switching, that is, the reduction of the collector current of the transistor from the saturation current Iкн to the current level Iк0; t01ф- duration of the potential increase on the collector of the transistor, due the charge capacities of loading and installation.

Then, the full cycle of switching or split time Tроз is equal to the sum of the named intervals:

Tроз = tзт.вкл + tвкл + tp + t01ф.

This is the time necessary to recharge the parasite capacitance of the circuit mounting, interelectrode capacitance of the transistor, to accumulate the charge of minority carriers at the base of the transistor at the offset and recombination of this charge when the transistor is locked.

Since the collector current of the bipolar transistor is a current of extraction of non-main charge (here - electrons) from the base and is proportional to the charge in the base, the transients are conveniently analyzed by the dynamics of charge changes.  Therefore, such method of analysis of transients is named the method of analysis of charge of base.Fig.1.9

Fig 1.9-а shows the variant of the equivalent circuit (see fig. 1.7-б) of the transistor switch, which shows the load capacitance of Сн and the equivalent input capacitance of the transistor Свх, due to the capacitances of the emitter and collector junction of the transistor, as well as the parasitic capacitance of the installation.

Consider the characteristic sections of the transitional process in time diagrams (Fig. 1.9-б).

Fig. 1.9

1. By the time t1, the transistor VT is locked at a low level of the input signal U01, which satisfies condition (1.5).  The current in the base circuit of the transistor is determined by the reverse current of the collector junction: ІБ = - Ік0.  The charge Q of the base in the absence of injection of minority carriers through the emitter transition is practically absent: Q ≈ 0. In the collector circuit, the return current of the collector transition is Ік = Ік0. At the switch output, a high level is maintained:

Ul2 = Uип - Ік0Rк.

2. At time t1, the potential at the input of the switch jumps increases from U01 to U11.  The base potential of the transistor Uб increases with charge capacitor Свх due to resistance Rб.  The voltage Uб increases with the exponential law with a constant time τб = RбСвх from the initial voltage U0Свх = U01+RбІк0 to the asymptotic level UСвх (∞) = U11 + RбIк0. By the time t2, until the voltage on the base remains less than the threshold voltage U0, the transistor remains in the cutoff mode, the charge of the base, the potential and collector current are not changed.  The time interval from t1 to t2, when the potential of the base Uб (the voltage at the emitter junction Uбе) reaches the threshold voltage U0, determines the duration of the delay of the transistor activation, tзт.вкл= t2 - t1. To determine the duration of the tзт.вкл, we use the property of the exponential function (Fig. 1.9-в) A(t) = А0+(А∞ - А0)(е-t/τ - 1), which is as follows: if the parameters of the exponential function А∞ - is the asymptotic value, τ is the time constant, A(t1) and A(t2) are the exponential levels, the interval of time from t1 to t2 is determined by the relation:

∆t = t2 – t1 = τ ln[(А∞ - А{t1})/( А∞ - А{t2})].		   (1.8)

Using this expression, we define the delay time for inclusion:

tзд.вкл = τб ln [(U11+Iк0Rб-{U01+ Iк0Rб})/(U11+ Iк0Rб-U0)] ≈

≈RбCвх ln [(U11-U01)/(U11-U0)].  					(1.9)

3. At time t2, the potential of the base exceeds the threshold voltage U0, opens the emitter junction, and the transistor switches from the cutoff mode to the active mode.  Injecting non-main charges into the base by the emitter (in the n-p-n-transistor they are electrons, and in p-n-p - holes) due accumulate charges in base. The rate of charge accumulation is higher, the larger the current of the base Іб(t). With a sufficiently large base current, the input circuit of the transistor can be represented as Fig.1.8-б. Then the current in the open base transistor base circuit:

іб(t) = (U1(t)-U0)/(Rб+rб).

The increment of the charge of minority carriers ΔQ per unit time in the interval Δt is determined by the expression:

∆Q/∆t = iб(t)-Q/τβ,				(1.10)

where τβ is the average lifetime of non-main charges. In expression (1.10), the first term describes an increase in the charge in the base (if Іб(t) > 0), and the second one is the decrease in charge due to the finite lifetime of non-main charges τβ and the recombination of a portion of the carriers in the active base region.

In the limit for infinitesimal intervals of time we obtain the differential equation of the first order:

dQ/dt = iб(t)-Q/τβ ,					(1.11)

If U1(t) = U11= const, the current of the transistor base remains practically constant:

iб(t) = Iб1 = (U11-U0)/(Rб+rб) ≈U11/Rб.

then the solution of equation (1.11) is an exponential function:

Q(t) = Q1[1-exp(-t/τβ)]+Q0 ,				 (1.12)

where      Q1 = τβ Iб1,							(1.13)

With the accumulation of charge in the base proportionally increases the collector current, the voltage drop on the resistor Rк increases and the collector's potential falls.  At time t3 the transistor switches from the active mode to saturation mode, the collector current growth (see Fig 1.9) stops at the Ікн = Uдж.ж/ (Rк + rк.н) and the collector potential falls at U02 level.

The interval from the moment t2 to t3presents the switching time of the transistor tвкл. Its duration can be determined using (1.12), if we take into account that the charge on this interval increases from Q0 = 0 to the value

Qгр = τβIбн,                                               (1.14)

which corresponds to the position of the operating point of the transistor on the boundary between the active mode and the saturation mode.  In this case, the charge increases exponentially with a constant time τβ and asymptotically approached to the Q1 level.  Then, using the expression (1.8) and taking into account the formulas (1.13), (1.14) we obtain:

tвкл. = τβln[Q1/(Q1-Qгр)] = τβln[(τβIб1)/(τβIб1- τβIб.н.)] = τβln[S/(S-1)] ,	(1.15)

where S = Iб1/ Iбн- saturation coefficient of the transistor.

The duration of the front edge of the output signal is t10ф ≈ tвкл.

4. At this stage, all currents and voltages established at time t3remain constant. The transition process is characterized only by the continued accumulation of charge in the base beyond the limit value of Qгр. The charge of non-main carriers exceeding the value of Qгр is called excess. The charge continues to increase in exponentiation, but with a changed exponential parameter τн = 0,7 ... 1,5 τβ, which characterizes the average lifetime of non-main charges in saturated mode.  The change in the average lifetime of non-main charges is due to the redistribution of charge in the active base region when the transistor passes from the active mode to saturation.  In this case, for fusion transistors, τн< τβ, and for diffusion τн > τβ. It can be assumed that at the time tн = 3τнthe process of accumulation of excess charge Qнадл ends and the charge reaches the value:

Q2 = Qгр+Qнадл = τнIб1				(1.16)

The ratio of the accumulated charge Q2 to the limiting Qгр in terms of expressions (1.14) and (1.16) approximates the saturation coefficient of the transistor:

Q2/Qгр = τнIб1/τβIб.н. ≈ S .

5. At the rear edge of the input signal at time t5, the current of the base Іб(t) of the transistor jumps varies by magnitude (and sign) Iб2 = (U01-U0) / (Rб + rб), the equilibrium state of charge of the base is violated and its recombination begins. The excess charge exponentially with a constant time τн decreases from the value Q2, seeking asymptotically to Q3 = τнIб2. At this stage, the charge in the base Q(t)>Qгр and the transistor remains saturated until t6 when the excess charge recombination ends and the saturation transistor enters to the active mode.  In the interval from t5 to t6 the collector current Ікand the output voltage U2 remain unchanged, and this stage of the transition process is called the recombination time.  Duration of the stage of recombination:

tp = t6-t5 = τнln[(Q3-Q2)/(Q3-Qгр)] = τнln[(Iб2τн-Iб1τн)/(Iб2τн-Iб.н.τβ)] ≈

≈τнln[(Sзап-S)/(Sзап-1)] ,						(1.17)

where  Iб2/Iбн = Sзап  - the coefficient of closure.

6. At time t6 the transistor enters the active mode and from the level Qгр the base charge exponentially with the constant time τβ decreases, seeking asymptotically to Q4 = τβIб2.  At the same time synchronously decreases the current collector Ік and begins to increase the output voltage.  This stage, which is called the exclusion stage, ends at time t7, when the level Q (t) ≈ 0is reached. Duration of the exclusion stage:

tвикл = t7-t6 = τβln[(Q4-Qгр)/Q4] = τβln[(Iб2τβ-Iбнτβ)/Iб2τβ]

= τβln[(Sзам-1/Sзам].	(1.18)

At time t7, the transistor goes into the cutoff mode, its input impedance sharp increases, the base current is set to Іб (t) = -Ік0, and the collector current is ІК (t) = Iк0.

7. The increase in the output voltage U2(t) is associated with the charge due to the collector resistance of Rк of the equivalent load capacity С0 = Сн + Ск..б. + См, where Сн, Ск.б, См - capacity of a load, collector's transition and installation.  The duration of the rear front is t01ф ≈ 3τзар = 3RкС0.  In the case of a purely active load and a small installation capacity (См ≈ 0), t01ф ≈ tвикл can be considered.

Analyzing the dependence of the duration of the time tроз difference of the transistor switch on the parameters of its components and control signals, we can draw the following conclusions: the time tроз less, then the less the τβ of the transistor, that is, the greater the marginal amplification frequency fα=(β+1)/(2πτβ); with increasing transistor saturation coefficient, the duration time tвкл decreases, the time of recombination tp increases, and the duration of the exclusion tвикл does not change,  the recombination  time tp and the time of the exclusion tвикл the smaller the bigger  factor of the SЗАМ.

Therefore, the minimum duration of the tдоз can be obtained using transistors of the required frequency range and the optimal choice of signal levels controlled by the switch U01 and Ul1. If, however, the minimum tдоз is more than permissible, it is necessary to use circuit technical methods of forcing transients in transistor switches.

- 1.4. The switch based on a bipolar transistor with a nonlinear feedback

The duration of the switch-on phase of the transistor tвкл can be reduced by supplying a larger discharge current to the base.  In accordance with the expression (1.15), the tвклis decreasing, but due to the growth of the saturation coefficient S, the duration of the resorption phase tp increases simultaneously. As a result, in spite of additional power losses, the speed of the switch is not increased. One of the outputs in this situation is the exclusion of the saturation mode of the transistor and thus the switching of its operating point between the active mode and the cut-off mode.

In Fig. 1.10. (a) is shown a schematic diagram of a transistor switch that uses a nonlinear feedback (NLF) via a VD diode (parallel negative feedback voltage).  The depth of such an NLF depends on the mode of the diode: if the diode VD is locked, the influence of feedback on the operation of the switch can be neglected.  If the VD diode is open, then due to its low resistance rдthe deep feedback and the voltage transfer coefficient in the circuit are realized:

Кuо.с. =U2/U1 ≈ rд/Rб -> 0 ,		rд<<Rб

and the potential of the collector of the transistor is fixed.

Fig.1.10

In Fig. 1.10 (б) shown time transient diagrams when applying the positive pulse switch to the input. Before the time t1 at the switch input there is a low level of U01 signal, the transistor VT is closed, on its collector is supported by high potential U12, which causes the inverse displacement of the diode VD. At time t1, the input sharp increases from zero U01 to the unit level U11. Through tзт.вкл at the moment t2 opens the emitter transition and the base of the transistor sets the current of the base:

Ібm1 = [U11R2-Uи.п2(Rб+R1)-U0(Rб+R1+R2)]/B,

where B = RбR2+R1R2+R2rб+rбR1+Rбrб, rб<<R1, R2, Rб.

As the collector current increases, the potential of the collector U2 falls, while the closing voltage on the diode VD decreases:

Uд = U*-U2 = U0+Iбm1rб+I1mR1-U2

where I1m = [U11(R2+rб)/(B-R2rб)]+[(Uи.п2rб-U0R2)/B] ≈ [U11/(Rб+R1)]   - the input current until the diode is unlocked.

At the time t3, when the collector potential is close to the saturation voltage, the VD diode opens and the input current I1m is redistributed between the VD diode and the resistor R1: the current of the resistor R1 is reduced by the magnitude of the diode current increase ΔІд.  Since the current of the diode closes through the collector circuit of the transistor at a practically constant collector potential, then the collector current from the moment t3 increases by an amount of ΔІд. The potential of the collector of the open transistor differs from the potential U* by the value of the voltage on the open diode. If we take it equal to the threshold voltage Uод of the diode, then approximate (since the small change in voltage U* after disconnecting the diode is not taken) voltage at the output:

U02 = U*-Uод = U0+Iбm1rб+I1mR1-U0д

At time t4, the constant values of the currents of the base Іб1, І1к collector and diode are set.

On the back edge of the input signal is current through a diode changed to the opposite direction, its return resistance is restored and until t6 the diode is turned off.  Thus, the NFL breaks apart and a recombination of the charge in the base of the current begins:

Ібm2 = [U01R2-Uи.п2(Rб+R1)-U0(Rб+R1+R2)]/B.

Further, the transient processes in the switches with NLF are similar to those previously considered in the transistor switch (see Fig 1.9).

In the scheme under consideration, there is no shutdown delay due to the excess charge of the base of the transistor, but there is a process of recombination of the charges accumulated in the diode.  Therefore, in practice for the implementation of NLF choose high-speed pulsed diodes or Schottky diodes that operate without charge.

The duration of the stages of switching on and off the transistor in the circuit with the NLF is determined by the disconnecting Ібm1 and the closing Ібm2 base currents, which, in turn, depend on the resistance of the resistors R6, R1, R2 and the voltage levels U01, U11, Uип.2.

Output voltage U02 also depends on the parameters of the input signal, which in the switching circuits is undesirable.  The circuit in which the resistor R1 is replaced by the VD2 diode is shown in Fig. 1.11-a.  In this scheme, when unlocking the diodes VD1 and VD2 on the transistor collector, the potential is fixed:

U02 ≈ U0+U0д2- U0д1 ≈ U0,

where U0 is the threshold voltage of the transistor, U0д1 and U0д2- the threshold voltages of the diodes VD1 and VD2, respectively. Voltage U02 slightly exceeds the voltage on the saturated transistor and does not depend on the parameters of the input signal.

If a Schottky diode is used as a diode VD1, which has a very small threshold voltage U0ш≈ 0.1 V, then the VD2 diode in the base of the transistor circuit is not required (Fig. 1.11-б), the Schottky diode and the collector junction of transistor are included in parallel,  but due to the fact that the limiting voltage of the diode U0ш is essentially less than the threshold voltage of the collector junction (for the silicon transistor U0к ≈ 0,5 ... 0,7 V), the Schottky diode opens earlier and thus prevents the transistor saturation.

Fig.1.11

In the open state, the potential of U02 is determined on the transistor collector, which can be determined by an equivalent switch circuit (Fig. 1.11-в):

U02 = Ібrб+U0+U0ш.

Voltage U02 is small (about 0.1 V), practically does not depend on the input signal and resistance in the base circuit and collector circuit. Another important advantage of the circuit (Fig 1.11-б) is its high speed, the Schottky diode works without accumulation of charge, so there is no stage for restoring the feedback resistance of the diode.  The advantages of a transistor with a collector-base connection through a Schottky diode have led to the creation of a monolithic structure of a diode, a Schottky-bipolar transistor manufactured in a single technological process, called the Schottky transistor (Fig. 1.11-г) and is widely used in integrated circuit engineering.

- 1.5. Electronic switches based on field effect transistors (FET)

For construction of electronic switches, it is possible to use field effect transistors with p-n-junction control, with an isolated switch and a built-in or induced channel. In the digital circuitry, Metal Oxide Semiconductor Field Effect Transistor (MOSFET) with induced p- or n-channel have become the most widely used.

In Fig. 1.12 в -shows the graphic image of a n-type inductive MOS transistor.

In Fig. 1.12-a, -б shows the drain and gate I–V curve of the MOS -transistor with induced n-channel.  If the voltage Uз.с<Uпop, then the charge transfer channel is not induced and the transistor is locked.  If Uз.с>Uпop, then the n-channel is induced in the sublingual region, through which the current flows, which is determined by the voltage Uc.н between the channel electrodes, the transistor is open and its working point is in the steep (triode, I) or gently sloping (pentode,  II) characteristics area.  Accordingly, in the field of characteristics I or II, the drain current of the transistor ic is described by the expressions:

І: ic=μ[(Uзи-Uпор)Uс.и-0,5U2с.и] ,

ІІ: ic=0,5μ (Uзи-Uпор)2 ,

where μ is the specific steepness of the control characteristic, the Uпор -threshold voltage of the current cutoff.

Fig. 1.12

In Fig. 1.13-a is a diagram of the simplest switch on the MOS transistor, included in the scheme with a general leakage and with the linear stock load Rc.  If the load of the switch is an active resistance of Rн, it can, by using the equations for the equivalent generator, be included in the equivalent resistance Rc.

Fig. 1.13

The static mode of such a switch is determined by the loading line on the family of stock characteristics (Fig. 1.12-а). At low level of the input signal U01<Uпop the MOS transistor is in the cutoff mode and, since the leakage current through the closed channel Іс is sufficiently small (Іс <10-9 А), it is possible to assume the high level of the output signal U12= Uи.п. The power consumption from the power source Uип in the closed state of the switch is Р1пот = 0. At a high level of the input signal U11 >Uпop in the transistor, the channel is induced and the current І1с flows through it, which is determined by the point of intersection of the loading line with the stock characteristic іс = f (Ucн).  Depending on the value of the input voltage U1, the supply voltage Uи.п and the resistance Rс, the working point of the transistor is detected in the "triode" region (the steep areas of the flow characteristics), to the left of the line, which is the geometric location of points of overlap of the flow characteristics (area I in fig.  1.12-a), or in "pentode" (area II grate areas in fig 1.12-a), to the right of the line (1.19):

Uсн = Uзи - Uпор.					(1.19)

At the output of the switch is set low potential:

U02 = Uип - IcRc ,					(1.20)

where Iс - flow current, which is defined graphically from Fig.  1.12-a.

To solve the analytical determination of the operating point of the open transistor (Uс.и= U02, ic = Ic), one must solve the equation (1.20) and one of the approximating stock characteristics of the equations for the triode region: iс= μ [(Uзи-Uпор)Uсн-0,5 U2си]or for the pentode area: iс = 0,5μ(Uз.и-Uпор)2where μ is the specific steepness of the drain and gate characteristic.

The transients processes in the switches on the field transistors are due to the transfer of carriers with mobility ν through the length L channel and the recharge of the inter electrode capacitances of the transistor (drain –bulk ССЗ, drain - source ССи ), as well as the load capacities of the Сн and the parasitic capacitance of the installation См.

The transport time of carriers through the channel tпер = 2,2L2/(νUс.и), where for electrons the mobility νn=0,04 m2/(Vs), and for holes - νр = 0,02 m2/(Vs)  , so n-channel MOS transistors have higher performance.  At length of channel L the order of units of microns and voltage Uc.н of order of tens volt tпер is very small and has the order of 10-9s.  Therefore, the speed of the switches on the MOS-transistors determined with processes of recharging of the interelectrode and external capacitances of the transistor.

In Fig.1.13-б - show the timing diagram of the idealized input signal U1(t)and the output signal U2(t).  The jump of the input signal at time t1 causes the in-phase interference δU12 at the output due to the transfer of the input part directly to the output through the capacitive voltage divider from the capacitance Сс.з and capacitance С0, which includes the capacity drain - source Сс.и, the load capacity Сн and assemblingСм

δU12 = [(U11-U01)Сс.з]/[ Сс.з+ С0],

where С0 = Сс.и+См+Сн.

Further, the capacitance C0 is discharged, and the capacitance Ссз is recharged through the open transistor and the drain resistor Rс.  In general, the speed of the process at this stage is determined by the conductivity of the open transistor, the output voltage exponentially falls with a constant discharge time:

τр = [(С0+Сс.з)Rc]/[μ(Uи.п-Uпор)Rс+1] .			(1.21)

Then the duration of the negative edge of the output signal (the time interval from t1 to t2 when the signal drops by 0.9 (U12-U02)) can be estimated as:

t10Ф ≈ 3τр =[3(C0+Cс.з)]/[μ(Uи.п-Uпор)].

Fig.1.14

At the back edge of the input signal at the moment t3, the common-mode emission δU02 = δU12 is formed.  Then the transistor is rapidly closed (in time  of nanoseconds), and the charge of the equivalent capacitance C0 and the recharge of the capacitance Cс.з, which proceeds according to the exponential law with constant charge time, proceeds: τз = (С0+Cс.з)Rc. Accordingly, the duration of the positive edge of the output signal is t01Ф ≈ 3τз =3Rc(C0+Cс.з).

From the standpoint of integral technology, the switch under consideration has significant disadvantages: it is difficult to produce transistors and high-resistance (drain -Rc) resistors in a single technological cycle. Except is high-energy consumption in the open state Р0пот = U2и.п/Rc.

The first of the disadvantages is eliminated in the integrated circuits by replacing the linear resistor Rc with a nonlinear one, which uses the MOS transistor with bipolar inclusion (Fig. 1.14-a) with a shorted bulk and drain. For transistor VT2: Uсн2 = Uзс2. If on the family of drain characteristics of the transistor VT2 to construct an infinite line dividing the pentode and triode regions (Fig. 1.14-б- curve 1) for which the expression is correct (1.19) and shift it along the axis of voltage to the value of the Uпор, we obtain a nonlinear dependence - I–V curve of the MOS-transistor in bipolar inclusion (Fig.1.14-b - curve 3). In the same graph, the I–V curve loading transistor VT2 is given at the supply of an arbitrary constant voltage Uип2 to its switch (curve 2 - if Uи.н1<Uи.н.2< Uи.н.2+Uпор, curve 4 - if Uи.н2<Uи.н1).  In order to provide I–V curves close to the linear, as transducers, transistors with a low specific steepness shut-off characteristic μ and a minimum voltage Uпор2 are manufactured as a load.

We construct the I–V curve of a nonlinear two-terminal on the family of flow characteristics of the switching transistor VT1 as a line of its load (Fig 1.14-в).  This allows you to graphically determine the levels of the output voltage of the switch in the locked U12 and the open U02 state, as well as the current of the open transistor VT1 with the load VT2.  Note that in the closed state, when both transistors are closed, the high level U12 is not defined strictly, since it depends on the random resistance of the leakage of the transistors VT1 and VT2:

U12 = Uи.п.1 Rвит.1/(Rвит.1+Rвит.2)

and may vary in the range Uи.п1> U12>Uи.п1-Uпop.2.

The considered circuit scheme is more technological for integrated circuitry, but it has high consumption of Р0пот = Uи.п1Ic when the transistor VT1 is open.  Reducing the power of Рпот by simply increasing the Rc resistance can only be made to certain limits, which are determined by the ratios for the maximum permissible length of the fronts t10фand t01ф.  Significantly, the power consumption of an electronic switch can be reduced if, as a load, use an element with a controlled internal impedance, which when closing the transistor VT1 should have minimum resistance, and when unlocking - the maximum.

In the scheme in Fig. 1.15-a as a load of the n-channel transistor VT1 used p-channel transistor VT2, controlled by the direct input signal U1.  Due to the symmetry of the circuit, each of the transistors is a controllable load for another transistor.  Different types of conductivity when operating with the same signal U1 provide mutually opposite modes of operation.  Therefore, it is said that the transistors in this scheme complement each other one, they form the so-called complementary structure.

Fig. 1.15

The type of static transfer characteristic of the complementary structure depends on the ratio of the supply voltage Uи.пand the threshold voltages of the n-channel (Uпор.1) and p-channel (Uпор.2) transistors.  If Uи.п> Uпор.1 + |Uпор.2|, then the transfer characteristic (Fig. 1.15-в) contains the following characteristic areas.

1. Uзс1 = U1< Uпор.1- Transistor VT1 in the cutoff mode, |Uзс2| = |U1- Uи.п|>>|Uпор.2| - transistor VT2 in triode mode.  Output voltage is defined as the result of the voltage supplied between the high resistor of the leakage of the locked transistor VT1 Rвит = 109 ... 1 012 Ω and the small resistance VT2 in the triode mode rTі2 ~ 102 Ω, so the output level is set to U12 = [Uи.пRвит.1] / [Rвит.1 + rTі2] ≈ Uи.п.  Typical value of high level U12 = 0.999Uи.п.  The current consumed from the power supply is insignificant: I1пот = Uи.п/ [Rвит.1 + rTі2] ≈Uи.п/ Rвит.1.

2. Uп.Т.1< U1< Uп.т.2; Uп.т.2- the limiting voltage of transistor transition VT2 from the triode mode to pentode.  In this area, VT1 works in pentode, and VT2 - in the triode of characteristics.  The complement pair is in amplification mode with the voltage transfer factor КU = -μ1٠(Uпор-U1)٠rTі2, where rTі2 is the differential resistance of the VT2 transistor in triode  mode, μ1 is the specific steepness of the control characteristic of the transistor VT1.  KU is insignificant because rTі2<< rпі2.  CurrentІпотthrough transistors is increasing.

3. Uп.т.2 < U1<Uп.т.1, Uп.т.1 - the limiting voltage transition of the transistor VT1 from the pentode mode to the triode.  Both transistors are in pentode mode and provide the maximum voltage transfer factor КU = - (μ1 + μ2) ٠(Uпор-U1)٠ [rпі1 || rпі2].  Consumption current continues to grow to the middle of area 3 to the value IC1 = IC2 = ICmax(fig 1.15-б, -в) and then with increasing U1 the consumption current begins to decrease, as the transistor VT2 from the value U1 = 0,5Uи.п  closes faster than opens VT1.

4. Uсп.т.1 ≤ U1<Uи.п. -| Uпор.2 |°- the transistor VT1 from the pentode goes into the triode mode, and VT2 remains in the pentode.  The transmission coefficient of the voltage is less than in the region 3, КU = -μ2٠(Uпор-U1)٠[rті1][rпі2]and with the growth of U1 decreases as the differential resistance rті1 of the transistor VT1 in the triode mode decreases.  The current consumption of Іпот is reduced due to the locking of the transistor VT2.

5. U1>Uи.п- | Uпор.2 |°- the transistor VT1 is in triode mode, the transistor VT2 - in the cutoff mode and because of its resistance to the flow of Rвит.2flows a very small current.  Output voltage of the switch U02 = [Uи.пrті1] / [Rвит.2 + rті1] ≈ 0. The typical low output level U02 = 10-3Uи.п.  The power consumption current is negligible:

I0пот = Uи.п./(Rвит.2+rті1) ≈ Uи.п./Rвит.2.

When using a power source Uи.п<Uпор + | Uпор2 | regions 2, 3, 4 of the transfer characteristic (Fig 1.15-в) are merged and remain only areas 1 and 5, for which all of the above is fair.  A zone of overlapping of region 1 and 5 appears, in which both transistors are in cut-off mode and the level of the output signal is determined by the leakage resistance of the closed transistors:

U2 = Uи.п.Rвит.1/(Rвит.1+Rвит.2),

With Rвит.1 and Rвит.2varying in fairly wide limits by chance law.  Uncertainty in the overlap area is eliminated when an external load is connected.

Fig.1.16

Transient processes in the switch on complementary MOS transistors are determined by recharging the interelectrode capacitances of the transistors VT1, VT2, as well as the capacitance of the load Cн and the parasitic capacitance of the mounting Cм (Fig. 1.16-б).

Suppose that by the time t1Ul = U01<Uпор.1. Then transistor VT1 is closed, and VT2 - in triode mode.  At the output of the switch we have a high signal level U12 = Uи.п, the capacities С'н, С'м, С'св are charged, and С''н, С''м, С''св - discharged.  The incoming input signal U1 at time t1causes an in-phase output jump:

δU12 = [(U11- U01)Cс.з]/(C'0+C''0),	 (1.22)

where Cс.з = Cс.з.1+Cс.з.2, C'0 = Cс.и.1+С'м+С'н, C''0 = Cс.и.2+С''м+С''н.  If U11>Uи.п.-|Uпoр.2|, the transistor VT2 turns out to be closed, and VT1 is open and its operating point as the output potential decreases, it moves from the pentode region to the triode.  The rate of increasing of the output voltage is determined by the discharging of the capacitance С'0, the charging С''0 and the recharging of the capacitance Cс.зthrough the open transistor VT1, the differential resistance of which rі1and with the decrease of the potential U2 also decreases.  Therefore, the change function U2(t) in Fig.  1.16-б is practically no different from the exponential and the duration of the negative edge of the output signal can be determined using the relation:

t10ф ≈ 3τ10= 3С rті1,	 (1.23)

where C = C'0+C''0+Cз.с, rті1- differential resistance of the drain of the transistor VT1.

At the back edge of the input signal at the moment t2at the output of the switch is also formed a single-phase obstacle δU2, which amplitude is determined from equation (1.22).  Transistor VT1 goes into cutoff mode, transistor VT2 - first in pentode and then with increasing output voltage U2 (voltage reduction Uc.в.2 = U2–Uи.п), in triode mode.  The positive front of the output signal is also formed by law close to the exponential and is determined by the relation:

t10ф ≈ 3τ01= 3С rТі2.	  (1.24)

The current consumption iпот (t) is also related to the recharging of the equivalent capacitance C, since the through-current through both transistors can be ignored when practically instantaneous closing one of them.

The switches by complementary MOS transistors (CMOST) provide high-speed performance with rather insignificant power consumption, which depends on the switching frequency.  They provide maximum power consumption (U12 - U02 ≈ Uи.п) and keep operating in a wide range of voltage supply: Uи.п = 3...15V. Lowest power consumption is characterized by the switches that are fed from the voltage source Uи.п< Uпoр.1 + | Uпoр.2 |, in which the active component of the consumption current in the static mode is Iпот.а= 0. Low power consumption allows the use of the switches on the CMOST as the basic elements of integrated circuits with a high degree of integration.

The mentioned advantages of such switch schemes are achieved by complication of technology of their manufacture and increase in cost, but as the technology of integrated microcircuit technology is improved, these disadvantages are becoming less significant.

- 2. Diode-transistor logic elements (DTL)

Elements such as diode transistor logic (DTL) have been widely used since the mid-sixties of the twentieth century, thanks to the simplicity of schemes, the flexibility of expanding functionality (combining outputs in the assembling OR, increasing the number of inputs by connecting external diodes, etc.).  Competitive technologies have pushed DTL elements due to higher performance, reduced power consumption and improved other operating parameters. However, the use of Schottky's high-speed diodes in DTL-elements significantly increased their performance and increased the interest of developers in the corresponding series of integrated circuits.

- 2.1. The basic element of the NAND (Sheffer-element)

Consider the diode-transistor element NAND (Fig 2.1). The scheme of the element contains the following functional parts:

• diode logic circuit AND with two inputs (R1, VD1, VD2);

• amplifier-inverter (VT1, VT2, VD3, R3, R4, R5);

• element of connection diode logic to amplifier (R2, VD3). (break down protection)

The number of similar inputs determines the functionality of such elements and characterized by input combining coefficient (fan-out) КFO. The fan-out coefficient is the number of inputs of a logic function that can be driven from a single output without causing any false output. It is a characteristic of the logic family to which the device belongs. For DTL, usually ≤ 8. [КFO =min( Iout/Iin) for high and low input signal]

At the output of the logic circuit AND, an intermediate signal is formed x’ = x1٠x2, the level of which U ' = min {U1i} + Uод.

Through the VD3 diode, this signal is received at the input of the inverter amplifier.

Fig.  2.1.

,    .

The amplifier inverter contains 2 cascades:

The input phase-inverter cascade is constructed on a transistor VT1, which forms two anti-phase signals that control the operation modes of transistors VT2 and VT3.

The output stage on transistors VT2 and VT3 provides switching of output between a general bus and a power supply.

The feature of this circuit is the use in the base circuit of the VD3 transistor diode, which is designed to increase the threshold voltage of the transistor switch and prevent it from interfering with the input signals at the logical zero level.  Resistor R2  provides a return flow of collector junction in the mode of the cutoff of the transistor VT1.

To open the transistors VT1, VT3, it is necessary to lock the diodes VD1, VD2 and open the VD3 diode.

Therefore, the threshold voltage Uпор is determined by:

Uпор=U0VD3+U0VT1+U0VT3-U0VD1≈1.4V,

where U0VD, U0VT are the threshold voltages of the corresponding diodes and transistors.

Let the input signal pass low level of input signal U10<Uпор.. Under this condition, transistors VT1, VT3 are locked, and VT2 can be in any of the possible modes: depending on the method of connecting the load: the transistor can be locked if the load is connected to a power source, is in active mode or in saturation mode, if the load is connected between the output and the common bus.

If the load is connected to the "ground", then through the open transistor VT2 can flow current, while at the output of the element set the high level of the output signal:

U21≅Uип-U0VD4-U0VT3-UR3

At typical voltage of power Uип= 5V  and U0VD=0.7V (for silicon diodes).

U21=5B-0.7B-0.7B=3.4V

At a high level at the all inputs of the inverter, when U’ > Uпор, the transistor VT1 goes into saturation mode.  Through the resistors R3 and R4 current flows, due to the voltage drop on these resistors, the transistor VT2 closes (goes into the cutoff mode), and the transistor VT3 goes into saturation mode.  At the same time the output is set to low signal level:

U'>Uпор, U20=Iн0.rкн3,

where rкн3 is the resistance of the saturated transistor VT3.

Transmission characteristic of an element - the dependence of the output voltage on the dominant input signal is  U1*:U2=f(U1). The type of transmission characteristics depends on the type and loading parameters (Fig. 2.2), where:

U1=min{U1i}.

On the transfer characteristic, four distinctive areas can be distinguished.

Fig. 2.2

In area 1, U1<Uпор the transistors VT1 and VT3 remain locked, and at their output, the logic element is maintained constant voltage:

U21=Uип-UОТ2-U0VD4

In the region, 2 transistors VT1 and VT2 are in active mode, and the circuit has a voltage transfer coefficient:

KU2≅-R3R4

In this area, the noise immunity of a logical element deteriorates at the level of the logical zero.  As it was mentioned, the threshold voltage increases due to the VD3 diode.

Uпор=U0VT1+U0VT3+U0VD3-U0VD1(2)

If there was no VD3 diode, the transistor VT1 would open due to the slightest noise at the level of the logical "0", and the inclusion of the VD3 diode necessitates the need for a resistor R2, and its resistance is selected from the condition:

R2.Ik01max≤U01T1min ,

where I k01max is the maximum thermal current, U0VT1min   is the minimum program voltage of the transistor VT1.

This follows from the condition of thermal stabilization.  At the maximum temperature, the minimum voltage at the emitter junction VT1 and the maximum thermal current Iко1 are set.

In area 3, all transistors are in active mode, so the voltage transmission factor КU3 is maximal and the transmission characteristic is sharply decreasing.

In the region, 4 transistors VT1 and VT3 are saturated, the transistor VT2 is locked and the output voltage of the logical zero is independent of the voltage U1*, but is determined by the load parameters:

U20=Iн0.rкн3 ,

where I0н – current of external load.  rкн3 – resistance of transistor VT3 in saturation mode.

- 2.2. Logic elements AND-OR-INVERT, NOR

In Fig. 2.3 is shown the example of the DTL, the element of the two-stage logic AND-OR-NOT.  It consists a power amplifier circuit, which often used in integrated circuits on bipolar transistors.

Intermediate logic signals are defined by logical expressions:

,

X″=X3⋅X4,

X*=X'+X″.

Output: Y=X*=X1X2+X3X4  implements a two-step logical function.

Fig. 2.3

Here, the first stage VT1 has two phase-inverter outputs, which control the modes of the output cascade transistors on the composite transistor VT2, VT3 and the transistor VT4. The resistor R6 provides the drainage of the thermal current necessary to ensure the temperature stability of the transistor VT3.  If the input signal is:

U1=max{min(U11,U12),min(U13,U14)}=maxU1',U1″<Uпор0

then transistors VT1 and VT4 are locked, and VT2 and VT3 are open and the output is formed by a high level:

U21=Uип-Iк01R3-UOT2-UOT3,

where UOT2, UOT3 - the voltage drop across emitter transistor transitions correspondently to VT2 and VT3.

If transistors VT1and VT4 are saturated, and VT2, VT3 are in the cutoff mode.  The output level is set to low:

U20=Iн0i2rкн.  .

In both states, the through current through transistors VT2, VT3 and VT4 does not leak, since in this circuit always one transistor is locked. Due to this, the output resistance of the DTL-element in both logical states is small. The through current through the output cascade can proceed in the transitive mode from the logical zero to the logical unit at the output.  For its limitation, a resistor R5 is included in the circuit.  The ability to load such elements reaches Kроз =10 and above.

The separate case of the scheme, which is presented in Fig. 2.3, is when the input circuits AND have only one input:

X'=X1,

X″=X3.

That responds to the implementation of the logical element NOR:

Y=X1+X3.

- 3. Transistor - transistor logic (TTL)

Transistor-transistor logic (TTL) elements are the technological development of DTL elements and are still in use.  The simplest TTL element can be obtained from the DTL element by replacing the group of input diodes VD1, as well as the displacement diode VD3 by a multi-emitter bipolar transistor (MET) with the number of emitters corresponding to the number of inputs m (Fig. 3.1).

Transmission characteristic of this scheme is similar to the characterization of the DTL element. The difference is to change the threshold voltage:

Uпор≈UКH1+U0VT1+U0VT4,

where UК.H1 - the voltage of the collector-emitter of saturated MET; U0VT1,U0VT4 - threshold voltage of transistors VT1, VT2.  Input current I1 at a high level of the input signal U11 is greater than in the DTL element, because it is the current of the transistor VT1 in the inverse active mode.

- 3.1. The basic element of the NAND (Sheffer-element)

The scheme of the basic TTL element of classic series, implementing the logical function of NAND, is shown in Fig. 3.1.

In the system of positive logic, the transistor MET with the resistor R1 in the base chain implements the logical operation "AND", and the push-pull power amplifier based on transistors VT1, VT2, VT4 (VT3 is used as a diode), performs the function NOT, provides the formation of standard logic levels of the output signal and the coordination of the TTL-element with a given load.

Fig.3.1

The mode of operation of MET and input current is determined by the dominant input signal:

U1*=minU1i,i=1,m,

as well as the input impedance of the transistor VT1.  If the potential of the base of the transistor VT1 UБ1is less than the threshold voltage U01 (for silicon transistors  U0Si≈0.7B), the transistor VT1 is in the cutoff mode and its collector maintains high potential Uk1≈Uи.п and the emitter is low UE1=UБ4≈0. Therefore, the transistor VT4 is also locked, and the transistors VT2, VT3 are open (in active mode).  When the load is depleted, current through VT2, VT3 is determined by the reverse current of the collector transition of the transistor VT4 IК.0.  Input impedance of the transistor VT1 is large (resistance to the leakage of its collector junction) and the input current is small: IБ1=-IК.01. With the increase in the potential UБ1>U01, the transistor VT1 goes into active mode, the currents IК1 and I1increase and the potential UE1 increases and the collector UК1 voltage decreases. While the potential UЕ1  is not sufficient to open the transistor VT1, the input impedance of the transistor VT1

Rвх1=rБ1+(β1+1)(ϕТ/IБ1+R3)≈R3β1ϕT/(UБ1-U01)+1,

the input current remains large IБ1=IК slightly increasing.  At the moment of opening the transistor VT4 is full emitter resistance Rвх4=rб4+(β4+1)ϕT/IЕ4||R3 and, consequently, the input impedance of the transistor VT1 sharply decreases and correspondingly increases the current Iб1.

Thus, the input impedes sharply at  UБ1≈U01+U04=Uпор≈1,4В. Therefore, for any combination of input signals, if U1*<Uпор, MET base current

IБ0=(UИ.П.-U1*-U0)/R1

flows through one or more MET straightforward emitter transitions and supports it in saturation mode.  Since the voltage collector is an open emitter in the saturation mode of the transistor UК.H.≈0, we can assume that UБ1=U1*+UК.H.≈U1*, that is the input voltage of the amplifier UБ1 is equal to the smallest of the input voltages.

If U1*>Uпор, all MET emitters are displaced in the opposite direction, while the collector is in the forward direction and MET operates in the inverse active mode.

On the collector of the saturated transistor VT1 and the base VT2 there is a low potential consisting of the voltage at the open emitter transition of the transistor VT4 and the residual saturation voltage of the transistor VT1:

UК10=UБ20=UБ4+UКH1≈UБ4.

Output voltage U20 is determined by the voltage of the collector-emitter transistor VT4 in the saturated state and proportional to the load current  I2:

U20=rкн4I2,

where rКH4 - resistance between the collector-emitter of the saturated transistor VT4.

In the case of saturation of the transistor VT1 between the base of the transistor VT2 and the emitter of the transistor VT3, taking into account the two previous formulas, the voltage UБ2-E3=UБ2-U20=UБ4+UКН1-rКН4I2.

In the worst case at idle on the way out I2=0 high-voltage UБ2-Е3  is maximum UБ2-E3≈UБ4≈0,7V. This voltage is capable of opening one emitter junction, but it is not enough to unlock two consecutive transitions of the transistors VT2 and VT3.  Therefore, an output transistor VT3 in the diode mode is introduced in the output circuit, which ensures the guaranteed locking of the transistor VT2, when VT1 is in saturation mode.

- 3.2. Modifications of LE NAND

- 3.2.1. LE with free collector

Sometimes there is a need to use for controlling the load of logic element (LE), which have a separate power sources with high consumption current. Such a load can be a relay winding, a light indicator, etc.  For this purpose, LE is used, in the collector circuit of the output transistor, which there is no resistor, and therefore it is called a logical element with a free collector.  Simplified scheme of such LE is shown in Fig. 3.2, where Rн - external load of the chip.  For this case:

Fig. 3.2

U21=Uип2-Iко4⋅Rн≈Uип2

U20=Iкн4⋅rкн4=Uип2Rн+rкн4⋅rкн4

For normal operation, the collector of the output transistor VT4 should be connected, as shown in Fig. 3.2, to the power supply through the external load circuit.  In this case, the external devices that connect to the output, can operate from other sources of power (Uип2) with higher voltage.

LE with a free collector allows the parallel connection of several LE’s to the total load (Fig.3.3).  With such a connection, if the on output of one of the elements is low potential U20, then the output of the entire system will also have a logical zero.  To provide a high level of potential U21 at the common output, it is necessary to close the output transistors of all LE’s, i.e. to set them in the state of the logical unit.  Thus, by creating a system that performs the "AND" operation, a parallel connection of several open collectors with a total load can be created.  In Fig. 3.3 gives an example of an assembly of two chips.  In this case:

| Х' | Х'' | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

X'=x1x2x3;  X''=x4x5

Y=X'⋅X''=x1x2x3⋅x4x5=x1x2x3+x4x5

Fig 3.3

Such a connection with an open collector is called "assembling AND".

- 3.2.2. LE with block

In Fig. 3.4 shows an LE with an input V, which provides, when V = 0, to disconnect the output of the chip from the load:

Fig. 3.4

If input V is applied 1: V = 1, then. Y=X1X2.

At V = 0 the output is disconnected, the transistors VT2 and VT4 are closed.  This is a state of high-resistance output. In this case, the state of output Y is determined by the external circuit.

Block inputs enable the inclusion of TTL elements on the common signaling bus.

- 3.3. LE AND-OR-NOT, NOR

The basic ICs of the TTL series include elements that implement the AND-OR-NOT, NOR logic functions.

In Fig 3.5 shown the circuit of the logic element 2-2AND-2OR-NOT.

Logical variables in the scheme are determined by the relationships:

X'=X1X2, X″=X3X4, Y'=X'+X″ , Y=Y'=X'+X″=X1X2+X3X4.

Fig. 3.5

Here the logic function AND is realized by multi-emitter transistors VT1' and VT1'', similar to the scheme NAND. The function OR is implemented by the parallel including transistors VT2' and VT2''. If at least one of them is open, through the resistors R2, R3 flows the current, which creates for the transistor VT3 locking, and for the VT5 - opening potentials on the base and on the output of the element is set to a logical zero. If VT2' and VT2'' are locked at the same time, the output level is set to U21. The number of inputs for AND may be different for each of the groups, but usually Kass.AND  ≤ 8   . In a specific case, when each of the transistors VT1' and VT1'' has one emitter, we obtain an element of one-stage NOR-logic. The number of inputs (groups) for AND is limited (Kass.OR ≤ 4 )   by considerations of speed and temperature stability, as well as the parallel inclusion of the transistors VT2' and VT2'' increases the equivalent capacity of the load of the phase-inverter cascade, while through the resistor R2 flows the total thermal current IК0 of the transistors VT2.

In Fig. 3.5 shown the outputs A, B, which can be used to connect additional external circuits that extend the logic capabilities of an element using a logic extender (Fig 3.6-a). In the circuit, shown in Fig. 3.6, the output signal is determined by the expression:

Y=X1X2+X3X4+X5X6X7.

Fig. 3.6

- 4. Emitter- coupled logic (ECL)

The digital elements of emitter-coupled logic (ECL) are based on current switches and differ from other types of   IC’s with the highest speed, but also with high power consumption.  High speed (or low switching cycle time) in ECL elements is due to the fact that bipolar transistors in these circuits operate without saturation, that is, they can be either in active mode or in cut-off mode.  Another important factor for increasing the speed is the use of low-resistance resistors in the elements that provide a quick recharge of parasitic capacities by increasing the power consumption and reducing the difference in logic signals, and thus the impedance of ECL elements.  A circuit’s tool for increasing the speed is the use of emitter repeaters that provide reloading of capacities in the load chains through small output supports.  At the same time, the load capacity increases: Кроз≤15.

Fig 4.1 shows the circuits of the basic logic element of the ECL, which implements the logic function 2NOR.

Structurally, such a circus is a bridge, to one of the diagonals that fed through the source Uип of a stable current I0. From the other diagonal (collectors of transistors VT1 and VT2), the output signal is removed.  The base of the transistor VT2 provides constant potential Uоп (reference voltage) from the source of the reference voltage.  If R1=R2=Rk and to the base of the transistor VT1 come the potential U1=Uоп<I0Rк, then when the parameters of the bridge transistors coincide, it is balanced iк1=iк2=0.5I0 і U21=U22=0.5RкI0. In this case, both transistors are in active mode and there is a transistor-amplifying cascade with an emitter connection and a symmetric (phase-inverse) output. In digital circuitry, this cascade is used in switching mode. For this, the reference voltage is chosen from the ratio of logical levels (U10, U11) control signal: Uоп≈0.5(U10+U11).

- 4.1 Basic logic element OR / NOR

Fig. 4.1

Construct a truth table for the two input signals and see how the transistors behave for each of the combinations.  Logic of the circuit:

| 0 | 0 | 0 | 1 |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 |

X'=X1+X2

X″=X'=X1+X2

Y1=X'=X1+X2

Y2=X″=X1+X2

In the circuit in Fig. 4.1, the current switch is constructed on a transistor VT2 and a group of parallel-transmitted transistors VT1 according to the number of logical inputs of an ECL element.  The total emitter current of transistors VT1 and VT2 is stabilized by high-resistor Rэ.  The reference voltage Uопis determined by the resistive divider R2, R3 voltage, which through the emitter repeater made on the transistor VT3, is fed to the base of the transistor VT2.  For the temperature compensation of the voltage in the base circuit of the transistor VT3 included diodes VD1, VD2.  Emitter repeaters on the outputs of the ECL element (transistors VT4 and VT5) provide amplification of the output signal by current and power, as well as the harmonization of the levels of input and output signals, reducing the signal levels at the outputs of the ECL element to U0≈0.7V  (lower than the potentials of the collectors of transistors VT1 and  VT2).  An external jumper connects emitter resistor R4 if necessary. This makes it possible to combine the outputs of several ECL elements into the "assembling OR" on one common resistor R4 (Fig. 4.2).

Fig. 4.2

Y3=Y1+Y2=X1+X2+X3+X4

Y4=Y1+Y2=X1+X2+X3+X4=(X1+X2)⋅(X3+X4)

Resistors RБ designed to securely lock transistors VT1 on unused inputs.  To improve the noise immunity of an element, usually the "ground" bus is divided so that the internal logical elements are connected to one bus, and the emitter repeaters to the other.  In this case, impulse interference in powerful circuits of emitter repeaters does not affect the operation mode of the switch of current.

Let us consider in more detail the principle of the basic ECL element (Fig. 4.1), which is based on the operation of the switch on the transistors VT1 and VT2.  The potential of the general emitter of the transistors VT1 and VT2 depends on the voltage of the dominant m input signals U1i:

U1*=maxU1i, i=1,m.

The potential of the transistor base VT3 is determined by the divider: UБЗ=Uип-2U0R2+R3⋅R2, and reference voltage: Uоп=UБЗ-UОЗ=const=UБ2=-1.3V.

Then the current: I0=Uоп-U0Rэ=const.

Voltage levels on collectors VT1 і VT2 while U1*=U0:

UК11=-IКO⋅RК⋅КОБ≈0V

where КОБ – the blockage factor at the input (number of inputs);

UК20=-I0⋅α⋅RК≈-0.9V.

As already mentioned, the transistors VT4 and VT5 in addition to the current gain provide the conform of input levels, reducing the potentials of collectors VT1 and VT2 by the value U0:

U21=UK2-U04, i.e. U20=-0.9-0.7≈-1.6V,

U22=UK1-U05, i.e. U21=0-0.7=-0.7V

Thus, the feature of these elements is:

• high speed;

• relatively high power consumption;

• small difference of voltage levels (U20 = -1.6V; U21 = -0.7V);

• no binding of logic levels to supply voltages.

- 4.2 Basic logic element E2CL

In the logical elements of E2CL the last drawback is eliminated (Fig. 4.3).

Fig. 4.3

The logic of the element is similar:

In this circuit, the transistor VT3 is a stable current generator I0.

Uоп.1=-Uип-U0VD1R3+R4⋅R3-U0VT5

Uоп.2=-Uоп.1-Uип-U0VD2R1+R2⋅R1+Uоп.1

I0=α⋅IЭ4=Uоп2-U0VT4-UипRЭ=const

U20=-I0⋅RK≈-0.9V

U21=-IK0⋅RK≈0V

Transistors VT1' and VT1'' perform the function of conforming the levels of input and output potentials and increase the input impedance.

The VD2 diode provides thermal stabilization of the current I0, and VD1 is the thermostabilization of the threshold level of the switching signals.

In the circuits E2CL - elements (Fig. 4.3) the matching of the signal levels is transferred from the outputs to the input, which is the m-input emitter repeater.  Speed of the E2CL-element is higher, since the equivalent input capacities of the cascade with the common collector are lower than the cascade with the total emitter, the equivalent capacity of the collector of the inverting transistor VT2 is also less and does not depend on the number of inputs. Output resistance E2CL-element Rвых=RK higher than that of ECL elements, but the branching factor Kраз does not decrease because of the large supports of the input load cells. The high level of the logic signal is practically equal to the potential of the earth, which reduces the effect of interference and facilitates the connection with logical elements such as DTL and TTL.

- 5. Logic elements on MOS transistors

Properties, electrical parameters and characteristics of logic elements on MOS - metal–oxide–semiconductor transistor are completely determined by the properties of electronic switches on which such elements are constructed.

- 5.1. LE NAND type

The basic NAND element is based on successively included MOS transistors, the number of which is determined by the required number of inputs m with the general linear or nonlinear loads, as well as on the basis of m complement pairs.

- 5.1.1. n-MOS technology

Let we implement the basic function of algebra of logic (Fig. 5.1) =X1⋅X2 .

Fig 5.1

| X1 | X2 | Y |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

The NAND logical element is constructed using two transistor switches VT2 and VT3, which connect consistently with the total load VT1 (Fig. 5.1). In this circuit, both switches will be closed, creating a path for current and providing a low output voltage  U20 only at X1=X2=1.  If one of the inputs to provide a logical zero, for example, X2=0, then the transistor VT3 will close, the current in the serial circuit of the transistors is absent and the output will be high voltage U21. Thus, the output signal Y will be obtained according to the truth table:

That is: Y=X1⋅X2.

Here, X = 0 means that the input signal U10<Uпор.  At the same time, Y = 1, which corresponds to the output potential. U21=Uип-Uпор1.

If X1 = X2 = 1, then the output LE is set to Y = 0, which corresponds, for

U20=(Uим-Uпор3)⋅Kоб⋅RiRi3+Ri⋅Kоб→0, if  Ri3>>Ri⋅Kоб.,

where Ri - differential resistance of the corresponding transistors.

Association coefficient Kоб, is not high, especially in the elements NAND (Kоб≤4), since with the increase in the number of inputs Kоб, the level of the logical "0" grows:

U20=Uим-UпорRін+Kоб⋅Rі⋅Kоб⋅Rі,

where Rін - resistance of the transistor-load (in Fig. 5.1 is VT1).

In addition, with the growth of Kоб, the duration of the front tф10 of the output signal increases:

tф10=3⋅Сн⋅Ri⋅Kоб,

Loading capacity of MOS-elements of all types is large (up to 20), because the output current in them is small.  This is due to the very large input impedance that loads the output of the MOS transistors (over than 1012 Ω).  However, it should be borne in mind that the increase in the number of load cells, as well as the increase in parallel connected input transistors, leads to an increase in loading capacities and, as a consequence, to decrease the speed.

- 5.1.2. CMOS technology

The NAND logical element is implemented by means of two CMOS (complementary metal–oxide–semiconductor) switches by parallel switching of p-channel transistors and serial connection of n-channel transistors.  Fig. 5.2 shows the electrical circuit of the two-input element, NAND.

Fig.5.2

If both input voltage are high, the transistors VT1 and VT2 are open, and VT1' and VT2' are locked. The output voltage is close to zero, U20≈0, i.e Y=0.

If at least one of the inputs has a logical zero, for example X1=0, then the transistor VT1 is locked, and the transistor VT1' is opened and therefore U21=Uип.

For all cases, the current consumption  Iпотр1,0=0.

Duration of the output signal fronts:

tф01=3τ01=3Cн⋅Rip,

tф10=3τ10=3Cн⋅Rin⋅Kоб>tф01,

where Rір, Rіn — differential resistance of an open p- and n-channel transistor respectively;

Сн – equivalent capacity load.

The main advantage of the CMOS-elements is that in both of their static states, the current from the power source does not almost flow, so the power consumed is very small.  However, during operation of an element, the current charges unwanted capacitances, so the dynamic power consumption is proportional to the switching frequency and can be several stags higher than the static one.

- 5.2. LE NOR type

In logic elements NOR electronic switches are combined into a parallel group of inputs: m≤Kоб. Resistance of group of multipled transistors is determined by the least from parallel links, i.e.  by a transistor on the breech-block of that the most is given from entrance voltages.

- 5.2.1. n-MOS-technology

Fig. 5.3 shows the circuit that implements the function Y=X1+X2:

Fig. 5.3

If both inputs have a low logic zero (X1=X2=0) level, transistors VT1 and VT2 are locked and the output will have a high voltage level: Y = 1.

If to apply a high level of the logical unit to at least one of the inputs, for example, the VT2 transistor opens, that is output Y through a small resistance Ri will be connected to the common bus and the output voltage will be low, that is Y = 0. Thus, the output signal Y is determined by the table states:

| X1 | X2 | Y |
| --- | --- | --- |
| 0 | 0 | 1 |
| 1 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 1 | 0 |

The logical "unit" at the output (Y = 1) responds to the voltage. U21=Uип-Uпор.3.

Logical "zero" at the output (Y = 0) responds to the voltage. U20=US-Uпор.3Ri2+Ri⋅Ri=Iпот0⋅Ri.

These elements have a drawback - they consume current (Iпот0) with all combinations of input signals except one: X1 = X2 = 0.

- 5.2.2. CMOS-technology

The NOR logical element is realized by parallel switching of n-channel transistors and serial connection of p-channel transistors. Fig 5.4 shows the electrical circuit of two-input element NOR with elements of protection of inputs from static interferences.

Fig 5.4

If on entrances it is given voltages of low levels, X1=X2=0,  then the transistors VT1 and VT2  are locked, and the transistors VT1' and VT2' are opened.  The current in the power circuit is very small.  The voltage at the output is close to the supply voltage: U21=Uип, i.e. Y = 1.

If at least one, of the inputs is a logical unit, for example X1=1, then the transistor VT1 opens, and the transistor VT1' closes.  The current in the power circuit will still be small, and the output voltage, that is, U20≈0, Y = 0.

Diodes and resistors at each input provide protection of inputs from the static voltage on the external outputs of the chassis.

Diodes VD1 and VD2 protect inputs from negative pulsation at the logical zero level (U10≪0), and VD3 diodes - from positive pulsation at the level of the logical unit (U11≫Uип).

The current in the static mode is zero for all cases. Iпотр10=0.   The duration of the front of the output signal

tф01=3τ01=3Сн⋅Rip⋅Kоб ,

tф10=3τ10=3Сн⋅Rin,  tф10<tф01.

For internal logic elements of any microcircuit, protection against static electricity is not provided (Fig. 5.5).

Fig. 5.5

- 5.3. Elements of two-level logic

The logical elements of the two-stage logic are constructed in the form of combinations of successive and parallel groups of MOS-transistors.

- 5.3.1. n-MOS technology

Fig. 5.6

If there is no jumper between VT2 and VT4 (dotted) in the circuit (Fig. 5.6), each of the pairs of sequentially connected transistors VT1, VT2 and VT3, VT4 in the positive logic system implements a logical multiplication operation and currents (X ', X ") flow  , if X1X2=1 (or X3X4=1).  If any of the X', X" currents or their sum pass through the load transistor VT5, the low potential is set at the output, that is, the logical element implements the function AND-OR-NOT. The table of output states has the form:

| X' | X″ | Y |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

Then in the absence of a jumper: X'=X1⋅X2, X''=X3⋅X4 and the output function looks like:

Y=X'+X″=X1X2+X3X4.

A logical element over, that will realize this function, is shown on in Fig. 5.7- а.

In the presence of jumpers, the output function changes:

Y=(X1+X4)⋅(X2+X3).

Graphic representation of such an element is shown in Fig 5.7-б.

Fig. 5.7

- 5.3.2. CMOS-technology

In the digital circuitry, logical elements that implement the operation of the arithmetic addition of two variables are widely used.  To do this, must be calculated a function Y=X1X0+X1X0, that can be implemented on the CMOS-transistors:

Y=X1X0+X1X0=X1⊕X0=X1~X0.

The circuit that implements the function EXCLUSIVE OR on the CMOS-transistors is shown in Fig.  5.8.

Fig. 5.8

| X1 | X0 | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

General rule for construction of two-stage logic on CMOS-transistors:

To build a two-level logic on the CMOS you need:

• bring the function to the form of MDNF or MCNF;

• for each intermediate module (MDNF), the serial circuit of the transistors is switched between the output and the power source by the number of arguments in the intercom.  The argument included in the minterm without inversion controls the n-channel transistor, and the inverse argument is p-channel;

• the number of such consecutive connections corresponds to the number of minterms.  Thus, the upper part of the circuit is constructed;

• complementary transistors are included between the output and the ground;

• each successive chain is supplemented by a parallel (and vice versa) circuit of transistors of opposite conductivity;

• the parallel links of complementary group inter themselves are included consistently

When constructing an inverse MDNF, the chains between the output signal and the power source, as well as the signal and the ground, change places.

- 5.4. Buffer Amplifiers

In order to provide a higher branching ratio at the output Kраз without reducing the speed or increasing the power consumption from the power source in the n-MOS technology, special buffer amplifiers with inverting (or without inverting) the signal are used.

Fig 5.8

If the connections between the first cascade of the amplifier (transistors VT1, VT2) and the second (transistors VT3, VT4) are implemented by dashed lines, then a signal  is  formed  as  Y=X.

In case of realization of connections between cascades, the shown continuous lines are form a signal .

The consumption current is determined from the expression Iпот.=UипRi2+Ri1  and may be very small, since the transistors with the parameters Ri1, Ri2 ≫ Ri3, Ri4 are selected.

In a two-stage buffer amplifier, the first stage (transistors VT1, VT2) is an inverter with a high-level load in the VT2 flow circuit, but with a low load capacity. The second cascade on transistors VT3, VT4 is executed on a two-stage circuit and is controlled by phase-in signals from the input and output of the inverter. Therefore, the through current from the power supply Uип. through the transistors VT3, VT4, which in the open state (Ri3, Ri4) have low resistance, is excluded. As a result, with low current consumption (mainly due to the first cascade) buffer amplifiers provide an overload of equivalent load capacity through small resistance of transistors VT3, VT4 in the open state.  At the same time, the load capacity without loss of speed increases up to Kраз.≤30.

Duration of the output signal fronts:

tф01=3⋅Сн⋅Ri3,

tф10=3⋅Сн⋅Ri4.

- 6. Integral injection logic Elements (І2L)

- 6.1. Base element

The desire of the developers of the elemental base to reduce the energy consumption and increase the degree of integration of logic elements on bipolar transistors, while maintaining their main advantage - high speed, led to the creation of integrated injecting logic (I2L).  Technology I2L provides packing density of elements (more than 1000 elements per 1 mm2), which exceeds the MOS technology.  By power, the scattered I2L elements can be compared with the CMOS while maintaining high-speed (tзд.р=5 ns) inherent in bipolar integrated microcircuits.

The quality criterion of technology is the switching energy: the less energy, the better technology.  For I2L technology, this energy is:

WK=Pпот⋅tзд.р.ср=50⋅10-12J

The above advantages of the I2L elements are achieved by the exception of the resistor circuits, which imply insignificant scattering power, the operation of bipolar transistors in unsaturated mode, small parasitic capacitances and a small difference in logical levels.  Use in the I2L elements of Schottky diodes allows you to increase the speed (tзд.р=0.1ns) without increasing the power consumption.

The basic logic I2L element (Fig. 6.1) contains the p-n-p transistor VT2, called the injector, which acts as a stable current sources , and the multi-collector n-p-n transistor VT1, which performs the function of the inverter.  The base type n transistor-injector is connected to the emitter type n transistor-inverter.  Similarly, it was possible to combine the collector of the injector and the base of the inverter, having conductivity type p.

Fig. 6.1

Transistor-injector VT2 is constantly in active mode and the current in each of its k collectors is:

Iи=α⋅Iэk=αk⋅Uип-U02RЭ=const,

where U02 is  threshold voltage for the emitter junctions of the transistor VT2, k - the number of collectors of the injection supply,  - the coefficient of current transmission in the circuit with the general base. The input voltage determines the state of the emitter junction of the transistor VT1.

If the number of collectors k, then Iэ is evenly distributed over all k collectors.

When U11>U01 (i.e., X = 1), the transistor VT1 is also open, voltage on its collectors equal

,

which responds to the value of the output logical variable Y = 0.

When U10≤U01 (i.e., X = 0), the transistor VT1 is locked and voltage on its collectors equal

U21=Iu⋅Rн,

where Rн is the load impedance. Most often, the load is the base of the next I2L element. Then U21>Uон  So:

Y1=Y2=X

- 6.1.1. Input characteristic (dependence I1=f1(U1))

Input current of the I2L element I1=IБ-IИ, therefore the input characteristic of this element is the input characteristic of the bipolar transistor displaced by the Fig. 6.2 axis of currents by the magnitude Iи (Fig. 6.2).

Fig.6.2

From the characteristics, it is clear that I10≈-IИ,I11≈0.  The logical voltage ΔU1=U11-U10=UБ.И.-UК.Н.≈0,6V , difference is characterized by voltage levels at the closed and open emitter transistor VT1.  Its temperature dependence determines the connection of the impedance of the I2L circuits with the ambient temperature.  When increasing temperature UБ.И. and  ΔU1 proportionally decreases and, accordingly, decreases stability.

- 6.1.2. Output characteristic

Output characteristic of the I2L element (Fig. 6.3) in the mode of transistor cutoff VT1 is the return current of the collector junction and does not depend on the output voltage, and in the open state, it is a line of saturation of collector characteristics.  The current of the injector  flowing through the saturated resistance rK.H. transistor determines the output level of the logical zero. U20≈IІrK.H.=UK.Н., and a high level at the output is formed when the current  flows through the load, that is, through the input circuit of the next element.  Thus, the low level V at the input of the I2L element responds to a high level U11=0.65V at its output, and vice versa, a high level at the input V responds to the level U20=0.05V.O

Fig. 6.3

On the characteristic, it is possible to construct a loading line, which is an input characteristic for other elements.

- 6.2. Realization of logical functions OR, NOR, AND, NAND

Logic functions in the I2L technology are realized with the help of assembly of n-p-n transistor-inverter collectors.

In Fig. 6.4 shows circuit with two inputs and two outputs:

Fig. 6.4

The work of such a circuit is characterized by a table of states of outputs:

| Х1 | Х2 | Y1 | Y2 |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 |

As can be seen from the table, the corresponding outputs implement the functions NOR (Y1) and OR (Y2):

Y1=X1+X2, Y2=Y1=X1+X2

For the implementation of the basic logic AND, NAND we have to use additional inverters for input signals (Fig. 6.5). As can be seen from the circuit:

Y3=X'+X''=X1+X2=X1⋅X2, Y4=Y3=X1X2

Fig.  6.5

Additional transistors in the path of the logic signal increase its delay, therefore, the logical elements AND, NAND in І2L-technology are more inertial than the logical elements OR, NOR, and it is desirable to do without them for the development of high-speed circuits.

In Fig.6.6 shown an example of implementing the following function:

Y=X1X2+X1X2=X1⊕X2=X1+X2+X1+X2=(X1+X2)(X1+X2)

X'=X1+X2, X''=X1+X2, Y=X'+X'', Y=X1⊕X2.

Fig.6.6

To implement an arbitrary function in the I2L technology it is necessary:

• bring the implemented function to the minimum CNF;

• using a double inversion and de Morgan's theorem to express a function with the NOR operation;

• implement the NOR function by combining the collectors of transistors that are switched by a two-stage logical structure;

• determine the number of injection pump units and build an appropriate injection currents generator.

Consider an example that illustrates the sequence of synthesis operations for a logical combinational circuit for I2L technology.

Let the function Y (X3, X2, X1, X0) be given in the form of the perfect disjunctive normal form (PDNF):

Y = (0,2,5,6,7,8,13,14,15).

We will find the MCNF:

| X1X0 | 00 | 01 | 11 | 10 |  |
| --- | --- | --- | --- | --- | --- |
| X3X2 |  |  |  |  |  |
| 00 | 1 | 0 | 0 | 1 | =(X2+X0)(X2+X1+X0)(X3+X2+X1) |
| 01 | 0 | 1 | 1 | 1 |  |
| 11 | 0 | 1 | 1 | 1 |  |
| 10 | 1 | 0 | 0 | 0 |  |

Let's transfer the function to the basis NOR:

Y=(X2+X0)(X2+X1+X0)(X3+X2+X1)=X2+X0+X2+X1+X0+X3+X2+X1=
=X'+X''+X'''

The intermediate variables X ', X' ', X' '' are realized by the basic elements NOR of the first stage, and the outputs of the first degree are arguments (inputs) of the second stage. In Fig. 6.7 is shown the synthesized circuit:

Fig.6.7

Checking the correctness of the circuit:

• all the bases of the transistors must have an injector supply;

• between any input and output, the signal must pass no more than 3 transistors.

- 7. Code Converters (CC)

Converters are designed to transfer codes submitting one form to another. For example, when entering information into computers successive should convert decimal numbers into binary, and the derivation of information on indicators or printers should binary or binary-coded decimal codes in turn control codes decoder, LED or LCD display panel, print engine.

- 7.1. Synthesis converter codes

May need to build a converter binary 3-bit gray code. Write a table of conformity:

| Inputs |  |  | Outputs |  |  |
| --- | --- | --- | --- | --- | --- |
| 22 | 21 | 20 | gray code |  |  |
| X2 | X1 | X0 | Y2 | Y1 | Y0 |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 |

Each of the functions defined in a certain number of input variables. To find these functions and minimize Yi use Karnaugh maps, writing in the cells Y value cards for each of the sets:

|  | Х1Х0 X2 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| Y0 = | 0 | 0 | 1 | 0 | 1 |
|  | 1 | 0 | 1 | 0 | 1 |

|  | Х1Х0 X2 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| Y1 = | 0 | 0 | 0 | 1 | 1 |
|  | 1 | 1 | 1 | 0 | 0 |

|  | Х1Х0 X2 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| Y2 = | 0 | 0 | 0 | 0 | 0 |
|  | 1 | 1 | 1 | 1 | 1 |

Variants converter code Gray binary code shown in Fig.7.1-7.3:

Fig.7.1

Fig.7.2

Fig.7.3

Qualified CC graphically depicted as follows:

For the synthesis of CC n -bit input code in the m -bit source, follow these steps:

- make a table matching the input and output codes;

- for each of the m outputs find MNDF, MCNF function of n input variables;

- using identical transformation MNDF and MCNF result obtained function to the form that is convenient for the joint implementation of a given technology;

- values obtained joint functions.

- 7.2. Encoder

A special case of a code converter is an encoder - this device provides the issuance of a certain code in response to the excitation of one of the inputs. Those. an encoder is a CC that converts a unitary code, or the code “1 of N”, into a code required by the conditions of the problem. In this case, N determines the number of inputs of the encoder.

Encoders are widely used to convert decimal numbers and alphabetic characters in binary code when entering information in computers and other digital devices. The encoder in the IMC type field is denoted by characters CD (CoDer).

Consider the example of building encoder to convert decimal numbers in code 8421. The input - a binary variable X0 ... X9, which are formed by pressing the corresponding switch input device. The variables are independent and can build 210 = 1024 combinations of input, but if imposed restrictions prohibiting pressing two or more switches, then out of 1024 input combinations there are 10 valid. The input code corresponding to this limit is called unitary or code “1 of N”. Encoder in which all input variables have the same priority called non-priority. In our case, such variables N = 10.

| X9 | X8 | X7 | X6 | X5 | X4 | X3 | X2 | X1 | X0 | Y3 | Y2 | Y1 | Y0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 0 |  |  |  |  |  |  | 1 |  | 0 | 0 | 0 | 1 | 0 |
| 0 |  |  |  |  |  | 1 |  |  | 0 | 0 | 0 | 1 | 1 |
| 0 |  |  |  |  | 1 |  |  |  | 0 | 0 | 1 | 0 | 0 |
| 0 |  |  |  | 1 |  |  |  |  | 0 | 0 | 1 | 0 | 1 |
| 0 |  |  | 1 |  |  |  |  |  | 0 | 0 | 1 | 1 | 0 |
| 0 |  | 1 |  |  |  |  |  |  | 0 | 0 | 1 | 1 | 1 |
| 0 | 1 |  |  |  |  |  |  |  | 0 | 1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |

Directly from the table can record the output function:

Fig. 7.4

In Fig.7.4 given encoder implementation on LE NOR (а) and NAND (б), and therefore conditional graphics:

Fig. 7.5

The limit on the number of keys pressed often is unacceptable and need to build encoder so that while you press multiple switches he responded only to the oldest (or youngest) of them. Converters of this type of code called a priority encoder. They implement the conversion code “k of N” in the necessary code.

Consider the example of construction of a priority encoder “k of 10” in code “8421”. The table according to priority encoder, in which the input variables with the highest number is the maximum priority values ​​of input variables to the right of the diagonal 1 should not determine the source code:

| X9 | X8 | X7 | X6 | X5 | X4 | X3 | X2 | X1 | X0 | Y3 | Y2 | Y1 | Y0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | * | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | * | * | 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | * | * | * | 0 | 0 | 1 | 1 |
| 0 | 0 | 0 | 0 | 0 | 1 | * | * | * | * | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 0 | 1 | * | * | * | * | * | 0 | 1 | 0 | 1 |
| 0 | 0 | 0 | 1 | * | * | * | * | * | * | 0 | 1 | 1 | 0 |
| 0 | 0 | 1 | * | * | * | * | * | * | * | 0 | 1 | 1 | 1 |
| 0 | 1 | * | * | * | * | * | * | * | * | 1 | 0 | 0 | 0 |
| 1 | * | * | * | * | * | * | * | * | * | 1 | 0 | 0 | 1 |

Priority encoder can be built based on non-priority encoder that converts code “1 of 10” in code “8421” when the pre-convert the input code “k of 10” to code “1 of 10”.

Fig. 7.6

Let the converter output variables “k of 10” to “1 of 10” through X9, ..., X0. Variable input has maximum priority, so do not depend on other input variables. Any other variable output Хі takes the value one ifand none of the “senior” inputs  not served logical unit, i.e.:

Similarly,

Fig.7.7 shows scheme for realizing this transformation. Its advantage is the uniform propagation delay on all outputs and disadvantage - the need to use of multi-circuits NAND.

Fig. 7.7

If it is not imposed strict requirements for speed, e.g. keyboard, code converter “k of 10” to “1 of 10” can be done with less equipment (Fig.7.8).

Fig. 7.8

In this scheme, signal priority prohibition applies to the older entry through junior series connected elements OR, because the whole duration of the conversion to code “1 of 10” is determined by the output X0 maximum delay time settings.

- 7.3. Decoder

Convert any input code in the code “1 of N” converters perform code called decoder. The most widely used decoder targeted systems storage devices, information display devices from computers and other digital devices to external devices visualization and documentation of alphanumeric information. You need to give a signal “1 of N”, such as cathode gas discharge indicator elements sample character printers and so on.

- 7.3.1. Linear decoder

Synthesis decoder structure, like any other converter codes beginning with writing table and matching input source.

Consider the example of transformation (decryption) binary code “8421” in a unitary code “1 of 10”.

| “8421” |  |  |  | “1 of 10” |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Minimization |  |
| 0 | 0 | 0 | 0 |  |  | N |
| 0 | 0 | 0 | 1 |  |  |  |
| 0 | 0 | 1 | 0 |  |  |  |
| 0 | 0 | 1 | 1 |  |  |  |
| 0 | 1 | 0 | 0 |  |  |  |
| 0 | 1 | 0 | 1 |  |  |  |
| 0 | 1 | 1 | 0 |  |  |  |
| 0 | 1 | 1 | 1 |  |  |  |
| 1 | 0 | 0 | 0 |  |  |  |
| 1 | 0 | 0 | 1 |  |  |  |

m

Each output function need to Karnaugh map and use it to get it minimized expression.

| Х1Х0 Х3Х2 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | Y0 | Y1 | Y3 | Y2 |
| 01 | Y4 | Y5 | Y7 | Y6 |
| 11 | * | * | * | * |
| 10 | Y8 | Y9 | * | * |

If the number of inputs and m outputs N number of decoder related:

,

then outputs defined for all sets and binary decoder called complete. At decoder called incomplete. Unused combinations (star map Carnot) to minimize the function exits.

Pictogram of the decoder shown in Fig.7.9-а, and its conditional pictogram on Fig.7.9-б.

Fig. 7.9

This type is called linear decoder. They are characterized Single decryption input m-bit codes using m-post of the door logic elements. Linear decoder provides a conversion code with minimum delay and used in most high-speed digital circuits. However, with increasing bit input code m rapidly growing load each of the inputs and the number of buildings decoder IMCs to implement. The linear structure commonly used if .

- 7.3.2. Pyramidal decoder

If the number of inputs then to reduce the number of IMCs housings decoder performed by the multi-scheme. The first step pyramid decoder - a simple linear decoder with the number of outputs (Fig.7.10). Each successive level of driven more input variables, allows to double the number of exits and get etc., i.e. k-step full pyramidal decoder has a number of outputs , and .

|  |  | - N |
| --- | --- | --- |

Fig. 7.10

The advantage is a regular structure are similar dual-input elements.

The disadvantage of such decoder are delay proportional to the number of digits, uneven load inputs, which increases with the number of stages.

- 7.3.3. Matrix decoder

If the number of inputs , it is better to build decoder on a matrix structure. When paired m number of rows and columns of the matrix is and matrix output valves Square comes. At odd m input variables divided into  and .

In both cases, selection rows and columns (Fig.7.11), in which nodes are connected dual-input valves, as well as row and column decoder used linear or pyramidal decoder. This type is called decoder or rectangular matrix.

Their advantage is that they provide a relatively small delay as the linear decoder and regular structure.

Fig. 7.11

- 7.4. Multiplexers

In digital devices often need to transfer digital information from various devices m to n receivers through a public channel. For this inlet channel, setting device called a multiplexer which, in accordance with the code for channel connects to one of the m sources. To connect inputs is set to release a single table that case looks like:

|  |  | Y |
| --- | --- | --- |
| 0 | 0 |  |
| 0 | 1 |  |
| 1 | 0 |  |
| 1 | 1 |  |

Address inputs A0, A1show which of the inputs X0, X1,X2,X3 must be connected to the output Y.

Inputs X0, X1,X2,X3 assign addresses and thus set the working condition of the switch. Then adopted addressing output function for the example is as follows:

We have MDNF that requires minimization. We can present it in basis NAND or NOR. Realization on the elements NOR shown in Fig.7.12.

Fig. 7.12

Here: X0. …X3- information inputs, A0, A1- address, name entry that can be connected, C - gating input (optional).

Gating input required to disable the output of the multiplexer at the time of change of address to prevent unauthorized connection release (random) input signals.

If C = 1: Y = f (A, X), and when C = 0, Y = 1.

At the time we submit a change of address C = 0, currently locked out because yak.ne depends on the information inputs.

In Fig.7.13 are conventionally pictogram multiplexer

Fig. 7.13

Since select one of N inputs, it is advisable to use a decoder. The signals control the decoder logic switches, allowing transmission of information via only one of them. While this scheme will look like multiplexer (Fig.7.14):

Fig. 7.14

Consider some use Schematic multiplexers. It is obvious to use multiplexer as the transducer parallel m-bit binary code consistent. It's enough to give the inputs of the multiplexer parallel code and then gradually change the address code in the required sequence. However, to avoid false signal multiplexer output strobe pulse for switching addresses must disable output of inputs.

Multiplexers can be used to build logic functions of several variables in a disjunctive normal form.

- 7.5. Demultiplexer

Demultiplexer - combinational devices that implement data bus connecting a single input to one of the N outputs according to address code (i.e. “1 of N”). Demultiplexer includes an address decoder.

Demultiplexer logic operation for the case N = 4 is given in the table below.

|  |  | X |
| --- | --- | --- |
| 0 | 0 |  |
| 0 | 1 |  |
| 1 | 0 |  |
| 1 | 1 |  |

A qualified graphic image demultiplexer:

Simple demultiplexer scheme that implements the specified conversion table is shown in Fig.7.15

Fig. 7.15

Demultiplexer can also build based on decoder. The scheme of the demultiplexer is shown in Fig.7.16.

Fig. 7.16

- 7.6. Realization of logic functions multiplexers

Multiplexers can be used to build logic functions of several variables in a disjunctive normal form.

Let set general view multiplexer functions:

As an example, take some function:

and we realize it on multiplexer.

We can assign one input variable information, and others - address. So identify A1=X2, A0=X0 then a comparison with expressions minterms  and  follows: , , ,

Multiplexers build on this feature:

Fig. 7.17

If we use multiplexer with 2 information inputs, then this function can be represented as:

For 2-inputs multiplexer, output function is:

If then a comparison  and  follows:

,

Realization of functions Y on 2-inputs multiplexer shown in Fig.7.18.

Fig. 7.18

Synthesis of logic devices based multiplexer can be formalized.

The algorithm synthesis device for realization logic based multiplexer includes the following:

- To obtain MDNF, fill a given function Karnaugh map (or Veitch diagram).

- Karnaugh map should highlight areas by the number of multiplexer inputs information that we use. The variables that retain their values ​​within the selected area is targeted for a multiplexer, and the rest - information.

- Each region must find minimal information on the form variables to control information multiplexer inputs.

- With identical shape are minimal changes to the form convenient for joint implementation.

- Implementation of schemes for each information input multiplexer.

Consider as an example the synthesis of logic device that implements the function YX3,X2,X1,X0=(1, 2, 5*, 6, 10*, 11, 12, 13). In parentheses are the decimal equivalents of binary numbers X3,X2,X1,X0. That is 11002 = 1210, which responds minterm X3 X2 X1X0. Similarly 210→X3 X2 X1X0,  610→X3 X2 X1X0 and so on. For data sets of arguments the function takes the value 1. Numbers with an asterisk means that these sets of arguments feature vague, i.e. sets of arguments X3X2X1X0 = * and  X3X2X1X0  = *  Y functions can be assigned any value that simplifies implementation.

- Step 1:

Y=X3 X2 X1X0+X3 X2 X1X0+ X3X2X1X0 *+X3 X2 X1X0+X3X2X1X0  *+X3X2X1X0+X3X2X1 X0+X3X2X1X0

- Step 2:

- Step 3:

For 4-inputs information multiplexer, Karnaugh map divide into 4 address areas so that address will be variables X3 and X1. Assign value A1= X3, A0 = X1. Then the numbers of address areas respond to the targeted areas shown on the Karnaugh map.

- Step 4:

Find the management functions of each information input of multiplexer:

; ; ; .

- Step 5:

In this case, no conversion is needed.

- Step 6:

Building a scheme that implements a given function Y ():

Fig. 7.19

- 7.7. Combination devices shift

The need to shift the digital data occurs, when the numbers are normalized, performing arithmetic operations on them, etc. If for one working cycle necessary to shift by only one digit to the left or right, then this operation, combining it with the function of storing information conveniently performed using shift registers to trigger. Where a single cycle shift must perform an arbitrary number of digits in either direction, convenient to use combinational logic device based multiplexers. Number of multiplexers required is equal to the output bit binary number.

Starting to build a table shifting device that communicates address code multiplexer that connects to each of their outputs discharge numbers shift. Bit address code multiplexer p determines the maximum possible number of shift per clock cycle:.

Suppose you want to build a combinational device that adjusts shift according to the table:

| Address |  | The outputs of the shift device |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |
| 0 | 1 |  |  |  |  |
| 1 | 0 |  |  |  |  |
| 1 | 1 |  |  |  |  |

If a combinational device shift numbers carried left, with senior level lost. Imagine this unit shift as a set of multiplexers, each of which forms a discharge source:

Fig. 7.20

This option is, , , ; all address inputs are combined and controlled variables simultaneously, .

If , , The outputs are connected to inputs .

If , (Shift by one digit), the output  to appear  the inputs .

If , (Shift of two bits)  there are signals of the inputs .

And when , respectively, offset by 3 bits and outputs connected to inputs  multiplexers.

Alternating bits can be in any order, which is an important advantage multiplexer.

Conventionally, a graphic representation of the Raman shift device (bullpen) shown in Fig.7.21.

Fig.7.21

Advantages:

- In combinational device shift performed on any number of bits per clock cycle (it depends on the sequence in which to switch addresses).

- Very simply chosen direction of displacement

- During the shift could easily modify the code shift.

Drawback: is a complicated device.

- 8. Combinational adders (CA)

Combinational adders are designed to perform arithmetic operations of addition and subtraction of single-and multi-numbers (operands). Multi- digit adder consists according numbers of single-digit adders.

- 8.1. Half adders

Single-digit adder whose input received two single-digit numbers A and B, and the output is also generated single-digit number S and transfer amount and carry P, called half adder. That half adder is a device which implements arithmetic addition of two single-digit binary numbers.

The truth table for half adder can record feature amount calculation unit S and transfer overcrowding P in the most significant bit (MSB):

| A | B | P | S |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 |

S=A⋅B+A⋅B=A⊕B=A⋅B+A⋅B=A⋅B+A⋅B=A⋅B+A⋅B=P+A+B

P=A⋅B=A+B

Implementation half adder based on identical transformations is shown in Fig.8.1 and Fig.8.2.

Fig. 8.1.

Fig. 8.2.

Conditionally half adder graphic is displayed as shown in Fig.8.3:

Fig. 8.3

- 8.2. Full adders

If single-adder implements the addition of three single-items A, B, C and thus forms the feature amounts S and P transfer function, it is called full adder. Full adder described by the following table:

| A | B | C | P | S |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Find the features MDNF amount of S & P and transport together carry some options. The Karnaugh Map for the three variables A, B, C:

|  | Х1Х0 Х2 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| P = | 0 | 0 | 0 | 1 | 0 |
|  | 1 | 0 | 1 | 1 | 1 |

P=A⋅B⋅C+A⋅B⋅C+A⋅B⋅C+A⋅B⋅C=C⋅(A⋅B+A⋅B)+A⋅B=

=C⋅S'+A⋅B=C⋅S'⋅A⋅B, S'=A⋅B+A⋅B

|  | Х1Х0 Х2 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| S = | 0 | 0 | 1 | 0 | 1 |
|  | 1 | 1 | 0 | 1 | 0 |

S=A⋅B⋅C+A⋅B⋅C+A⋅B⋅C+A⋅B⋅C=A⋅B⋅C⋅A⋅B⋅C⋅A⋅B⋅C⋅A⋅B⋅C=

=C⋅(A⋅B+A⋅B)+С⋅(A⋅B+A⋅B)=C⋅S'+С⋅S'=C⊕S'=C⊕A⊕B

Realization of this adder on element NAND shown in Fig.8.5:

Fig. 8.5.

Conditionally pictogram full adder shown in Fig.8.6:

Fig. 8.6.

Adder circuit elements on the EXCLUSIVE OR and NAND represented on Fig.8.7:

Fig. 8.7.

Advantage circuit using logical elements exclusive OR and NAND is easy implementation. A disadvantage - a significant delay in the output of exclusive OR elements.

- 8.3. Subtractor

Subtractor is a device that implements the subtraction of two single-digit numbers Ai, Bi and borrow from LSB of Vi-1with forming of difference signals Di and borrow Vi of MSB. Let needed from  Ai subtract Bi and Vi-1 where  Vi-1 borrow unit of the MSB:

_Ai

_Bi

Vi-1

ViDi

Then the possible values for feature differences Di and functions borrow Videfined below truth table. According to the truth table subtractor write the expression for the difference Di and functions borrow Vi:

| A | B | Vi-1 | Vi | Di |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Then the function numbers the difference is:

|  | Ai Bi Vi-1 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| Di= | 0 | 0 | 1 | 0 | 0 |
|  | 1 | 1 | 1 | 1 | 0 |

Di=Ai⋅Bi⋅Vi-1+Ai⋅Bi⋅Vi-1+Ai⋅Bi⋅Vi-1+Ai⋅Bi⋅Vi-1=

=Vi-1⋅(Ai⋅Bi+Ai⋅Bi)+Vi-1(Ai⋅Bi+Ai⋅Bi)=

=Vi-1⋅S'+Vi-1⋅S'=Vi-1⊕S'=Vi-1⊕Ai⊕Bi.

Function borrow Vi:

|  | AiBi Vi-1 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| Vi= | 0 | 0 | 1 | 0 | 0 |
|  | 1 | 1 | 1 | 1 | 0 |

Vi=A⋅B⋅Vi+A⋅B⋅Vi+A⋅B⋅Vi-1+A⋅B⋅Vi-1=

=Vi-1⋅(A⋅B+A⋅B)+A⋅B=Vi⋅S'⋅A⋅B

Using identical expressions for Di and Vi we can realize the full single-digit subtractor with different element bases.

Scheme of such subtractor on elements NAND and EXCLUSIVE OR is shown in Fig.8.8:

Fig. 8.8.

A comparison of expressions Siand Di shows that Si≡Di. Therefore, subtractor can build on a full adder, replacing subtracted numbers in reverse compiling code:

Ai

+Bi

+Vi-1

Pi*Si*

For such an operation truth table is:

| A | B | Vi-1 | P* | Si* |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |

Comparing the value functions Pi*, Si* with Vi and Di tables for subtractor and adder can be noted that Pi*=Vi, Si*=Di.  Therefore the output of the adder transfers Pi* necessary to invert.

This subtractor based on adder shown in Fig.8.9:

Fig. 8.9.

- 8.4. An algebraic adder

An algebraic adder can be used to add or subtract as necessary. The combination of addition and subtraction operations need additional signal F, which sets the mode combiners using controlled inverter:

Y=X,F=0X,F=1,

where F- control signal.

As such controlled inverter can be used EXCLUSIVE OR element:

| Y=XF+XF |  |
| --- | --- |

If F = 0, we have Y=X without inversion signal transmission, while F = 1 we have inversion of the input signal on output: Y=X.

Such circuit of algebraic adder shown in Fig.8.10

Fig. 8.10

At F=0 device performed the operation of addition of Ai, Bi, Ci and became sum Si and transfer Pi. While F=1 device performed the operation of subtraction and became difference Vi and borrow Di.

- 8.5. Multi-bitadder

Adder of the two multi-bit numbers can be implemented using single-bit adders. So can be realized the consecutive or parallel adder.

Suppose you want to add two 4-bit numbers A and B:

+A4A3A2A1B4B3B2B1P4S4S3S2S1

Scheme of consecutive addition of the transfer Pi is shown in Fig.8.11:

Fig. 8.11

Consecutive adder requires minimal equipment costs. However, the time of adding is proportional number bit of operands. By delay duration unit transfer as serial adder can be used in the relatively slow operating digital devices.

Cycle duration T of serial adder is proportional to n bits adder and time of delay of the transfer in one single-digit adder tзд.p.n.:

T=n.tзд.p.n.

In parallel m-bit adder amount of each bit, Si defined as the logical sum 2i + 1 arguments. Therefore, the complexity of implementing output functions rapidly increases with the number output level. In addition, equipment costs are rising rapidly with increasing bit operands. Cycle duration T and complexity of realization of parallel adder depend on how the unit of the transfer would bring.

To construct a parallel circuit transfer function we introduce a notation function of transparency Ai⊕Bi=Hi and function of generating transfer Ai⋅Bi=Gi. Using the tool transparencyHi and generating functions Gi for any output bit of adder can write:

Si=Ai⊕Bi⊕Pi-1=Hi⊕Pi-1,

Pi=Ai⋅Bi+(Ai⊕Bi)Pi-1=Gi+HiPi-1.

Based on these ratios can write:

S1=H1⊕P0,  P1=G1+P0H1

S2=H2⊕P1,  P2=G2+P1H2=G2+G1H2+P0H1H2

S3=H3⊕P2,  P3=G3+P2H3=G3+G2H3+G1H2H3+P0H1H2H3

S4=H4⊕P3,  P4=G4+P3H4=G4+G3H4+G2H3H4+G1H2H3H4+P0H1H2H3H4.

That is, each of the outputs the multi-bit adder can be recorded as DNF of the functions of generation and transparency. In this case, all output variables S1, S2, S3, S4 and P4 = S5 are calculated simultaneously. This ensures minimum delay its conclusions, and therefore the highest speed.

Parallel transfer scheme the more complex, the more arguments defining feature.

Section of 4-bit adder graphically is shown in Fig.8.12.

Fig. 8.12

The complexity of parallel transfer scheme rapidly increases with the number of bits adder, therefore multi-combiners are often divided into sections, which are implemented in parallel transfer. Between sections the transfer sections can be implemented consistently. The best find multi-bit adder division into 4-bit sections.

In the presence of 4-bit sections, the multi-number operand A can divided into tetrads:

&  A8A7A6A5    A4A3A2A1&+                 +  B8B7B6B5     B4B3B2B1 +                 +                                                      P4                    P0   P8  S8 S7S6S5  P4 S4S3S2S1

Scheme of connection sections adder with serial transfer between the sections shown in Fig.8.13:

Fig. 8.13

- 8.6. Binary-coded decimal adder

The calculator, device registration and conversion of digital information, represented in decimal form, commonly used arithmetic devices with binary-coded decimal operands.

The basic element of a decimal adder may be 4-bit binary adder section that should realize the addition of two decimal digits k-th level: AK4AK3AK2AK1 and BK4BK3BK2BK1 considering transfer Pk-1of previous tetrads (k-1)-th decimal places. At the exit k-th section must be the result amounts  SK4SK3SK2SK1 and transfer Pk  to a tetrad (k + 1)-th decimal places.

The table shows the values Si' and Pi' derived when adding binary-decimal numbers (tetrads) using binary adder sections and values Si,Pi, which will ultimately be received.

| Σ10 |  | 24 | 23 | 21 | 20 | Binary-coded decimal |  |  |  |  | Σ10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P' | S4' | S3' | S2' | S1' | P | S4 | S2 | S3 | S1 |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |
| 2 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 2 |
| 3 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 3 |
| 4 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 4 |
| 5 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 5 |
| 6 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 6 |
| 7 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 7 |
| 8 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 8 |
| 9 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 9 |
| 10 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 16 |
| 11 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 17 |
| 12 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 18 |
| 13 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 19 |
| 14 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 20 |
| 15 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 21 |
| 16 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 22 |
| 17 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 23 |
| 18 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 24 |
| 19 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 25 |

As the table, shows summing binary-decimal operands giving sum SK4'SK3'SK2'SK1' and transferPK', which starting from the 10-th row do not respond to the addition of decimal numbers. In lines 16…19 need only correction of sum, and in lines 10…15 - the sum and transfer. Correction must convert the sum  SK4'SK3'SK2'SK1' in the sum SK4SK3SK2SK1 and transfer PK' in Pk.

Correction of transfer is performed by logical adding:

PK=PK'+Pcor,

where Pcor - corrective term, that is set to logical “1” on sets  SK4'SK3'SK2'SK1' 10-15 lines of the table, that is:

| S'2S'1 S'4S'3 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 0 | 0 |
| 01 | 0 | 0 | 0 | 0 |
| 11 | 1 | 1 | 1 | 1 |
| 10 | 0 | 0 | 1 | 1 |

Pcor= =S4'⋅S3'+S4'⋅S2'.

Then  PK=PK'+Pcor=PK'+S4'⋅S3'+S4'⋅S'=PK'⋅S4'⋅S3'⋅S4'⋅S'.

The need of correction result is always arises in PK= 1. In this case, to get the correct result should to intermediate result SK4'SK3'SK2'SK1' add code 01102 = 610, that is:

Σ2-10=Σ2+P(0110)2- (binary-coded decimal binary + number of 6 to 10 combinations).

Implementation of section binary-decimal adder based on two sections of the binary adder shown in Fig.8.14:

Fig. 8.14

- 8.7. Combinational Multiplier

Due to the high speed, combinational adders are widely used in various devices handling digital information. One possible application is the multiplication unit based on combinational adders.

Let we need multiply of two binary numbers A4A3A2A1 andB4B3B2B1. To take product П8П7П6П5П4П3П2П1 need do arithmetic multiplication:

Here products of the type AiBj determined using conjunction of relevant variables.

Multiplication of two numbers A and B can be done using the addition and shift. Some works are uniquely determined by multiplying number A4A3A2A1for the next bit Bi multiplier B. Each subsequent single product must shift by one digit compared to the previous. The final product comes with sequential addition of intermediate products.

Implementation multiplier based on the single-digit adders shown in Fig.8.15:

Fig. 8.15

The main advantage of combinational multiplier is high speed, which is not related to clock synchronization device and determined only by the signal delay in the logic elements.

Combination multipliers can be used effectively in the construction of digital filters to perform the necessary calculations of fast Fourier transform in microprocessor systems.

- 9. Digital comparators

Comparator - a combinational device that performs the functions of the ratio of two or more operands (code combinations that are involved in arithmetic expressions).

For two operands A and B are functions:

, ,

and their superposition:, .

Most importantly comparator detects equality arguments that A = B. Universal comparator must identify all possible related arguments.

- 9.1. Single-bit digital comparator

Functions of the relationship between single-bit operands are presented in the table below:

| A | B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 0 | 1 | 1 |

The functions , , can be realized as follows:

,

,

.

On fig.9.1-a, б are presented some realization that matched logical expressions, and on the fig.9.1-в - pictogram of one-bit comparator.

Fig. 9.1

- 9.2. Multi-bit digital comparator

Suppose there are a 3-binary number, Compare it with the number . Then we get the relation for =, > and <,:

The realization of these functions F3> and F3= shown on Fig.9.2 and Fig.9.3:

Fig. 9.2

Fig. 9.3

As seen from the scheme, the delay signal  is proportional to the number of digits that are compared. To decrease delay time of output signal functionsF3>,F3<should lead to the kind of DNF (CNF) and minimized.

After minimization for F3>we get expressions:

The logical scheme of functions F3> is shown on fig.9.4. This scheme provides a minimum delay output signal and therefore maximum performance. Scheme for realization function F3< will be similarly, if the relevant variables Ai and Bi to swap.

Fig. 9.4

If to merge the circuits that implement functions F3=,F3>,F3<, then the digital comparator to compare two 3-binary numbers will be (conditional graphic images on Fig.9.5):

Fig. 9.5

9.3. Digital comparator based on adder

The ratio of the two numbers can be found, if from the number A4A3A2A1 deduct number B4B3B2B1 and analyze the difference D4D3D2D1. That is the same if to A4A3A2A1we add the number B4B3B2B1in reverse code:

→

The table shows the possible situations on outputs of adder and the corresponding function relationships:

| 1 | * |  |
| --- | --- | --- |
| 0 | 1 |  |
| 0 | 0 |  |

Here F4==S4S3S2S1- function for equality of 4-bit numbers, which is a conjunction output signal of the adder.

The scheme, which realize presented in the table expressions is shown in Fig.9.6.

Fig. 9.6

9.4. Sectioned digital comparator

Multi-bit comparators are typically combined on the basic sections. Most often as a base can be used parallel 4-bit comparator with three outputs F3>, F3=,F3<. In addition to numbers A and B on its inputs get bits of the binary signals  F3>, F3=,F3<, which representing the result of comparison in younger tetrads of comparable numbers.

Suppose given two numbers A and B. Present them as combine of senior AC, BC and junior AM, BM  tetrads:

| , |  |  |
| --- | --- | --- |
| Ac |  | AМ |
| . |  |  |
| BС |  | BM |

Function is the result of equal of senior and junior tetrads:

.

For the functions  and can take the expressions:

,

.

Section 4-bit comparator (Fig.9.7) became as an integrated circuit (Fig.9.8-a). Sectioned comparator for multi-bit numbers can build as series circuit of 4-bit sections (Fig.9.8-б).

Fig. 9.7

Fig. 9.8

- 10. Flip-flops (triggers)

Flip-flop is a regenerative triggered device with two or more stable states that are switched according to the state information inputs. In addition to information, flip-flop can have special   input for synchronization and other control inputs. In our scientific and educational literature such devices often called as triggers. Therefore further we will use also this term for such class of devices.

10.1. Asynchronous RS-triggers

In asynchronous trigger states changes in consequence of the events on the information inputs. Such devices have two information inputs: R (from word “Reset”) and S (from word “Set”).  Active level on input S switch RS-trigger over to state “1”, and active level on input R switch RS-trigger over to state “0”.

10.1.1. RS-trigger based on elements NOR

Circuit of the RS-trigger, based on the logic elements NOR, shown on Fig. 10.1.

Fig. 10.1

Two-logic element NOR closed in the positive feedback loop: the output of the logic element NOR with output  connected with one of the inputs of the other logic element, and the output of the second connected with one of   the inputs of the first.  Available inputs are used as informational inputs R and S of trigger.  By definition, the input signal S switch over trigger in the state "1" and the input R switch over trigger in the state "0". For identification of state of trigger, take level on output Q (direct). The second output of trigger P is called inverted because normally it is the opposite state of the output Q.

The logic of the trigger can be represented as a state table:

| R | S | Q | P |  |
| --- | --- | --- | --- | --- |
| 0 | 0 |  |  | saving mode |
| 0 | 1 | 1 | 0 | installation 1 |
| 1 | 0 | 0 | 1 | installation 0 |
| 1 | 1 | 0 | 0 | trigger gap relationships |

On the output element NOR is logical zero if at least one of the inputs given logical “1”. Directly under the scheme can determine that when R = 0, S = 0 due to feedback between any logic elements state of trigger (= 1 or= 0) is stable. This combination of control signals corresponding to the storage mode.

The combination of   R = 0, S = 1 translates to trigger a logical”1” if he was in a state of logic “0”, or if it was a logical “1”, keeps it in this state. This is installation mode in "1".

The combination of R = 1, S = 0 retains state of the trigger if it was “0”, or transfers them to “0” if the trigger was before them in state “1”. This is installation mode in "0".

The combination of R = 1, S = 1 is prohibited. With this combination trigger condition is not determined, as in this case Q = P = 0 and output signals   are not due to connection between logic elements. Therefore, this regime called as trigger gap relationships.

Boolean equation that describes the logical trigger conditions can be written with a state table or map Karnaugh:

| RnSn Qn-1 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | ? | 0 |
| 1 | 1 | 1 | ? | 0 |

Qn=

=Qn-1 .Rn+SnRn=

Qn-1+Sn(Sn+Rn)

Installation time trigger:  .

10.1.2. RS-trigger based on logic elements NAND

Functional diagram of asynchronous RS-trigger-element NAND shown in Fig.10.2:

Fig. 10.2

It can be obtained directly from the circuit elements in NOR, using the principle of duality. According to this principle, any logic circuit built on elements of NOR circuit can be replaced by the elements and NOR, if this signals to the inputs and outputs replaced by their opposite meaning:

.

Karnaugh map describes the state of triggers:

|  |  | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | ? | 1 | 0 | 0 |
| 1 | ? | 1 | 1 | 0 |

=Sn.Rn+Qn-1Rn=Qn-1+Sn(Sn+Rn)

Mode of storage is provided on condition that the control signals. Setting "1" is performed while changing with logical “1” to “0”, and setting a "0" - when changes  of the logical “1” to “0”,.

Combination is prohibited. With this combination, state of trigger is not identified.

10.2. Synchronous RS-triggers

Synchronous triggers include information inputs S and R rather than input for synchronization C, which serves short pulses on this input determined time of transfer trigger in new state and allows the entry of new information in the trigger. All synchronous triggers are switched simultaneously.

10.2.1. RS-trigger based on logic elements NAND

Functional diagram of a synchronous RS-trigger with data inputs A, B and input synchronization C and contains an asynchronous trigger on elements 1, 2 and schema synchronization (elements 3, 4, Fig.10.3).

Fig.10.3

Input and output signals for asynchronous trigger ratios are determined by:.

Information signals on the R and S only cause the information to be recorded. While C = 0, A = B = 1, the trigger is in storage mode. Recording information in the trigger clock starts after the appearance of the C = 1. If C = 1, whichever is recorded in the trigger logic unit (S = 1) or a logical zero (R = 1), a trigger is set according to the "1" or "0".

State of trigger for different combinations of R and S on inputs shows the Karnaugh map:

|  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 0 | 0 |
| 01 | 1 | 1 | 1 | 1 |
| 11 | 1 | 0 | ? | 1 |
| 10 | 0 | 0 | ? | 1 |

=Cn.Qn-1+Qn-1.Rn+CnSnRn=Qn-1+Cn(Qn-1+Sn)(Sn+Rn+Cn)

10.2.2. RS-trigger based on logic elements NOR

Functional diagram for a synchronous RS-trigger based on logic elements NOR shown in Fig.10.4.

Fig.10.4

Information signals on the R and S define the information recorded. Until,   the trigger is in a storage mode. Recording information in the trigger begins after the appearance of the clock pulse . If , so depending on what is written in the trigger logic “0” () or logic “1” ()  trigger is set according state.

State of   trigger for different combinations of R and S on inputs shows the Karnaugh map:

|  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | ? | 1 | 0 | 0 |
| 01 | ? | 1 | 1 | 0 |
| 11 | 1 | 1 | 1 | 1 |
| 10 | 0 | 0 | 0 | 0 |

=CnQn-1+Qn-1Rn+CnSnRn=Cn+Qn-1Qn-1+Sn(Cn+Sn+Rn)

A graphic representation synchronous RS-trigger shown in Fig.10.5 (a - on NOR, б - on NAND).

Fig.10.5

10.3. Varieties of RS-triggers

10.3.1. R-trigger

R-trigger - this variety of RS-trigger, which reacts on the combination of input signals S = R = 1 with transfer to "0".

Transfer of R-trigger described accordance to map Karnaugh with equation:

|  |  | 00 | 01 | 11 | 10 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 0 | 0 | 0 | 0 |  |
|  | 01 | 1 | 1 | 1 | 1 | =CnQn-1+Qn-1Rn+CnSnRn |
|  | 11 | 1 | 0 | 0 | 1 |  |
|  | 10 | 0 | 0 | 0 | 1 | =Qn-1+CnCn+Rn(Qn-1+Sn) |

The circuit, which implements this equation in basis of logic elements NAND, shown on Fig.10.6.

Fig.10.6

Through communication input logic element 3 of 4 with simultaneous release of the managing action C = S = R = 1 output 4, which is set B = 0, blocking other inputs element 3 and the signal S = 1 trigger rejects. Therefore, the combination of S = R = 1 sets the trigger in the state "0".

10.3.2. S-Trigger

S-trigger is a variant of RS-trigger, which reacts on the combination of input signals S = R = 1 with transfer to "1".  Transfer of S-trigger described accordance to map Karnaugh with equation:

|  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 0 | 0 |
| 01 | 1 | 1 | 1 | 1 |
| 11 | 1 | 0 | 1 | 1 |
| 10 | 0 | 0 | 1 | 1 |

=CnQn-1+Qn-1Rn+CnSn

=Qn-1+CnQn-1+SnCn+Sn+Rn

The circuit, which implements this equation in basis of logic elements NAND, shown on Fig.10.7.

Fig.10.7

Through communication output of logic element 3 to input of logic element 4 with simultaneous release of the managing action C = S = R = 1 output 3, which is set A = 0, blocking other inputs element 4and the signal R = 1 trigger rejects. Therefore, the combination of C = S = R = 1 sets the trigger in the state "0".

10.3.3. E-trigger

E-trigger is a variant of RS-trigger, which on restricted combination of C = S = R = 1 does not respond, that keeps previous state: Qn = Qn-1.

|  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 0 | 0 |
| 01 | 1 | 1 | 1 | 1 |
| 11 | 1 | 0 | 1 | 1 |
| 10 | 0 | 0 | 0 | 1 |

State asynchronous E-trigger described with characteristic equation accordance to map Karnaugh:

=CnQn-1+Qn-1Rn+SnQn-1+CnSnRn=Qn-1+CnQn-1+Sn(Qn-1+Rn)(Cn+Sn+Rn)

The circuit, which implements such equation in the basis of logical elements NAND, shown on Fig. 10.8.

Fig.10.8

Additional inverters in case of C = S = R = 1 block with signals  and  logic elements 3 and 4, the outputs of which are supported with the level A =B = 1, which corresponds to storage mode previously recorded information.

10.4. RS-trigger “Latch” type

RS-trigger type of “Latch" consist the asynchronous trigger on logic elements 1 and 2, which is controlled by signals A and B, and the rest – are the synchronization circuit elements.

Circuit of RS-trigger type of “Latch" is shown on Fig.10.9-a. Besides the main trigger on logic elements 1 and 2 circuit includes triggers for synchronization: trigger on elements 3-4, trigger on elements 4-5 and trigger on elements 5-6. A graphic representation of synchronous RS-trigger type of “Latch" shown on Fig.10.9-б.

Fig.10.9

In front-synchronized RS-triggers type of “Latch” information signals S and R can be switched at any time, but the trigger records the state corresponding to the combination of input signals S and R immediately after the corresponding positive clock front on input C. Usually such trigger can have inputs of asynchronous installation S*, R*.

When C = 0 at the outputs of the elements 4, 5 we have A = B = 1, so the main trigger (on elements 1,2) is in the storage mode of the previous state. In addition, the triggers on elements 3-4 and 5-6 are set to the state corresponding to the levels at the information inputs. That is shown in the table below.

|  |  | C = 0 |  |  |  |  | C = 1 |  |  |  |  | Regime |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | E | F | Qn | A | B | E | F | Qn |  |
| 0 | 0 | 1 | 1 | 1 | 1 | Qn-1 | 0 (1) | 1 (0) | 1 | 1 | ? | Unspecified |
| 1 | 0 | 1 | 1 | 0 | 1 | Qn-1 | 1 | 0 | 0 | 1 | 0 | Setting "0" |
| 0 | 1 | 1 | 1 | 1 | 0 | Qn-1 | 0 | 1 | 1 | 0 | 1 | Setting "1" |
| 1 | 1 | 1 | 1 | 0 | 0 | Qn-1 | 1 | 1 | 0 | 0 | Qn-1 | Storage |

When switching C = 0 → 1 signal synchronization circuit switch according to the table when C = 1. As seen from the table, the main trigger when C = 0 does not react to events on the inputs because A = B = 1 corresponds to storage mode. At constant C = 1 the main trigger is also not responsive to the input switching because level of A = 0 blocks the elements 3,5 or B = 0 blocks the elements 4,6 or E = F = 0 blocks the elements 4,5. Thus, the main trigger can switch only when the signal C switch from "0" to "1", that means the positive front clocks C. The triggers synchronized by clock Front, also known as “untransparent”. The functioning of the trigger described by characteristic equation (with S*R*=1):

Qn=CnQn-1+Qn-1Rn+CnSnRn=Cn+Qn-1Qn-1+Sn(Cn+Sn+Rn)

Pictogram of the untransparent RS-triggers is shown on Fig.10.9-б. Risk synchronization input C indicate a response on the positive front.

10.5. RS-trigger MS (Master-Slave) type

This trigger (Fig.10.10-a) contains two stages: input trigger M (from “Master”, first stage) and output trigger S (from the “Slave”). The untransparency of the trigger is provided with counter-phase levels clock synchronization and of two stages. This at any level on the input C is one of two triggers is in storage mode, which eliminates the transmission of information from inputs S, R directly to outputs of the trigger, . At C=0 trigger of a stage M is in storage mode, and the signal allows overwriting information from trigger M to stage S. When C = 1 the trigger M is setting in according to inputs S, R, and the stage S is in storage mode ( = 0). Updating the information on the outputs and happens in 2 steps, so these triggers are called push-pull and denote by two letters T (Fig. 10.10-б).

Fig.10.10

State of RS-trigger type MS represented in Karnaugh map and described by the characteristic equation that derived from it:

|  | CnQn-1 | Sn-1Rn-1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 | CnQn-1+Qn-1Rn-1+Sn-1CnRn-1 =Qn-1+CnQn-1+Sn-1(Cn+Sn-1+Rn-1) |  |  |
| Qn | 00 | 0 | 0 | ? | 1 |  |  |  |
|  | 01 | 1 | 0 | ? | 1 |  |  |  |
|  | 11 | 1 | 1 | 1 | 1 |  |  |  |
|  | 10 | 0 | 0 | 0 | 0 |  |  |  |

Due to the untransparency of the RS-triggers of type “Latch” and MS, it is allowed to include feedback   and  (Fig.10.11-а). In this case only one input C left free, which switches the trigger to the opposite state at the front of the clock (Fig. 10.11-б). This is a T-trigger which works as a frequency divider on 2:.

Fig.10.11

10.6. D-triggers

D-triggers include sequential devices with one information input D (from word Delay) and clock input C, which are set to a state due to the logical signal level at the input Dn = (0,1 ) and are paused between the clock pulses in the information storage mode (Qn = Qn-1).

Operation of synchronous D-trigger described with static Karnaugh map and the corresponding characteristic equation:

| CnDn | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |  | 0 |
| 1 | 1 | 1 | 1 |  | 0 |

=CnDn+Qn-1Cn

=Qn-1+Cn(Cn+Dn)

Implementing synchronous D-trigger satisfying the resulting equation shown on Fig.10.12-a:

Fig.10.12

If the on input of synchronization act level C = 0, which is dominant for the elements 3 and 4, their outputs set the levels A = B = 1 and do not depend on the status of the information input D. That provide the storage mode of the asynchronous trigger.

If C = 1 D information input determines the state of the output element 3: , which in turn determines the level inverse output element 4. When D = 0 is set to zero trigger condition:0 (). When D = 1 trigger is set in state.  That means, that  in  D- trigger written information presented to the input D to the setting of synchronizing C = 1. Thus, information on the outputs of D-trigger appears with delay of the information on input D, due to delayed clock pulses on the information signal C and delay time switching of logic elements 1-4.

Graphic representation of such transparent D-trigger is shown on Fig.10.12-б.

10.7. D-trigger “Latch” type

D-trigger type of “Latch” that synchronized with positive front of signal C is shown in Fig.10.13-a:

Fig.10.13

It is built on three triggers, including triggers for synchronization on logic elements NAND 3-5 and 4-6 and the basic cell synchronization circuit for storing information on logic elements NAND 1-2.

Switching output status,  and of control signals A, B, E, F due of positive front of the impulse shows next table:

| D | C = 0 |  |  |  |  | C = 1 |  |  |  |  | Trigger mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | E | F | Qn | A | B | E | F | Qn |  |
| 0 | 1 | 1 | 0 | 1 | Qn-1 | 1 | 0 | 0 | 1 | 0 | Setting "0" |
| 1 | 1 | 1 | 1 | 0 | Qn-1 | 0 | 1 | 1 | 0 | 1 | Setting "1" |

As the table shows, when C = 0 switch of level on  the input D affects only the logical state of the levels E and F triggers synchronization scheme:  if D = 0, E = 0, F = 1, while D = 1 we have F = 0, E = 1. Therefore one of the triggers synchronization circuit is in a stable condition, while the other trigger is in mode break ties at the level of "logic 1" on both outputs.

On the positive front of the clock pulse, the trigger, which was previously in the mode of breaking the trigger connections, goes into a normal stable state and at the inputs of the main trigger are formed mutually inverse logic levels. If D = 0, then A = 1, B = 0 and the trigger is set to Qn=0; if D = 1, then A = 0, B = 1 and the setting is Qn=1.

Characteristic equation that describes a static state "latch" is similar transparent D-trigger:  Qn=CnDn+Qn-1Cn=Qn-1+Dn(Cn+Dn)

A graphic representation of D-trigger type of “Latch” is shown in Fig.10.13-б.

In the untransparent   D-trigger may be included feedback  (Fig.10.14-а). This one trigger input C acts as a T-input and that is counter to mod2 and frequency divider (Fig. 10.14-б):.

Fig.10.14

10.8. D-trigger MS (Master-Slave) type

MS-type push-pull structures are also used to eliminate cross-control in D-triggers. Two-stroke D-trigger is built because on two D-triggers that are synchronized with anti-phase levels (Fig. 10.15-a).

Fig.10.15

A graphic representation of D-trigger type of MS is shown on Fig.10.15-б.

Through to the synchronization with the anti-phase clock C and  the recording of new information in the triggers of the stages M and S is fundamentally separated in time, which eliminates the transfer of information from the input D to the outputs.

If C = 0 trigger of stage M is in storage mode, and the output trigger (stage S) level  = 1 is allowed to overwrite the contents of the stage M in stage S. Switching clocks  and  triggers changes the mode M and S: M trigger switches to record information of the door D, and trigger S - saving mode information by the previous step. In trigger M is possible recording mode that is not synchronized during the sync interval C = 1. After switch of the clock C () input D is locked and in the main trigger S rewritten completely steady state of stage M (Q = P).

Static operation of the trigger type D-MS described with Karnaugh map and the equation:

|  | Qn-1 | SnDn-1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 | =CnQn-1+CnDn-1=Qn-1+Cn(Dn-1+Cn) |  |  |  |
| Qn = | 0 | 0 | 1 | 0 | 0 |  |  |  |  |
|  | 1 | 0 | 1 | 1 | 1 |  |  |  |  |

10.9. Universal JK-trigger MS type

JK-trigger is the bistable device with two information inputs J (from the English  “Jerk”)  and K (from the English “ Kill”), which in the case of input combination J = K = 1 switch trigger in the opposite position, and in any other combination they function as RS-trigger inputs, whose role inputs S and R inputs are performed under J and K: .

Functional diagram JK-trigger MS type is shown in Fig.10.16-a. The principal difference between JK-trigger of two-stroke RS-trigger is to use feedback from the outputs, to the inputs of logic elements 1,2. Through feedback banned for RS-trigger combination S = R = 1 for JK-trigger J = K = 1 switch clearly to the opposite state.

In the circuit on Fig. 10.16, instead of additional inverter that generates a signal used signals A, B,  which  blocking logic elements 3,4 when writing a new state to the stage M (when A = 0 or B = 0).

States of such triggers describe Karnaugh map and the characteristic equation:

|  | CnQn-1 | Jn-1Kn-1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |  |  |  |  |
| Qn = | 00 | 0 | 0 | 1 | 1 |  |  |  |  |
|  | 01 | 1 | 0 | 0 | 1 |  |  |  |  |
|  | 11 | 1 | 1 | 1 | 1 |  |  |  |  |
|  | 10 | 0 | 0 | 0 | 0 |  |  |  |  |

Two-stroke JK-trigger (Fig.10.16-) is not critical to the duration of controlling and timing signals.

Fig.10.16

Fig.10.17-a shows how to the use JK-trigger as D-trigger, and Fig.10.17-б shows how to the use JK-trigger as T-trigger.

Fig.10.17

Additional inverter on input allows JK-trigger as a simultaneous two-stroke D-trigger. If J and K inputs connect to a constant level of A = 1, we get T-push-pull trigger (Fig.10.17-b). This is the universality of JK-flip-flops, which are widely used in the construction of parallel and serial registers, calculating various devices, adders etc.

10.10. JK-trigger "Latch" type

The circuit of JK-trigger of type "Latch" is shown on Fig.10.18-a. The main trigger built on logic elements 1,2. The scheme of synchronization consist the triggers on logic elements 3-5, 3-4 and 4-6. The logic elements 7,8 with the information inputs respectively J and K connected feedback signals from the outputs  and  so that when J = K = 1 provided switching of the trigger to the opposite state.

Modes trigger condition and intermediate logical variables A, B, E, F when switching clocks C = 0> 1 are shown in the table below:

| Jn | Kn | Qn-1 | C = 0 |  |  |  |  | C = 1 |  |  |  |  | Regime |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | E | F | Qn | A | B | E | F | Qn |  |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | Storage "0" |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | Storage "0" |
| 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | Setting "1" |
| 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | Setting "1" |
| 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | Storage of "1" |
| 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | Setting "0" |
| 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | Storage of "1" |
| 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | Setting "0" |

| J | K | C = 0 |  |  |  |  | C = 1 |  |  |  |  | Regime |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | E | F | Qn | A | B | E | F | Qn |  |
| 0 | 0 | 1 | 1 | 0 | 0 | Qn-1 | 1 | 1 | 0 | 0 | Qn-1 | Storage |
| 0 | 1 | 1 | 1 | 0 | Qn-1 | Qn-1 | 1 |  | 0 | Qn-1 | 0 | Qn = 0 |
| 1 | 0 | 1 | 1 |  | 0 | Qn-1 | Qn-1 | 1 |  | 0 | 1 | Qn = 1 |
| 1 | 1 | 1 | 1 |  | Qn-1 | Qn-1 | Qn-1 |  |  | Qn-1 | Qn-1 |  |

As can be seen from the table of states of the trigger, at C = 0 the variables A, B, which control the state of the main trigger on logic elements  1-2,  retain the value A = B = 1, which corresponds to the storage mode, since for the logical elements NAND  dominant is level "0". In this case, the variables E and F take the values determined by the input signals J, K, depending on the state of the trigger   Qn-1 before switching the level at the clock input.

Switching the clock signal C  causes the transition of the triggers of the timing circuit on logic elements 3-5 and 4-6 in a stable state and the corresponding installation (or storage of the previous state Qn-1) of the main trigger on logic elements 1-2.

JK-trigger can change state only when the transfer clock signals, that is, its positive front. At constant C =1 or C = 0 JK-trigger does not respond to information inputs J and K. At C = 0 stored A = B = 1, and at C = 1 levels E = F = 0 block the inputs of the trigger on logic elements 3, 4, or A = 0 blocks logic elements 4, 5 or B = 0 blocks logic elements 3,6. This ensures the opacity of the "latch" and only works on the positive front. The JK-trigger may have asynchronous control inputs R*, S* for pre-installation.

States of JK-trigger (when R* = S* = 1) describes the Karnaugh map and the characteristic equation:

|  | CnQn-1 | JnKn |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |  |  |  |  |
| Qn = | 00 | 0 | 0 | 0 | 0 |  |  |  |  |
|  | 01 | 1 | 1 | 1 | 1 |  |  |  |  |
|  | 11 | 1 | 0 | 0 | 1 |  |  |  |  |
|  | 10 | 0 | 0 | 1 | 1 |  |  |  |  |

Pictogram of such circuit is shown in Fig.10.18-б.

Fig.10.18

Figure 10.19 shows how to enable the JK-trigger as a D-trigger (Fig.10.19-a) with one information input D and as a T-trigger (Fig.10.19-б) as a single-digit counter.

Fig.10.19

- 11. Registers

Registers - are devices for receiving, storing, simple transformation and transmission of multi-codes. In simple transformations implied decimal numbers to shift specified number of digits and convert serial binary code in parallel and parallel to serial. The basic elements of the registers are triggers that are complemented combinational logic elements for various connections between bits of the register and to manage the reception and transmission of operands. The main functional using of the registers is memory for multi binary numbers.

Depending on how the reception and transmission of binary information is distinguished parallel, serial, ring registers and registers in the code of Johnson. Technical parameters registers determined by the parameters of the basic functional unit of the trigger and the number of operands.

In serial registers input / output information realized through one entrance and one exit with successively shift of number. Because serial registers called shift register. For single cycle, information entered or displayed, shifted by one digit to the right or left. Shift registers that implement the command management information offset to the right or left, called reversible.

Offset number sold between neighboring states overwriting register triggers shift in direction. Thus, each bit register simultaneously receives information from the previous level and transmits the information to the next. To avoid the phenomenon of racing these processes should be separated in time. This is achieved by the inclusion of elements delays link between discharges or using triggers stroke.  Parallel registers are the main functional elements for building operational storage devices.

Serial-parallel registers have one information input for sequentially entering a number in shift mode and output valves for issuing an n-bit number in parallel. Such registers convert the sequential code to parallel. These registers perform convert sequential code into parallel.

In parallel-serial registers code information is entered in parallel in one clock cycle after clock input valves and are displayed sequentially on one level in each clock interval. Thus the transformation operation is implemented parallel to serial code.

Universal registers combine the capabilities of the above types of registers and, in addition, provide a shutdown mode inputs and outputs (third logical state) register of common information bus switch of   places the inputs and outputs of the register and thus switching functions of receiving / sending information to general information bus.

Technical parameters registers determined by the parameters of the basic functional unit of the trigger and the number of   operands.

- 11.1. Parallel registers

Usually in parallel registers commonly used simple asynchronous RS-triggers and synchronous RS- and D-triggers. On Fig.11.1, 11.2, 11.3 are variants of parallel registers, different type of triggers and composition of input / output information tires. Register-based asynchronous RS-flops (Fig.11.1) before entering the next n-bit number from inputsrequires prior reset all flip-flops in the zero state. Reset signal is held. Entering information happens in case when there is a signal,  . If on somethe entrance is then and given a trigger (with inverted control) switches to state "1". If on the input then  and such trigger saves the state "0". Output data from the register realized when occurs signal   , which defines the output status . If B = 0, all outputs are set at 'logic 1', while we have .  The main disadvantage of such register is the need for pre-treatment register, therefore   the update information need two cycles.

Fig.11.1

The register 0n Fig.11.2-a is also based on asynchronous RS-flops pre-treatment is not necessary, because the update information in it is the setting triggers a state of "1" and "0" in one clock cycle. To do this, the input of the register requires twice as many logical elements and communication lines.

The diagram on Fig.11.2-a shows a method of issuing information in forward code (if command B1=1) AND / OR in reverse code (if command B2=1). Asynchronous RS-triggers in combination with input logic (Fig. 11.1 and Fig. 11.2-a)   essentially form variants of synchronous RS-triggers.

Fig.11.2

Figure 11.3 shows the scheme of a parallel register based on synchronous D-triggers. Here as inputs, the logical elements included in the D circuit of the triggers are used. The input of information occurs at the synchronization interval at C=0. As output, the logical elements AND-OR-NOT are used, which form the output signals according to the expression:

,

that is at,  and information from the register issued in direct code, while at , in reverse.

If the expression is identically converted to a form:

,

the output register circuit (Fig.11.3) can be put into logical elements EXCLUSIVE OR (Fig.11.2-b). Choose how to enter information in the register or withdrawal usually due to performance requirements and conditions of implementation of the register in digital devices.

Fig.11.3

- 11.2. Shift register

Let us consider series-parallel and parallel-serial registers that need to perform the operation with sequential shift input and / or output. The offset of the number is realized by overwriting states between adjacent register triggers in the shift direction. Thus, each bit of the register simultaneously receives the information from the previous digit and transmits the information to the next. In order to avoid the phenomenon of racing, these processes must be separated in time. This is achieved by the inclusion of delay line elements in the communication between the digits or the use of two-stroke triggers. In integrated circuits, static shift registers are built mainly on two-stroke D- and JK-triggers.

When unidirectional shift enough information to realize direct links between inputs and outputs previous next MS-trigger (Fig.11.4, 11.5).

In the case of D-triggers (Fig.11.4) to transfer information between bits necessary link between  and  in the case of JK-flip-flops (Fig.11.5) double bond:  with J i+1 and  with .

Fig.11.4

In both structures, the principle of operation and parameters of these registers are similar. The input of information can occur sequentially from the input X, with the subsequent shift of the information to the right one digit for each clock pulse C or in parallel from the inputs AI on the asynchronous number recording signal to the previously cleared register (asynchronous reset of the triggers to the state "0").

Fig.11.5

Fig.11.6

On the positive front of the clock pulse is recorded information from the inputs D and X in the triggers of the first stages of M. The output levels of the triggers remain the same (timing diagram in Fig. 11.6). On the negative front of the clock, the inputs of the D-triggers of degree M are blocked and the information recorded therein is transmitted to the flip-flops of degree S. The output levels of the D-flip-flops are switched. Thus, a sequential register with a shift of the number to the right (input - X, output - Y), serial-parallel (inputі - X, outputs - ) and parallel-serial (inputs – A, output Q3) is implemented. In addition, when closing the output of the last digit Q3 with a serial input X (dashed line) is realized annular shift register. The information in the ring register is entered by a parallel code from the inputs and then circulates in a closed ring under the action of clock C pulses.

In the reverse shift, register to enable the shift of information in both directions outputs of the triggers should be linked through logic elements of direction shift to inputs from the previous and following digits. Let the direction of shift given with logic level signal E, so that the made the shift to the right: , while  made  a shift to the left: . Then the D-trigger Qi of reverse shift register must be controlled by signal:

.

Fig.11.7

Such circuit on the logical element AND-OR-NOT is showed on Fig.11.7.

Figure 11.8 shows a variant of the reversing register, built based on JK-triggers. The signal at the inputs Ji is determined similarly Di, and at the inputs  always, due to inverters in the input circuits.

Fig.11.8

- 12. Counters (CT)

Counters are called sequential digital devices designed to calculate and store the number of pulses delivered at a specific time interval to its counting input. In addition to counting input counters may also have inputs asynchronous and synchronous setting of initial states. Addition, subtraction, and reversible counters are distinguished by the nature of state changes. According to the method of organizing transfers between the bits, they can be divided into counters with serial, though, parallel and combined transfer.

The main technical parameters of the counters are the system of calculation, the conversion factor and the performance.

12.1. Asynchronous binary counters

In asynchronous counters, there is no total synchronization for the stages and the transition of the digits to the new states takes place consecutively stage by stage, starting from the input, which receives the counting impulses.

The asynchronous binary counter can be made in the form of a chain of T-triggers, for each of which the counting impulse is formed by the trigger of the previous (lower) stage (Fig. 12.1).

Fig. 12.1

Installation time of n-bit counter in new state:

where  - time of installation of the trigger of the digits of the counter.

The main advantage of a serial counter is the low cost of circuits and the minimum of electrical connections, which simplifies the wiring of communication lines and increases the noise immunity of the circuit. The main drawback is the low performance, which is lower, the greater the coefficient Kсч = 2n, i.e. the greater the number of bits n.

One way to increase the speed of asynchronous meters is to organize transfers between stages through additional logic elements (Fig. 12.2). Control signal A determines the mode of operation of this counter:

A = 0 - storage mode,

A = 1 - account mode.

Fig. 12.2

If the first trigger of the DD1 counter (Fig. 12.2) is in the state "1", then the next counting impulse T drops it to the state "0" on the negative front. When A = 1 before switching the output Q1 = 1, the trigger DD1 the counting impulse T through the valve DD2 comes in the form of a transfer pulse P1 to the input of the second stage  and the valve DD4, and if Q2 = 1, then the counting impulse passes further through the valve DD4, etc. . The counting pulse T goes through the valves, to the second input of which comes Qi = 0. The triggers from the first to the (i-1)-th are set to "0" and the i-th trigger is set to "1". In the n-bit end-to-end counter, the installation time is determined by the delay of the counting impulse in the transfer circuits tзд.р.ср. and the installation time  of the last of the switching triggers:

tуст.max = (n-1) tузд.р.ср .И+ tуст.Т .

Then maximum account frequency:

fсч.max = [ t0 + (n-1)tзд.р.ср.И  + tуст.Т ]-1

will be higher than the frequency of the serial counter because the n-1 valve AND switches faster than the n-1 trigger.

In terms of structure, operation and other parameters, the variants of asynchronous counters are equivalent. Depending on the number of bits n, they realize the coefficient of Ксч.=2n and can be used as frequency dividers:

.

12.2. Reversible binary counters

The asynchronous counters considered are in the class of summaries. To implement the subtraction mode in Table 12.1 it is enough to replace "0" with "1" and vice versa, which is equivalent to removing information from the inverted outputs of the triggers (Table 12.1).

Table 12.1

| № | Q1 | Q2 | Q3 | P1 | P2 | P3 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| 2 | 0 | 1 | 0 | 1 | 0 | 1 |
| 3 | 1 | 1 | 0 | 0 | 0 | 1 |
| 4 | 0 | 0 | 1 | 1 | 1 | 0 |
| 5 | 1 | 0 | 1 | 0 | 1 | 0 |
| 6 | 0 | 1 | 1 | 1 | 0 | 0 |
| 7 | 1 | 1 | 1 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 |

If you change the designation of the outputs of the triggers (see Figs. 12.1, 12.2) to and vice versa, the subtraction counter is obtained by sequentially connecting the inverse outputs of the previous digits with the counting inputs of the next digits of the T-triggers. Shown in Fig. 12.3 T-triggers are switched by the negative edge of the input signal, that is, the signal from the inverse output of the previous trigger. As can be seen from Fig. 12.3-b, the subtraction counter is implemented in such inter-row connections. Thus, the only difference between summation and subtraction counters is the organization of the chains of transfer from the lower digits to the senior ones or the way of reading information.

Controlled account counters are called reversible. To build a reversible counter, it is necessary to include a logic circuit between the stages, which provides the connection of the counting input of the second and subsequent digits with the outputs  (addition) or  (subtraction) of the triggers of the previous digits.

Fig. 12.3

Suppose that the direction of the account is given by the signal A such that at A = 0 the account with addition is fulfilled, and at A = 1 - with subtraction. Then the transfer signal in the i-th bit Pi is determined by the logical expression:

.

According to identical expressions, different circuit variants of transfer circuits in reversible meters can be implemented (Fig. 12.4). Reversing counter with inter-row logic of account management according to the above expression is shown in Fig. 12.4. The inclusion of additional logic elements between stages increases the time of installation of the counter and reduces the maximum rate of change of logged states.

Fig. 12.4

12.3. Counters with arbitrary coefficient of account

Often, when designing digital devices, there is a need for frequency dividers for which Ксч is any integer. If the arsenal of circuitry of the designer has the simplest details, so required Ксч can receive due except for some states of the counter. In Fig. 12.5-а, б, в are shown the successive counters for Ксч = 3.5,7.

Fig. 12.5

The general principia of construction is shown in Fig. 12.5 counters with odd Ксч is shown in Fig 12.6.

Fig. 12.6.

If a frequency divider, included between the input and output triggers, is a divider of any positive integer n, then this full scheme provides a frequency division on Ксч.=2n+1.

For example, the counter is a frequency divider by 3 (see Fig. 12.5-a) implemented at n = 1, which corresponds to the direct connection between the input DD1 and the output DD2 triggers. For Ксч = 5 it is necessary n = 2, so between the input and output triggers it is necessary to include one additional trigger - frequency divider by 2 (Fig. 12.5-b). For the implementation of Ксч = 7 (Fig. 12.5-в) used structure (Fig. 12.6), respectively, when n = 3 obtained by the scheme of Fig. 12.5-a.

Shown in Fig. 12.5 sequential counters are intended mainly for use in frequency division mode without decoding their states, the change of which when using the structure (Fig. 12.6) does not correspond to the binary account.

12.4. Binary counter with controlled ratio Kсч

When constructing counter with decoding Kсч states, it is necessary to ensure the regularity of their change according to the binary account. For their synthesis it is necessary to determine the required number of triggers N according to the ratio:

.

In addition to N triggers, this counter should contain a logic circuit that decodes the Kсч  state and generates a reset signal of all digits to the "0" state.

If there is a need to control automatically the value of Kсч within, then a binary counter containing Nmax bits, which are determined from the condition

,

where Nmax  is the nearest greater integer, is constructed.

In Fig. 12.7 shows the scheme of the counter with controlled Kсч  = 1..15 for Nmax = 4.

After Kсч -1 pulses B1,…., B4 logic elements DD1,…., DD8 logic levels are set:

,

which are fed to the inputs of the match scheme DD9. The following counting impulse T = 1 through the DD9 valve sets the RS-trigger DD10 to a state “1” and the inverse output of the trigger causes the counter to be set to "0". Thus, due to Kсч pulses, the counter returns to its initial state. For example, in order to realize a module 10 account, outputs A1, A2, A3, A4 must be given to code “1001”.

Uneven loading of the digits of the counter may fail - some digits will not have time to reset. To eliminate the failure in the reset scheme add RS-trigger. Because of the trigger duration the counting pulse, the input T provides a reliable reset of the entire counter.

Fig. 12.7

The elimination of "superfluous" states can be performed using a digital comparator that would compare the status of the Q outputs of the counter with control code A (Fig. 12.8). Here, the reset signal R of the counter to the state "0" is determined by the expression:

.

Fig. 12.8

12.5. Synchronous binary CT

Synchronous, or parallel, are the counters, in which the switching of the digits occurs simultaneously, regardless of the number of the stage of the counter. This is achieved by supplying to all triggers the clock pulses that switch all the triggers in accordance with the logic of the counter. This synchronization achieves a minimum time of counter installation that does not exceed installation time of one trigger. Thus, the maximum frequency of changing the states of the counter   ensured:

.

The scheme of the synchronous addition counter is shown in fig. 12.9. Here the transfer potential Pi is formed sequentially as logical “1” is propagated. The accumulation of the delay due to the valves of the transfer circuit causes the mutual displacement of the counting pulses T and the transfer pulses Pi. As long as the coincidence of T and Pi pulses are not disturbed by such a shift, the counter operates without interruption with the highest possible counting frequency.

The transfer from the previous category is determined by the mode of operation of counter A, as well as the conjunction Qi:

,

where if A = 0 - storage mode, and if  A = 1 - counter mode.

Fig. 12.9

The potentials from the outputs of the triggers are supplied simultaneously to all valves of the transfer of the higher digits, and the switching of the states of the triggers occurs synchronously. As the number of digits grows, the number of inputs of the logic elements of the transfer chain increases, so with the increase in the number of stages N, the counter circuit becomes more complicated:

.

If the synchronous counter is built based on sections (for example, 4-bit), between them parallel circuits of parallel transfer are realized (Fig. 12.10).

Fig. 12.10

12.6. Reversible synchronous counters

Synchronous binary subtraction counter, as well as asynchronous, is realized by the connections of the inputs of the triggers with the inverted outputs of the signals of the lower digits. As a rule, such counters should contain the inputs of the previous (synchronized) setting of the number from which the counting starts. The scheme of such a counter based on JK-triggers is shown in Fig. 12.11.

Signal A controls the state of the counter:  if A = 0 - storage mode, and if A = 1 - counter mode.

Signal B controls the direction of the account: if A = 0 – addition mode, and if A = 1 – subtraction mode.

The transfer signal Ri is formed by coincidence “1” on the direct outputs of the previous triggers, and the loan signal Vi by coincidence “1” on the inverse outputs:

: ; .

The synchronous reverse counter can be built based on sections. Fig.12.12 shows the scheme of synchronous reverse counter with serial connection of transfer circuits and loans. In such counter of K sections the time of installation of the new state is:

,

Fig. 12.11

where  - delay propagation of the transfer signal () or loan()3 sections. The group transfer and the loan can also be implemented in parallel, similarly to fig. 12.10. The scheme is complicated, but the installation time of the new state is minimal:

.

Fig.12.12

12.7. Synthesis of synchronous CTs with an arbitrary conversion table

If you want to build a synchronous counter with alternating states other than binary calculations, you need to perform a synthesis procedure that involves a certain sequence of operations. In this case, as the main components can be used any trigger synchronized front, in combination with the control circuit.

At any transitions from the previous state of the counter to the next for each type of flip-flops to the information inputs, the signals given in Table 12.2 must be reported.

Table 12.2

| State | D | RS |  | JK |  | Mode |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | R | S | K | J |  |
|  | 0 | * | 0 | * | 0 | Storage status «0» |
|  | 1 | 0 | 1 | * | 1 | Transfer from «0» to «1» |
|  | 0 | 1 | 0 | 1 | * | Transfer from «1» to «0» |
|  | 1 | 0 | * | 0 | * | Storage status «1» |

* in the table indicates that both "0" and "1" can be submitted for this transition to this input.

The basis for the synthesis control circuits of the triggers inputs D, R, S, J, and K is a consecutive states table of the counter, which change along the front of the clock signal C. The current state of the counter, through the logical controls of the inputs of the triggers of the counter prepares the transition of all digits to a new state immediately following the current one. That is, the previous state uniquely determines the status of the control inputs of the triggers of the counter to move to the next state. Then the complete set of states of the counter defines the control functions of each of the inputs, and the arguments of these functions are the states of all outputs of the triggers in the previous step of operation.  Finding and implementing such control functions is the essence of the synthesis task.

Consider the procedure of synthesis of a synchronous counter with a next example. Suppose that we have to construct a 4-bit counter with the transfers according to Table 12.3.

Usually, synchronous counters are built on the same triggers. In this example, we will select different triggers to illustrate the method.  Let Q4 and Q1 be formed by D-triggers, Q2 by RS-trigger and Q3 by JK-trigger.

To find the control function of the D-trigger output Q1, we construct a Karnaugh map, whose arguments are all variables Q1, Q2, Q3, and Q4 from the table.

Table 12.3

| N | Q4 | Q3 | Q2 | Q1 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 1 | 1 |
| 3 | 0 | 0 | 1 | 0 |
| 4 | 0 | 1 | 1 | 0 |
| 5 | 0 | 1 | 1 | 1 |
| 6 | 0 | 1 | 0 | 1 |
| 7 | 0 | 1 | 0 | 0 |
| 8 | 1 | 1 | 0 | 0 |
| 9 | 1 | 1 | 0 | 1 |
| 10 | 1 | 1 | 1 | 1 |
| 11 | 1 | 1 | 1 | 0 |
| 0 | 0 | 0 | 0 | 0 |

The initial state of Q4 = Q3 = Q2 = Q1 = 0 must condition the transfer to the state of Q4 = Q3 = Q2 = 0 and Q1 = 1, as shown in Table 12.2, for this the combination Q4 = Q3 = Q2 = Q1 = 0 must provide at the input trigger Q1 state D1 = 1, enter in the cell of the Karnaugh map "1". Similarly, fill in all the cells of the Karnaugh map corresponding to the states of Table 12.3

|  | Q2Q1 Q4Q3 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| D1 = | 00 | 1 | 1 | 0 | 0 |
|  | 01 | 0 | 0 | 1 | 1 |
|  | 11 | 1 | 1 | 0 | 0 |
|  | 10 | * | * | * | * |

In the Karnaugh map, * states not indicated in Table 12.3 are indicated. That is, any cells that minimize D1 can be entered in these cells. From the Karnaugh map, we find MDNF and MCNF:

MDNF: ,

MCNF: .

For the RS-trigger that generates the signal Q2, we find the control functions of the inputs R2, S2 taking into account the event table 12.2:

|  | Q2Q1 Q4Q3 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| R2 = | 00 | - | 0 | 0 | 0 |
|  | 01 | - | - | 1 | 0 |
|  | 11 | - | 0 | 0 | 1 |
|  | 10 | * | * | * | * |

|  | Q2Q1 Q4Q3 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| S2 = | 00 | 0 | 1 | - | - |
|  | 01 | 0 | 0 | 0 | - |
|  | 11 | 0 | 1 | - | 0 |
|  | 10 | * | * | * | * |

From the Karnaugh maps, we find:

MDNF: ; ,

MCNF: ; .

For the JK-trigger that generates the Q3 signal, we find the control functions of the inputs J3, K3:

|  | Q2Q1 Q4Q3 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| K3 = | 00 | - | - | - | - |
|  | 01 | 0 | 0 | 0 | 0 |
|  | 11 | 0 | 0 | 0 | 1 |
|  | 10 | * | * | * | * |

|  | Q2Q1 Q4Q3 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| J3 = | 00 | 0 | 0 | 0 | 1 |
|  | 01 | - | - | - | - |
|  | 11 | - | - | - | - |
|  | 10 | * | * | * | * |

From the Karnaugh maps, we find:

MDNF: ; ,

MCNF: ; .

For the D-trigger that generates the Q4 signal, we find the control function of the input D4:

|  | Q2Q1 Q4Q3 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| D4 = | 00 | 0 | 0 | 0 | 0 |
|  | 01 | 1 | 0 | 0 | 0 |
|  | 11 | 1 | 1 | 1 | 0 |
|  | 10 | * | * | * | * |

From the Karnaugh maps we find:

MDNF:  ,

MCNF: .

We choose the basis of logical elements for this technology. If it is appropriate to use AND-NOT elements, the resulting expressions must be converted:

;

;

;

; ; .

According to the obtained expressions for the selected types of triggers we build a scheme of synchronous counter (Fig. 12.13):

Fig. 12.13

Similarly, using the same MCNF transformations, it is possible to synthesize control circuits in other bases (NOR, AND-OR-NOT).

Thus, the synthesis of a synchronous counter with an arbitrary conversion table includes the following steps:

1. Drawing up a conversion table.

2. Choosing the type of triggers.

3. Making a Karnaugh  map for each trigger information entry.

4. Finding MDNF and MCFF.

5. Convert MDNF, MCFF to the form corresponding to the selected type of logic elements.

6. Drawing up a schematic diagram of the meter.

12.8. Synchronous counter with multiplex control

Trigger control circuits can be implemented on multiplexers. The simplest counters with an arbitrary conversion table are implemented on D-triggers with one control input. The outputs of the triggers are connected to the address inputs Ai, and the outputs of the multiplexers control the information inputs of the D-triggers. The information inputs of the multiplexers provide code combinations that determine the next state of the counter.

Consider the state of the synchronous meter states defined in Table 12.4.

Table. 12.4

| N | А | Q1 | Q2 | Q3 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 2 | 0 | 1 | 0 |
| 2 | 3 | 0 | 1 | 1 |
| 3 | 5 | 1 | 0 | 1 |
| 4 | 7 | 1 | 1 | 1 |
| 5 | 1 | 0 | 0 | 1 |
| 6 | 4 | 1 | 0 | 0 |
| 7 | 6 | 1 | 1 | 0 |
| 8 | 0 | 0 | 0 | 0 |

From the state with the number "0" when Q1 = 0, Q2 = 0, Q3 = 0, the counter must go to the state "1" when Q1 = 0, Q2 = 1, Q3 = 0. To do this, the input of the multiplexers with the address A0 = Q1 = 0, A1 = Q2 = 0, A2 = Q3 = 0, it is necessary to submit a combination of signals "010", which the next clock pulse will be entered in the triggers of the counter. Changing the status of the counter changes accordingly the number of information inputs of the multiplexers, from which the next combination of signals will be entered into the counter. The counter implementing the transitions according to Table 12.4 is shown in Fig. 12.14.

Fig.12.14

- 13. Impulse devices

Pulse Devices - Pulse devices are devices designed to generate, generate, amplify, transmit, and transform pulses. These include pulse generators, triggers, multivibrators, counters.

13.1. Impulse front detectors

These are functional elements, designed to generate output pulse with specified parameters at the time of switching the input signal.

1                                                         1

0              Positive front;                          0    — Negative front

As parameters of the output signal is amplitude U2 and duration t.

13.1.1. Detector of positive front (DPF)

The detector of positive front of signal U1 shown in Fig.13.1. For each positive switching signal U1 at the input formed output positive pulse with amplitude

U2 = U1-U0,

Fig. 13.1

and duration

t1 = τ10 ln [(U0-U1) | (U0-Uth.)] = C (R0out + R) ln [(U0-U1) | (U0-Uth.)]

where U0 - logic zero level; U1 - logic units level, Uth - threshold voltage shift logic element NOT, R0out - output impedance inverter in the state "0".

13.1.2 Detector of negative front (DNF)

The detector of negative fronts of input signal U1 shown in Fig.13.2. For each negative switching input signal U1 at the output formed positive pulse with amplitude

U2 = U1-U0

Fig.13.2

and duration:

t2 = τ01ln [(U1-U0) | (U1-Uth.)] = C (R1out + R) ln [(U1-U0) | (U1-Uth.)]

where U0 - logic zero level; U1 - logic units level, Uth - threshold voltage shift logic element NOT, R1out - output impedance inverter in the state "1".

13.1.3. Front detector (DF)

The signal edge detector U1 is shown in Fig. 13.3. At each switch of signal U1 at the output of the circuit, positive pulses are formed with the amplitude

U2= U1-U0

Fig.13.3

and the duration alternately t1 and t2.

13.2. Pulse Expander

In information systems, it is often necessary to obtain wider pulses of a certain duration from short pulses. This task is easily accomplished with the help of a pulse expander.  The pulse expander (Fig. 13.4) generates a single output pulse of duration tвих  for each input pulse of duration tвx of an external control signal:

tвих= tвх + ∆t.

This implies that the impulse that is generated exceeds the duration of the input impulse.

Fig.13.4

For the correct operation of the pulse expander, it is necessary that the duration of the input pulse tвх be sufficient to allow the capacitor to discharge completely. After the input tвх pulse is completed, the capacitor is charged through resistor R to the supply voltage Uип. In this case, as soon as the voltage reaches Uпор , RS-trigger will switch in state “1”. In this case, the additional duration of the output pulse ∆t depends on the nominal values of the installed capacitance C and resistor R. The simplified formula allows you to calculate roughly the additional pulse duration:

∆t  = RC ln [Uип/( Uип - Uпор )],

where Uип  is the supply voltage of the circuit; Uпор  is the level of switching threshold of the RS-trigger.

13.3. Single vibrators

When working with digital devices, it is often necessary to generate impulses of a certain amplitude and duration. Special devices – single vibrators (SV), perform this function. These are trigger devices, which in response to each input pulse generate an output pulse with the desired amplitude U2 and duration tи. Such impulse generators can be implemented on logic elements and time-determining components R and C.

13.3.1. SV on logical elements NAND

The scheme of SV on logical elements NAND is shown in Fig. 13.5.

Fig. 13.5

In the scheme of the simplest single-vibrator there are two logic elements, one of them is used for its intended purpose - as a logic element NAND and the other as an inverter. For normal operation, the pulse duration of the triggering pulse must be less than the duration of the output pulse that is generated.

The pulse duration, generated by a single-vibrator, can be calculated based on the discharge condition of capacitor C. Until capacitor, C is discharged to the limit voltage Uпор, his voltage U2 on input of the element NAND response to the level of logical “1” and its output is maintained at level “0”. Over time, the voltage on the capacitor C becomes equal to Uпор and at the output of the element NAND, the level of logical unit will appear. Assuming that the voltage before the start of the discharge on the capacitor was equal to the voltage level of the logic unit U1, then the change in voltage on capacitor  C over time can be represented as an exponent with constant τ≈RC, so the duration of the output pulse tи equals the time of discharge of the capacitor to the limit value Uпор is determined by the ratio:

13.3.2. SV on logical elements NOR

In Fig. 13.6 shows the scheme of a single-vibrator on logic elements NOT (1) and NOR (2).  Alternatively, inverter 1 may be a logic element NOR with integrated inputs.

The single-vibrator, according to the scheme shown in Fig. 13.6, works as follows: in the initial state of the device at the output U21 of the inverter (1) there is a low voltage level supported by the input voltage from the power supply Uип.

Fig.13.6

At a low level at the input U1 of a single vibrator at the output of the logic element NOR (2) there is a high level, which makes the capacitor C practically discharged. A short positive input pulse U1 switches the logic element NOR (2) and through the capacitor C switches the output level of inverter 1 to the "0" state.  At the stage of forming the duration of the output pulse of a single vibrator, the capacitor C is charged through the resistor R and the output impedance of the logic element NOR (2) R0 вых  with a constant exponential function τ3 = C (R + R0 вых  ). The process of forming the output pulse is completed when the input voltage of the inverter 1 reaches the threshold level Uпор. As a result, rectangular positive impulse is formed at the output of the single-vibrator U21 and negative impulse of is formed at the output of U22. A duration tи  on both outputs is equal:

tи = τз ln [(Uип /(Uип - Uпор)] .

At the stage of restoration of the single-vibrator to the next starting pulse, the capacitor C is discharged through the output resistance R1 of the logic element NOR (2) in the state of logic "1" and through the resistance rпр of the diode VD with constant exponent τв= C (R1выx + rpr). The recovery time of a single-vibrator tв is equal to:

tв ≈ 3 τз  =  3 C(R1вих+rпр).

13.3.3. SV based on RS-trigger

Figure 13.7 shows the scheme of single-vibrator based on RS-trigger. The feedback circuit of resistor R and capacitor C between the output of trigger Q and the reset input of trigger R determines the timing of the output signal.

Consider the charging and discharging circuit of the capacitor C in a single vibrator.  At the stage of forming the time interval tи, the capacitor С is charged from "0" (more precisely, from residual voltage U0) to the threshold voltage Uпор. Circuit of charge: Rвых-R-C- “ground” with constant of exponent τ3 = C (Rвых + R).

Fig.13.7

The duration of the charging process determines the duration of the output pulse tи:

tи = τз ln [(U1-U0)/(U1- Uпор)].

In the recovery phase, the capacitor is discharged from the stop to 0 through the diode VD and the output impedance of the trigger Rвых with constant exponential

τроз = С(R+R0вых).

The duration of SV recovery tв  can be estimated as 3τроз, that is:

tв ≈ 3 τроз = 3С(R+R0вых).

The diode is almost completely closed when the voltage on diode drops below 0.5 ... 0.6 V, and the capacitor completes the discharge with the same constant time as when forming a time interval. Thus, when the requirements for the residual voltage on the capacitor are increased, the recovery time increases.

The described single-vibrator on the RS-trigger has a simple structure and generates two anti-phase rectangular pulses. However, it has some disadvantages. First, the charge of the capacitor C occurs through the output impedance of the trigger, which causes some violation of the rectangular signal U21 at the direct output of the trigger. The change in Rвых affects the duration of the generated impulse. Secondly, it is large duration time of the output pulse, the time to restore the voltage on the capacitor to its initial level.

13.3.4. SV on operational amplifier

The basis of such a single-vibrator is the Schmitt trigger (TS) on the operational amplifier (OP) (Fig.13.8). Such a trigger in this case is invertible and has two threshold levels of state switching Uп1 and Uп2, which are determined by the parameters of the positive feedback components (R1, R2) according to the ratios:

Uп1= U+0 R1/R2 ,        Uп2= -|U-0| R1/R2,

where  U + 0, -U-0  are the limit values at the TS output in two possible states, which depend on the supply voltages + Ucc1, -Ucc2. In practice, it is possible to take U0 ≈0.9  Ucc in the calculations.

Fig.13.8

The scheme of the single-vibrator is shown in Fig. 13.9. In addition to the TS, the circuit contains components that determine the duration of the output pulse of the SV. In this case, such components are resistor R and capacitor C in the negative feedback circuit. In addition, the SV contains a circuit for its excitation, which includes the differentiation circuit C1-R3 and the diode VD2.

Prior to the arrival of the excitation pulse U1 SV is in a stable state when the output voltage of the circuit is stable voltage U2 = -U-0. The diode VD1 is open and the capacitor C is discharged practically, since the voltage on it is equal to the voltage U0д   on the open diode VD1. The positive input pulse U 1 through the diode VD2 switches the TS to a state when the output is set at U2 = U+0. At a high level of U + 0 at the output, the diode VD1 closes and begins to charge the capacitor C through resistor R. The voltage on the capacitor C increases by exponential law until it reaches the threshold level Up1.  Upon reaching the threshold Uп1 TS switches to the state U2 = -U-0. This ends the formation of the output pulse of duration tи:

tи = RC ln [(U+0- U0д)/( U+0- Uп1)].

At the recovery stage of SV  the capacitor C is discharged through the resistor R to the diode unloading voltage VD1, i.e. to U0д. The duration of the recovery stage tв is determined by the ratio:

tв = RC ln [(|-U-0| - Uп2)/( (|-U-0| - U0д)].

Fig.13.9

The amplitude of the positive output pulse Um is equal to:

Um = U+o + U-o.

If it is necessary to obtain an SV that generates a negative output pulse in the scheme shown in Fig. 13.9, it is sufficient to change the polarity of the inclusion of diodes VD1 and VD2.

13.4. Multivibrators (MV)

The multivibrator is a relaxation generator designed to generate periodic signals (most often rectangular) with predetermined parameters: amplitude Um, frequency f, intermittency Q. MV can operate in self-oscillation mode or in the standby mode of an external start signal.

In the self-oscillation mode, the MV does not have a stable equilibrium. When operating a multivibrator in this mode, there are two alternating quasi-stable states. The state of quasi-stable  characterized by a relatively slow change in currents and voltages, leading to some critical state, which creates the conditions for the jumping transition of the multivibrator from one state to another. The oscillation period depends on the scheme parameters.  MV circuits can be built on the basis of bipolar transistors, unipolar transistors, operational amplifiers, logic integral elements like NOR, NAND, RS-triggers, Schmitt triggers and also on the basis of negatrons (tunnel diodes, thyristors, transistors, dynistors ). Recently, MVs most often are built based on integrated circuits.

13.4.1. MV on inverters (NOT)

The scheme of MV based on logic elements NOT is shown in Fig.13.10. In addition to the inverters 1.2, the circuit contains a capacitor C, which provides positive feedback and, therefore, a regenerative oscillation mode. Together with the negative feedback resistor R, the capacitor C determines the frequency f and the duty cycle Θ of the MV output signals.

Fig. 13.10

When the power is turned on, any fluctuations or noises under the influence of positive feedback MV transfer into one of   two quasi-stable states. In one of the states at the output U1 is set low, and at the output U2 - a high level of potential, and in the second state, on the contrary, at the output U1 high and the output U2 - a low level of potential. Relaxation processes of recharging the capacitor C through resistor R, determine the duration of the half-period T1, T2, the frequency of generation f and the duty cycle Θ of the output pulses.

At the time interval T1 at the input of the first element NOT is the voltage U3> Uпор, so its output is maintained low U01, and the output of the second element is not high U12. The charge current of capacitor C flows from the power source in the circuit  and decreases exponentially with constant time:

τ1 = С(R + R0вих1 + R1вих2) ≈CR,

where R0выx1  is the output impedance of inverter 1 in state "0",   R1выx2  is the output impedance of inverter 2 in state "1".

In this case, the voltage at the input of the first element NOT fall exponentially from the initial value Uпор+ U12. When the voltage at the input of the first element does not reach the threshold level Uпор, the inverter of the first element goes into amplification mode, its output voltage, increasing, switches the second inverter. Further, under the influence of positive feedback, the scheme regenerative switches to the second quasi-stable state. This completes the formation of the interval T1 of the output signal:

T1 = τ1 ln [(U21+Uпор )/Uпор ].

At the time interval T2, the voltage at the input of the first element NOT is U3 < Uпор, so the output of the first element NOT is high U11, and the output of the second element NOT is low U20. The charge current of the capacitor C flows in the opposite direction from the power source of the first element NOT in the circuit  and creates a voltage drop on the resistor R sufficient to maintain at the input U3 a voltage element in the logic region "0". As the capacitor C is recharged, the current through the resistor R decreases exponentially with constant time:

τ2 = С(R + R1вих1 + R0вих2) ≈CR,

where R1вых1 is the output impedance of inverter 1 in state "1", R0вых2  is the output impedance of inverter 2 in state "0".

In this case, the voltage at the input of inverter 1 increases exponentially from the level Uпор - U12, asymptotically approaches the level U12. At the moment of coincidence U3 = Uпор, the circuit switches again and all processes are repeated.

The duration of the T2 half-life, given the above formulas, is determined by the ratio:

T2 = τ2 ln [(2U21-Uпор )/( U21-Uпор) ].

The frequency of generation f = (T1 + T2) -1   and duty cycle of output signals (ratio of output pulse duration and oscillation period):

Θ1 = (T1+T2)/T1 = T/T1 ,   Θ2 = (T1+T2)/T2 = T/T2.

The advantages of this multivibrator are the simplicity of the scheme and the stability of the frequency of generation, and a slight disadvantage is the distortion of the tops of the pulses.

13.4.2. MV on NAND with auto start

Scheme MV with auto start is shown in Fig.13.11.

Fig.13.11

This multivibrator contains two circuits R1-C1 and R2-C2, which set the time parameters of the MV, that is, the frequency f and the duty cycle Θ of the output signals. The MV is built on the logic element NAND DD1, which is used as an inverter, and the logic elements NAND DD2. The DD3 and DD4 logic elements are designed to automatically trigger the generation mode.

In the stationary oscillation mode, anti-phase signals are set at the inputs of the DD3 element, so a constant high level of U31 is maintained at the output of the U3 of the DD3 element, and a low U40  0 at the output of the DD4 element, and the resistor R2 is almost "grounded".  In the case of failure of the oscillation signal U1 = 0, the outputs of the elements DD1 and DD2 are set equal   = , therefore output DD3 we have U3 = 0, and the output DD4 U4 = U41, which through the resistor enters the input DD2 and provides the start of the multivibrator when the trigger signal U11. Thus, the generation mode is started. This allows the scheme to generate pulse packets of U11 control signal duration.

External diodes provide fast discharge of capacitors C1, C2 that set the time parameters of the MV: duration for the half-periods T1 and T2, the frequency f = (T1 + T2) -1 and the  Θ1 = (T1 + T2) / T1.

The resistance values of the resistors R1, R2 must be selected large, but subject to the limitation:

U11min / I0вх   ,

where U11min  - the minimum voltage level for the high level;

I0вх - input current of logic elements at low level at inputs DD1, DD2.

The duration of the T1, T2 half-cycles of the multivibrator is determined by the time-setting circuits respectively ,, and ,:

T1= C1R1ln (U1/ U11min ) , T2= C2R2ln (U1/ U11min ),

and the frequency of oscillations generated by    f = .

The capacitance of capacitors C1, C2 defining the duration of the intervals T1, T2 is determined from the given ratios, respectively, for the selected resistance values of resistors R1, R2.

13.4.3. MV on the Schmitt trigger (TS)

Scheme MV on the Schmitt trigger is shown in Fig.13.12.

Fig. 13.12

If the multivibrator is to provide only a given frequency of generation f and the duty cycle is insignificant, it is advisable to use the scheme of the simplest multivibrator based on the Schmitt trigger.

The transfer characteristic of the Schmitt trigger has a pronounced hysteresis character. It distinguishes two threshold voltages Uп1 and Uп2 of switching the output voltage  and   respectively.

The simplest multivibrator is based on the inverting Schmitt trigger is the inclusion of an integrating RC-link between output and input.

At the moment of connection of the power supply Uип capacitor C is discharged, U1 = 0, a high level is set at the inverting output of the trigger, which determines the charge of the capacitor through the resistor R with a constant time

τ1 = С(R+R1вих),

where R1вых - the output resistance of the Schmitt trigger in the state "1" at the output.

In the new TS state, the input voltage drops exponentially as it approaches . When  compared to Uп2, the trigger switches to a new high output state, and a new charge cycle begins.  Thus, the multivibrator excites itself, generates rectangular pulses, duration T1, and pauses T2, which are determined by the ratios:

,

.

These expressions allow us to calculate the frequency  of multivibrator generation and the duty cycle of output pulse:

Θ1 = (T1+T2)/T1 = T/T1 ,   Θ2 = (T1+T2)/T2 = T/T2.

The frequency stability of the multivibrator is low, since the difference in the transfer of voltages Uп1 and Uп2 is insignificant and the temperature drift of each of them significantly affects the duration of the time intervals T1, T2.

13.4.4 MV based on an operational amplifier

The scheme of MV based on the operational amplifier (OA) is shown in Fig.13.13.

Operational amplifiers are widely used in the construction of multivibrators because they have the following positive properties:

• high voltage gain () that guarantees self-excitation conditions;

• large drop in output voltage, the levels of which are close to the voltage of the power sources;

• large input and low output resistance;

• high rate of change of the output voltage S = V / s.

Fig.13.13

To provide a regenerative mode of switching, the OA is covered by positive feedback (Fig. 13.14), resulting in a transfer characteristic that acquires a hysteresis character similar to that of the Schmitt trigger.

Fig. 13.14

The scheme in Fig.13.14 is a Schmitt trigger on the OA. The threshold voltages Uп1, Uп2 of such a trigger are determined by the part of the output voltage supplied by the positive feedback circuit (R1, R2) to the non-inverting input.

Since the output voltage of the OA in the mode of the Schmitt trigger can accept only two static levels corresponding to the levels of positive  or negative  constraints , the threshold voltages are determined by the ratios:

Uп1 = Uo1 R1/(R1 + R2) =  Uo1 ß ; Uп2 = -Uo2 R1/(R1 + R2) =  -Uo2 ß,

where  ß = R1/(R1 + R2).

The multi-vibrator based on the Schmitt trigger on the OA can be obtained by introducing a connection between the output of the OA and its inverting input through an integrating RC-circuit. The principle of operation of such a multivibrator is to monitor the voltage delay at the inverting input U11 by the voltage at the non-inverting input U12, which repeats virtually inertia with a coefficient of proportionality ß <1 output voltage U2. At the interval T1 at the output of the OA, a high level was established, at the non-inverting input there is also a practically constant potential Uп1 = Uo1 ß, and at the inverting voltage the voltage exponentially approaches U + 11 → Uo1  as the charge of the capacitor C is constant:

τ1 = C(R+ R+вих.) ≈ CR,

where R+вых - the output resistance of the OA in the positive limit mode.

The charge of the capacitor C causes a decrease in the differential input voltage Ud = U12-U11 and when it reaches Ud = U12 - U11 ≈0 OA goes into active mode. Under the influence of positive feedback, the Schmitt trigger switches to a low-output state.  During the switching, the voltage on the capacitor C does not have time to change significantly, so the negative feedback (RC) on the switching processes has no effect. At the non-inverting input of OA the voltage Up2 = -Uo2 ß is set.

At interval T2, the voltage across capacitor C approaches with a constant time:

τ1 = C(R+ R-вих.) ≈ CR,

where R-вых - the output resistance of the OA  in the mode of negative limitation.

The interval T2 ends at the moment of coincidence Ud = U12 - U11 ≈ 0 and subsequent regenerative switching of the OA to the state of positive restriction. The duration of the intervals T1, T2 is determined by the exponential capacitor recharge functions in the voltage range between Uп1 and Uп2:

T1 = τ1 ln [(Uo1 - Uп2)/( Uo1 - Uп1)],

T2 = τ2 ln [(-Uo2 - Uп1)/( -Uo2 - Uп2)].

Whereas Uп1 = Uo1 ß; Uп2 = = -Uo2 ß, the above relations can be represented in the form:

T1 = T2 = RC ln [(1 + ß)/(1 – ß)] = RC ln [(2R1 + R2)/R2],

i.e. the multivibrator generates rectangular pulses with an opacity of  θ = 2. An independent setting of the duration T1, T2 and the required velocity θ is realized when a nonlinear bipolar circuit replaces the resistor R.

13.4.5. MV on RS – trigger

The scheme of MV based on the RS-trigger is shown in Fig.13.15.

Fig.13.15.

To implement a multivibrator based on the RS-trigger, it is necessary to include two circuits defining the time parameters of the MV between the outputs and their respective inputs. Therefore, we get a self-oscillating multivibrator with independent control of half-time durations and time constants:

τ1 = С1(R1 + R1вих), τ2 = С2(R2+ R1вих),

where R1вых is the output impedance of the RS-trigger in the state "1" at the output.

The duration of half-periods is determined by the ratios:

T1 = C1(R1 + R1вих) ln [(U121 – U021)/( U121 – Uпор)],

T2 = C2(R2 + R1вих) ln [(U122 – U022)/( U122 – Uпор)],

where Uпор is the threshold switching voltage of the trigger.

Discharge of capacitors C1, C2 flows rapidly through diodes VD1, VD2 with constant time:

τр1 = С1(rпр + R0вих) ≪ T1  ,    τр2 = С2(rпр + R0)≪ T2

and does not affect the frequency of generation:

f = (T1 + T2)-1.

This multivibrator can be implemented on both TTL and CMDC IC. The relative disadvantage of such a MV is the temperature instability of the frequency (duration) of the pulses that are generated.

13.4.6. MV on two operational amplifiers

To increase frequency stability, it is possible to replace the relaxation processes occurring in exponential time linear circuits in linear circuits. Linear relaxation can be realized with a constant-level integrator. The scheme of such a multivibrator is a series connection in the closed circuit of the inverting integrator on the OA DA2 and the non-inverting Schmidt trigger on the OA DA1 (Fig.13.16).

Fig.13.16

The transfer characteristic of a non-inverting Schmitt trigger is characterized by two threshold levels. The switching thresholds of the trigger Uп1, Uп2 are determined by the resistance resistors R1 and R2 of the positive feedback at two possible voltage levels at the output of the Schmitt trigger:

Uп1 = -U+o R1/R2 ,  Uп2 = U-o R1/R2,

where U +o, -U-o are the output voltage levels of the Schmitt trigger in states of positive and negative constraints, respectively, determined by the supply voltages of the operational amplifier.

The output voltages of the Schmitt trigger U +o, -U-o  are the input voltages of the integrator whose voltage at the output varies according to a linear law between the threshold levels Uп1, Uп2.

At the time interval T1 the output voltage of the integrator decreases linearly from the initial value Uп2:

U21(t) = Uп2 - U+o t / (RC).

At the end of the interval T1, the output of the integrator sets the voltage Uп1 switching the output of the Schmitt trigger to the state -U-o.  So

U21(T1) = Uп2 - U+o T1/ (RC) = Uп1.

From this relation we obtain an expression for the length of time interval T1:

T1 = RC (Uп2 +Uп1) / U+o = RC k (U+o + U-o)/ U+o ,

where k = R1/R2.

Similarly, we obtain the expression for the time interval T2:

T2= RC (Uп2 +Uп1) / U-o = RC k (U+o + U-o)/ U-o .

If the operating amplifier has symmetrical power, i.e.  U + o = | -U-o | , duration T1 and T2 can be calculated by the expression:

T1 = T2 = 2kRC.

The output U21 of the multivibrator produces a series of rectangular pulses of the type "meander", and at the output U22 - pulses of triangular shape with frequency

f = (4kRC)-1

and with the duty cycle θ = 2.

If it is necessary to obtain output pulses with arbitrary magnitude, the resistor R of the integrator must be replaced by a nonlinear bipolar.

The frequency stability of such a multivibrator is determined by the stability of the parameters of the components R, R1, R2, and C.

13.5. Generators of linear output voltage

Generators of linear output voltage (LOV) are electronic devices whose output voltage changes linearly for a specified time.

A number of parameters characterizes the linearly variable voltage U (t):

- the duration of the running stroke tp, that is, the time during which a linearly output voltage is formed;

- the duration of the reverse tв (recovery time) is the time, during which the reverse transitions to the beginning of the linear portion of the output signal;

- the repetition period of the output signal T = tр + tв;

- amplitude f saw tooth pulses Um;

- nonlinearity coefficient  ε.

One of the most important parameters of the LOV is its coefficient of nonlinearity ε. To determine nonlinearity coefficient ε, we use the well-known assertion that a linear function is characterized by a constant derivative at all its points, so the deviation from the linear law can be estimated by the coefficient of nonlinearity. Nonlinearity is determined by the maximum deviation of the real waveform from the ideal linear shape. The coefficient of nonlinearity is found as the ratio of the difference of the maximum and minimum derivatives of the function on the working area of the signal to the maximum derivative:

ε % = [dU(t)dt]max - [dU(t)dt]min[dU(t)dt]max 100 .

13.5.1. LOV generator on the dinistor (Shockley diode)

The scheme of the MV based on the dinistor (Shockley diode) and the operational amplifier is shown in Fig.13.16.

In the scheme of Fig.13.17, the resistance of the resistor R and the voltage of the power supply Ucc  are chosen such that the load line intersects the voltage-current characteristic (VCC) of the dinistor with the parameters of switching up Umax and switching off Umin on the negative resistance section (to ensure the mode of generation).

Fig.13.17

When the power is turned on through the resistor R, the charging of the capacitor C with a constant exponent  starts when the capacitor is switched off. When Uc  reaching Umax, the breakdown of the dinistor  begins to switch on, its resistance is regenerative reduced to rпр. When dinistor is opened, the capacitor C is discharging through it, and at the moment when the voltage on it Uc reaches Umin, the dinistor shuts off and a new charge cycle of the capacitor C through the resistor R begins. The duration of the running stroke tp or the exponential growth law is determined by the relation:

tр = τзар ln [(Ucc – Umin)/(Ucc – Umax)] = RC ln [(Ucc – Umin)/(Ucc – Umax)].

The duration of recovery, that is, the return tv also for the exponential law of reducing the voltage on the capacitor C with constant τроз  = Сrпр is determined by the relation:

tв = τрозln (Umax/Umin) =  Сrпр ln (Umax/Umin)≪ tр.

Thus, the frequency of generation of saw tooth output signals is:

f = (tр + tв )-1.

The amplitude Um of the output signals depends on the parameters of the components of the negative feedback and is determined by the ratio:

Um = (Umax - Umin)(1+R2/R1).

To ensure linearity of the signal work area, it is necessary to use the exponential initial section, that is, to select a dinistor so that the requirement is satisfied: Umax ≪ Ucc. This condition is easily fulfilled if the VD dinistor is replaced by the scheme shown in Fig.13.18 by including it between points a and б. The maximum voltage on the capacitor C (instead of the parameter Umax) is determined by the control voltage Ucm.

Fig.13,18				Fig.13.19

The nonlinearity of the signal work area can be significantly reduced if, if  use instead of the resistor R, the generator constant current shown in Fig. 13.19, which is connected between points d and a. In this case, the capacitor C is charged by the generator constant current of the collector Ik  of the transistor VT1, which is determined by the ratio:

Ik =(Ust –U0)/R3 = const,

where Ust -  the stabilization voltage of the Zenner diode VD,

U0 - voltage drop at the emitter junction of transistor VT1.

In this case, the voltage across the capacitor C increases according to a linear law:

and  the duration of the linear portion of the signal, when the signal increases from Umin to Umax, is:

tp = (Umax - Umin)C/Ik.

13.5.2. Shaper of LOV on an operational amplifier

The scheme of the shaper LOV based on the operational amplifier is shown in Fig.13.20.

Fig.13.20

The shaper is based on an integrator on an operational amplifier covered by negative feedback due to the RC circuit. Transistor VT is used to set zero initial conditions. If the voltage at its gate U11>Uпор, the channel is induced and has a low resistance Ri, shorting the capacitor C. This determines the initial conditions of integration:

U20 = - U12 Ri /R.

If the ratio Ri ≪ R is chosen correctly, we have U20≈0, i.e. almost zero initial integration conditions when the capacitor C is practically discharged.

If a rectangular pulse of amplitude U11 < Uпор is input to U11, the VT transistor closes and capacitor C begins to charge by the currant:

IC = IR = U12/ R.

The output of the integrator generates a voltage:

U2(t) = U20 -   1C  U12 dt.

If U12 = const, then IC = IR = const and the output signal over the duration   tp   of the input pulse U11 is formed by a linear law:

U2(t) = U20 -   1C  U12R t.

At the end of the input pulse tp output voltage reaches the amplitude value:

U2m = - U12 tр/(RC).

After the input pulse, the transistor opens and through it the capacitor C is discharged. Duration of recovery time

tв ≈ 3 C Ri .

The considered scheme provides a high quality output signal with a low coefficient of nonlinearity and high load capacity.

- 14. Digital to analog and analog to digital converters

Digital to analog converters (DAC) are a class of devices that realize the conversion of code combinations (dimensionless quantities) into some physical value (most often, voltage) according to some conversion factor. DAC have a very wide range of applications in measurement, computing, biomedical engineering, transmission, storage and display information, in process control systems, etc. In the Ukrainian literature, DAC stands as ЦАП.

Schemes of use of digital-to-analog converters apply not only to the field of code-to-analog conversion. Using their properties, you can determine the composition of two or more signals; build function divisors, analog units controlled by microcontrollers, such as attenuators, integrators and more. An important area of DAC is also signal generators, including arbitrary waveforms.

The main function of the DAC is:

U(X) = K× Xn,

where X is some digital code that, in the case of n-bit binary encoding, looks like:

Xn =  xi*2i ,

where xi is the value of the i-th  bit of the binary code: xi {0,1}. The coefficient K determines the scale of conversion and has usually a dimension of voltage.

The conversion error reflects the difference between the nominal conversion function and the real one. This parameter can be specified both before the output and before its input. If the conversion error is output, then it is expressed in units of output (voltage or current), if input, then in dimensionless portions of the unit of the converted code.

14.1. DAC based on analog adder

The scheme of DAC based on the analog adder is presented in Fig. 14.1. The principle of such a DAC is based on the addition of each bit current Ii, proportional to the weight of the bits number and which are controlled by the switchable logic variables xi of the converted code. The value of the bits current Ii is determined by the value of the reference voltage Uоп and the resistance of the resistor of the i-th bit  Ri:

Ii = Uоп/ Ri .

Fig. 14.1

The highest bit xn-1 of the converted code corresponds to the maximum bit current In-1, which is determined by the smallest resistance Rn-1.  The current of the second bit In-2 is twice less, that is, the resistance Rn-2 accordingly twice. The minimum input current gives a lower bit x0 and corresponds to the maximum resistance R0.

That is, the resistance in the circuit of the i-th binary code should be equal to:

Ri  = R0/2i.

Connecting of the bit current Ii to the input of the operational amplifier or switching it to "ground" is carried out by the electronic switch, shown in Fig. 14.2.

Fig. 14.2

If xi = 0, transistor VT2 is open and VT1 is closed and current Ii is disconnected from the input of the operational amplifier. At xi = 1, the transistor VT1 is opened and the current of the current Ii is fed to the input of the operational amplifier together with other bit currents. Thus, the total current supplied to the operational amplifier is determined by the expression:

I( Uоп , Ri , xi ) =  xi  Ii     =  Uоп  xi / Ri     = Uоп/R0 xi 2i      = Uоп Xn/ R0,

where Xn is the input digital code.

Almost all of these currents flow through the resistor Roc of the feedback circuit in the operational amplifier (OA), since the OA's own input current is disappearing small. Then the output voltage of such DAC is equal to the voltage drop across the resistor in the feedback circuit Roc (taking into account the sign):

Uвых  = - Rос × I( Uоп , Ri , xi ) =  - Rос Uоп Xn/ R0.

At high bit DAC, current-setting resistors must be determined with high accuracy. The most stringent precision requirements are imposed on the higher-order resistors, since the current variance in them must not exceed the current of the lower-bit current. Therefore, the variation of the resistance denominations in the i-th bit Ri   must be no more than Ri / Ri (%) = 100 / 2i + 1.

From this condition it follows that the variation of the resistor resistances, for example, in the fourth digit should not exceed 3%, and in the 10-th digit - 0.05%.

The above DAC scheme is simple, cheap, the ability to convert binary-decimal codes without first recoding into binary code.  Its performance is uniquely determined by the frequency properties of the OA and the performance of the transistors of the electronic switch on transistors VT1, VT2.

The scheme under consideration has a number of disadvantages. The main one is the need for a large number of precision resistor types. In addition, the resistance of the higher-order resistors in the multi-bit DAC can be comparable to the resistance of the locked key, which causes an additional conversion error. These disadvantages limit the allowable bit of conversion codes. As a rule, for such schemes have no more n  6.

Figure 14.3 shows the DAC graphic designation in a Workbench environment with an output voltage proportional to the digital code.

Fig. 14.3

14.2. DAC based on R - 2R series of resistors

To eliminate the main disadvantage of circuits based on the analog adder, a DAC scheme based on the R-2R resistor line, shown in Fig.14.4, has been developed and widely used. Resistors 2R of such line through electronic switches on transistors VT1, VT2 (Fig. 14.5) are connected by a logical signal xi either to ground through transistor VT1, or to a potentially grounded input of an operational amplifier. Therefore, the current of the lowest bit x0 is determined by the resistance of the resistor 2R and the node voltage, which is the result of the separation of the reference voltage Uoп for the discharge x1 between the resistor R and the parallel connection of the two 2R resistors, that is, half the voltage.

Fig. 14.4

For the i-th order, the node voltage is:

Ui =Uоп / 2n-1-i.

Then the current of the i-th order Ii is determined by the expression:

Ii = Ui / 2R = Uоп / (2R *2n-1-i) = Uоп 2i/ (R 2n).

The input of the OA, taking into account the action of logical variables xi receives a current signal equal to:

Iоу =  xi Ii  .

Fig. 14.5

This current, flowing mainly through a resistor in the negative feedback circuit R0, generates an output voltage:

Uвих = - Iоп R0= - R0 xi Ii  =  - R0Uоп / (R 2n) xi 2i = - R0UопXn / (R 2n) =  U Xn.,

where the value of U = R0 Uоп / (R 2n) is a discrete amount of voltage corresponding to the minimum increment of the output voltage, i.e.. the contribution of the smallest bit  to the output signal.

The main advantage of the DAC discussed in this section is the minimum number of precision resistor types - only 2 (R and 2R). Main disadvantage is due to the non-ideality of the electronic switches, the resistance of which is added with a resistance of 2R and introduces a corresponding statistical error, and in this DAC there is. However, the number of such DACs is greater (n = 10 ... 12) and their application is wider.

14.3. DAC on current switches

The design of the DAC on diode current switches is to eliminate the influence of the resistance of non-ideal electronic keys on the value of the controlled variable bits of current. For this purpose it is necessary to ensure the formation of each bit currents from sources with the maximum possible internal resistance (preferably, from an ideal current source), the value of which could not be affected by the internal resistance of the electronic key. As such a current source with very high internal impedance can be used bipolar transistor included in the scheme with a common base.

In Fig.14.6 is shown DAC circuit based on the R-2R resistive matrix, bipolar transistors VT0,… VTn-1  included in the common-base circuit, and switches for the bit  currents of diodes VD1, VD2.

Currents of each bit are formed by the resistor matrix R-2R. The potentials of the transistor bases are the same and offset by the Rб – VD0 circuit so, that the transistor emitter potentials are approximately zero. Then the currents of the emitters of the transistors are determined by the node voltages formed by the resistor matrix R-2R.

Fig. 14.6

For the transistor VTn-1, this voltage is Un-1 = - Uоп, for the VTn-2 transistor it is twice less than the absolute value: Un-2 = - Uоп / 2 and further, at the transition from node to node, the voltage is halved. For the i-th bit, the node voltage is:

Ui = – Uоп/2n-1-i.

Then the emitter current of the i-th transistor is determined by the expression:

Iэi  = Ui/2R = Uоп/(2n-1-i*2R) = Uоп/( R 2n-i).

Accordingly, the current of the collector of the i-th  transistor is constant and is equal to:

Ii =  Iэi    Uоп/( R 2n-i),

where 1 is the current transfer ratio of the transistor in the scheme with a common base.

The collector currents of the each bit transistors flow either through the diode VD1 if the logic variable of the discharge code is low (xi = 0) and the diode VD2 is closed or through the diode VD2 if the logical variable discharge is high (xi = 1). That is, if the diode VD1 is closed and VD2 is open, and if the diode VD1 is open and VD2 is closed.

Currents of VD1 diodes are added and form the total current:

,

which flows mainly through resistor R0 in the negative feedback loop of the operational amplifier OA. At its output, taking into account the bias voltage Ucm formed voltage:

Uвых1 = Uсм  - R0 = Uсм   = Uсм  -  Xn ,

where Xn -  the input digital code.

The bias voltage Ucm is required to ensure the normal active mode of transistors VT0… VTn-1. To neutralize the voltage Ucm and simultaneously scaling the output voltage of the DAC uses a subtraction scheme for OA2. At its output voltage is equal to:

Uвых2 =  Xn  .

The main advantage of this DAC is that its output voltage is almost independent of the resistance of the electronic keys (in this case, diodes VD1 and VD2). This makes it possible to increase the maximum digit of conversion  codes: n 14.

The disadvantage of this scheme is the operation of bipolar transistors in active mode, which increases the power, dissipated on the collectors of the transistors, and have to solve the problem of heat removal.  Highest power is dissipated on the VTn-1 transistor.

14.4. ADC of deployment compensation

The principle of operation of analog-to-digital converters (ADC) is to measure the level of the input signal and output the result in digital form. As a result of the ADC, discrete samples of continuous analog signal are converted to the corresponding digital codes.

In order for the ADC to function properly, the input signal must not change during the conversion time, which is usually accompanied by a sampling-storage circuit that captures the instantaneous signal level and stores it throughout the conversion time. In Fig. 14.7 shows ADC of deployment compensation, that converts an analog signal Ua into an n-bit binary code Xn.

Fig. 14.7

This converter is a typical example of sequential one-way ADCs containing an RS-trigger, a clock pulse generator CPG, a n-bit binary counter CT, an analog comparator AK, a digital-to-analog converter ПA , and a n-bit register RG controlled by a signal output from a positive output front of the ДПФ. At the output of the register, RG generates the output code Xn. One input of the AK comparator receives an analog input signal UA and the other inputs connected with a DAC ПА output.

The operation of the converter begins with the arrival of the startup pulse St, which sets in a state “1” RS-trigger, the output of which opens the DD2 valve and pulses from the clock generator CPG with frequency f0 are fed to the input of the binary counter CT2, which sums the number of input pulses. The output code of the counter is fed to the DAC, which converts it into voltage. The equilibration process lasts until the DAC output voltage equals or exceeds the input voltage UA. At the time of comparison or exceeding the output voltage of the DAC with the analog voltage UA, the analog comparator AK is triggered.

Switching the output of the comparator from “1” to “0” indicates the completion of the conversion process. The output code Xn is proportional to the input voltage at the end of the conversion.

The conversion time of this type of ADC is variable and depends on the input voltage. Its maximum value tпр.max responds to the maximum input voltage and for the n-bit binary counter and clock frequency f0 is equal to:

tпр.макс = (2n-1)/ f0.

For example, at n = 10 and f0 = 1MHz, tпр.макс = 1024 μs, providing a maximum sample rate of the order of 1 kHz.

When operating without a sampling-storage device, the aperture time coincides with the conversion time. As a result, the result of the conversion is extremely dependent on the ripple of the input voltage. In the presence of high-frequency ripples, the average value of the source code nonlinearly depends on the average value of the input voltage. This means that the ADCs of this type without the sampling-storage device are suitable for operation with constant or slowly varying voltages, which during the transformation change no more than the value of the conversion quantum.

Thus, the ADC of the deploying compensation is the low conversion rate, which reaches several kHz:

fизм.макс  1/ tпр.макс = f0 / (2n-1).

The advantage of ADC of this class is the comparative ease of construction, due to the consistent nature of the conversion process, and the disadvantage - low speed.

14.5. ADC following compensation

In Fig. 14.8 shows the scheme of the ADC following compensation, the main functional elements of which are a reversible binary counter CT2, digital-to-analog converter ПА , clock pulse generator CPG, analog comparator AK, detector positive and negative fronts ДФ and n-bit commutator  of code Xn.

The principle of the ADC following compensation is based on the use of a reversible counter whose source code is converted by a digital-to-analog converter into a sampled voltage UЦ . The voltage of the UЦ is compared with the input signal UA of an analog comparator, which generates a signal Uk of the control of the direction of the counter account. Thus, a numeric code Xn is formed in the counter, which corresponds to the instantaneous value of the analog value UA:

UА = Xn ΔU + δU,

where ΔU = Uоп    / 2n,

δU - conversion error; moreover, ΔU ≤ | δU | .

In Fig. 14.8 shows a block diagram of the ADC following compensation.

Fig. 14.8

Clock pulses with a frequency f0 are fed to the input of the counter CT2, the output code of which is converted into analog form and how the voltage Ux is supplied to the input of the comparator AK and compared with the converted analog voltage UА. For example, if Ux <Ua, voltage of high level from comparator controlled the counter CT2 and it works in the sum mode. In time the value of voltage Ux steppedly increased and at Ux> Ua, the comparator switches the controlled input of the counter CT2 and turned it to the subtraction mode. When the detector has a DF front, it forms an impulse, which means that the value of the digital code Xn corresponds at this moment to value Ua. From this moment the value of voltage Ux steppedly decreased to quiet feast leave Ux> Ua. At the moment of equals or Ux <Ua the comparator will switch over and put the counter in the sum mode. When the detector is used, the front of the ДФ is redefined as a pulse of a specific value of the digital code Xn. In this way, the value of the output voltage Ux follows to changes of input voltage Ua.

For normal work of such an ADC, it is necessary to ensure that speed of change voltage of the analog signal Ua was smaller and then the speed of the speed of the output signals Ux of DAC. That means, that must be used condition:

dUa/dt  <  ΔU f0  = Uоп f0/2n ,

where ΔU is the quantum of the increase in the eddy spacing of the DAC, Uоп is the bearing voltage of the DAC.

The smaller the speed of the input of the analogue signal dUa / dt is, the higher is the frequency of the output code Xn. At Ua = const, the frequency of output code Xn  is expensive f0.

The advantage of such ADC is simplicity of structure, and disadvantage is asynchronism of the type of output code Xn.

14.6. Successive approximation ADC

A successive approximation converter ADC is the most widespread variant of successive ADC.

Work of this class of converters principle of dichotomy is the basis of, i.e. successive comparing of measured to 1/2, 1/4, 1/8 et cetera from her maximal possible value. It allows for n- bit of ADC to execute all process of transformation after n of successive steps (iterations) instead of 2n times at the use of successive account and to score a substantial advantage in a fast acting.  For example, already at n = 10 this winning arrives at 100 times and allows to get by means of such ADC to 105 ... 106 converting in a second. In the same time static error of this type of converters, that is determined mainly DAC, that is used, can be very small, that allows realizing a discriminability to 14 binary digits at frequency of selections of to 200 kHz.

Let us consider principles of construction and work of successive approximation ADC on the example of the structure brought around to Fig.14.9, that consists of next basic knots: clock pulse generator CPG, RS- trigger, register of successive approximation of РПП, digit-to-analog converter DAC, analog comparator of АК, detector of positive fronts of ДПФ and n- bit register  RG for storage and delivery of current values of digital code of Хn.

Fig. 14.9

The starting entrance impulse of St sets RS-trigger in the state "1" and n- bit register of successive approximation of РПП  in the initial state, when in n-digit is brought "1", and in other digits - "0". Content of РПП will grow into an exit DAC. Output of DAC in analog form of Uц and compared on the analog comparator of АК with the current value of analog signal of Uа. If Uц < Uа, the state of n- digit is fixed at the level of "1", and in case of Ux  > Uа the state of n- digit is fixed at the level of "0".

Next impulse brought "1" in the (n - 1)-th digit of РПП and like on the state the initial signal of analog comparator of АК the value of (n – 1)-th digit of source code is determined. At n times will be certain all n digits of code of Хn. Through n times on the exit of РПП the signal of overfilling of Р is formed, that transforms the detector of positive fronts of ДПФ in the impulse of reset of RS- trigger in the state "0". The same impulse provides the  transfer  of content of register РПН as n- bit code of Хn in the register  of RG, that corresponds  to the current value of analog signal of Uа.

This class of АЦП occupies intermediate position after a fast acting, cost and discriminability between consistently parallel and integrating ADC and finds wide application in control system, control and digital treatment of signals.

14.7. ADC of double integration

In fig. 14.10 are brought the principle over of transformation (б), analog integrator on an operating amplifier (а) and example of realization of ADC of double integration (в). Such ADC can be in one of the 3th states: accumulation, measuring, storage. For fixing of 3 states the used 2 RS-triggers (Q1, Q2). Output signals of triggers used for a control by the keys of К1 К2, К3 by signals accordingly V1, V2, and V3. In a table 14.7 the brought states over of triggers in accordance with time intervals and signals of control of the keys on the transistors VT1, VT2, and VT3.

a.

б.

в.

Fig. 14.10. ADC of double integration: а-conceptual chart; б- time diagram of transformation; в- fundamental circuit

Table 14.7

| State | Time interval | Q1 | Q2 | V1 | V2 | V3 | Function V |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Storage | t<t0;t>t2 | 0 | 0 | 0 | 0 | 1 | V3=Q1∙Q2=Q1+Q2 |
| Accumulation | t0≤t≤t1 | 1 | 0 | 1 | 0 | 0 | V1=Q1∙Q2=Q1+Q2 |
| Measuring | t1≤t≤t2 | 0 | 1 | 0 | 1 | 0 | V2=Q1∙Q2=Q1+Q2 |

Work of ADC begun with the serve of starting impulse of St, that sets RS- trigger of DD1 in the state of Q1=1, and RS-trigger of DD2 confirms the previous state of Q2= 0. The same signal to the counter of DD9 (entrance of D) is add the constant of nн, that determines the fixed amount of times of accumulation of nн. In accordance with the table of the states by the logical elements of NOR DD4, DD5, DD6 is formed signals of V1=1, V2=V3=0, the key of К1 on the transistor of VT1 is switched on, and keys of К2, К3 on transistors accordingly VT2, VT3 states switched off. Through VT1 the input analog signal of UA (for shown example UA < 0) acts on the entrance of integrator of DА1. Increase of initial tension of integrator of UI > 0 stipulates establishment on the exit of analog comparator of DА2 of high level of U1k, that provides the element of DD8 serve of time impulses on the exit of С of meter of DD9. The mode of operations of the reversible counter is determined by the state of input A: at A= 1 counter functions in the mode of subtraction, and at A = 0 - in the mode of add. After of nн times the state of counter changes from nн to 0, when on the exit of V of counter a high level (V=1) that grows the detector of positive fronts ДПФ of DD12 into a short positive impulse is formed. Impulse from ДПФ (DD12) resets RS-trigger of DD1 (Q1=0) and sets in the state of Q2 =1 RS- trigger of DD2, that answers passing of ACD to the state of measuring. Thus signal of V1=0 switch off the key of К1 on the transistor of VT1, and signal of V2=1 switch on the key of К2 on the transistor of VT2. On the input of integrator the fixed supporting voltage of Uoп (in the set example of Uoп > 0) is given on the output of integrator of voltage of UІ diminishes on a linear law. Thus on an entrance A of counter DD9 operates the low level of A=Q1= 0, i.e. a counter functions in the mode of add. When the output level of integrator reaches level UІ ≤ 0 (moment of time of t2), the output signal of comparator is transfer in the state of UК = 0, that blocks the receipt of impulses on the input of counter of DD9. Thus, in a counter the number of Xn is kept, that is the digital equivalent of analog quantity of UA. The detector of negative fronts (ДВФ) of DD11 forms a short positive impulse that reset RS- trigger of DD3 in the state of Q2=0 and ADC transfer in the mode storage to the next starting impulse of St. Impulse from ДВФ is given on the input С of parallel register of DD10, in that the certain code of Xn, which is proportional to the input voltage of UA:

Xn = - UA nн/ Uoп .

Time of transformation of ADC of this type is to the variables and depends on input voltage of UA. The maximal value of tпр.макс  answers maximal input signal UA and for n-bit binary counter and frequency of impulses of f 0 is:

tпр.макс = (2n + nн)/f0,

where nн - an amount of time interval of accumulation.

Basic advantages of the considered type of ADC are:

- it is independence of result of transformation from stability of parameters of integrator, that allows to realize ADC with the bit of output code of n ≤12, without using of high-fidelity and high-stable components;

- the error of conversion does not depend on stability of parameters of components of integrator of R, C ( range of parameters and their changing in time as a result of aging, change of temperature and others like that);

- the error of conversion does not depend on stability of frequency of f0 of time generator, if  frequency unimportant changes in continue to the interval of t0 …t2. The error of the conversion depends only on stability of supporting voltage Uоп  .

14.8. АDC of parallel transformation

The circuit of АDC of parallel conversion is shown in fig.14.11.

Fig. 14.11

In parallel ADC principle of direct comparison of analog quantity of UA is used with the discrete values of supporting voltage of Uоп.

In Table 14.8 is shown a comparison of analog voltage UA with the discrete levels of supporting voltage of Uоп, divided in this case into 8 ranges. Physical realization of discrete levels of the voltage with resistance divider is provided.

Comparing of input voltage UА to the discrete levels is executed by the comparators of DA1 - DA7. If UA > Uоп × i / 8, on the output of comparator is set X i =1. Thus, on outputs of comparators appears a code (code of Johnson) that can be recorded in the binary code of Y2 Y1 Y0 with the help of transformer of codes (X/Y) or with priority encoder (СD) (for to the most significant digits).

Table 14.8

| Порівн. UA з UОП | X7 X6 X5 X4 X3 X2 X1 | Y2 Y1 Y0 |
| --- | --- | --- |
| 0UA < 1/8 UОП | 0 0 0 0 0 0 0 | 0 0 0 |
| 1/8 UОП ≤ UA < 2/8 UОП | 0 0 0 0 0 0 1 | 0 0 1 |
| 2/8 UОП ≤ UA < 3/8 UОП | 0 0 0 0 0 1 1 | 0 1 0 |
| 3/8 UОП ≤ UA < 4/8 UОП | 0 0 0 0 1 1 1 | 0 1 1 |
| 4/8 UОП ≤ UA < 5/8 UОП | 0 0 0 1 1 1 1 | 1 0 0 |
| 5/8 UОП ≤ UA < 6/8 UОП | 0 0 1 1 1 1 1 | 1 0 1 |
| 6/8 UОП ≤ UA < 7/8 UОП | 0 1 1 1 1 1 1 | 1 1 0 |
| 7/8 UОП ≤ UA | 1 1 1 1 1 1 1 | 1 1 1 |

The considered type ADC does not need synchronization of the process conversion. Forming of the output code of Xn will be realized with only time delay of passing of signal through a comparator and transcoder that provides the maximal fast-acting of АCD. It is necessary to mark that complication of circuit is doubled at the increase of bit of source code of n on n+1. For example, for n=8 it is necessary 255 comparators and corresponding encoder on 255 entrances. Therefore for realization of multibit parallel ADC use parallel-sequence  ADC, that yet name conveyer.

14.9. Conveyer АDC

Principle of construction of conveyer АDC is based on determination of difference between input signal of UA and the nearest the discrete level of Uопi, strengthening of the got difference to the range of UA, following  discretization of the distinguished and increase difference in the digital code of the least significant bits, selection to the difference, strengthening and further transformation. Example of such to the "conveyer" is made an in Fig.14.12.

Fig. 14.12

On the first stage of transformation parallel ADC DD1 converts input analog voltage of UA into a 3-bit code (x8 x7 x6) with the error of discretization ΔU1 < UОП /8. Code of x8 x7 x6  with a help a DAC DА2 transfers into the analog quantity of UЦ1. With the help of circuit of subtraction on the operating amplifier of DА1 the error of discretization ΔU1=UA - UЦ1 is calculated, that simultaneously increases in 8 times, that means the down-scaling of error to the range of supporting tension of UОП :

UА2 = 8(UA-UЦ1).

On the second stage voltage of UА2 with the help of ADC DD3 transfers into the code of the least significant bits of x5 x4 x3, which DAC DD4 converts into discrete voltage of UЦ2. The circuit of subtraction – amplifying on DА2 forms the analog signal of UА3, which ADC DD5 converts into the 3-bit code of x2 x1 x0. The 9-bit code of x8x7…x0 thus formed that is the digital analogue of input level of UА.

Note that all operation of transformation does not need synchronization of components of conveyer structure. The latest forming the least significant bits of x2 x1 x0 with the delay time in the stages of the described structure. Therefore such a ADC is classified as parallel-consequent. They provide minimum time of tп transformation of UА→ Xn,  i.e. maximal quantum frequency of analog signals:

fп max≤1/tп.